# The Space Mapping Super Model Concept

J.W. Bandler and M.A. Ismail

SOS-98-12-R

July 1998

 J.W. Bandler and M.A. Ismail 1998

No part of this document may be copied, translated, transcribed or entered in any form into any machine without written permission.  Address inquiries in this regard to Dr. J.W. Bandler.  Excerpts may be quoted for scholarly purposes with full acknowledgement of source.  This document may not be lent or circulated without this title page and its original cover.

THE space MAPPING SUPER MODEL CONCEPT

J.W. Bandler and M.A. Ismail

Simulation Optimization Systems Research Laboratory

and Department of Electrical and Computer Engineering

McMaster University, Hamilton, Canada L8S 4L7

Tel 905 525 9140 Ext. 24305

Fax 905 523 4407

Email awad@soya.sos.mcmaster.ca

Abstract  This report presents a concept called Space Mapping Super Model (SMSM) to improve the accuracy of empirical models of microwave circuit elements such as microstrip open stubs, microstrip lines, coupled microstrip lines, etc.  SMSM transforms the set of physical parameters of the considered microwave element to another set of parameters to be used by the empirical model of this element such that the empirical model response matches the response obtained using an electromagnetic (EM) simulator.

I.  Introduction

Empirical models of microwave elements usually behave very well at low frequencies, however at higher frequencies their accuracy degrade.  For example, consider a microstrip line with length L, width W, substrate height H and dielectric constant r equal to 50 mil, 10 mil, 15 mil and 9.8, respectively.  This element was analyzed by Sonnet’s em simulator [1] and by the empirical model presented by Jansen et al. [2] supplied by the OSA90/hope simulator [3].  The size of the grid used in Sonnet’s em simulator is 1.0 mil by 1.0 mil.  Fig. 1 shows the magnitude of the reflection coefficient S11obtained by Sonnet’s em simulator [1] together with S11 obtained by the empirical model in the frequency range 1 GHz to 40 GHz with a step of 3 GHz.  It is clear that S11 obtained by both simulators are different at high frequencies.

In this report, we present a concept called Space Mapping Super Model (SMSM) to overcome this deficiency in empirical models.  This technique transforms the physical parameters of the analyzed microwave element to another set of parameters.  These transformed parameters are used by the empirical model so that the response obtained by the empirical model matches that obtained using the EM simulator in a certain frequency range.  SMSM is based on a novel optimization technique called Space Mapping (SM) by Bandler et al. [4].

Two spaces are defined in SMSM.  The first space is called the EM space and it contains the physical parameters of the microwave element to be analyzed (i.e. the length L, the width W, the substrate height H, etc.).  The second space is called the linear simulator space or the empirical model space, where every element represents the transformed physical parameters to be evaluated by the SMSM.  In Section II, we present the Space Mapping Super Model (SMSM) technique and the algorithm to implement it.  Section III discusses the parameter extraction problem which is crucial to the performance of the SMSM technique.  In Section IV, we apply the SMSM to an example and compare the results before and after using the SMSM.  Finally, the report is concluded in Section V.

II.  SPACE MAPPING SUPER MODEL (SMSM)

Consider a microwave element E with physical parameters represented by the vector xem( for example, the parameters of a microstrip open stub are represented by the vector xem=[LWHer]T).  The space of xem is called the EM space and is denoted byXem.  If the element E is analyzed by an EM simulator and by an empirical model, the responseRls(xem)produced by the empirical model is different from the responseRem(xem) produced by the EM simulator, particularly at high frequencies.  Therefore, the aim of this work is to find a new set of parameters represented by a vector called xlssuch that

| Rls(xls)»Rem(xem) | (1) |
| --- | --- |

The space of xls is called the linear simulator space or the empirical model space and is denoted byXls. 	The considered problem is illustrated in Fig. 2.  We have a region Regem in the EM space Xem and it is required to establish a mapping P from this region to a region Regls in the Xls-space such that

| xls=P(xem)and Rls(xls)−Rem(xem)≤ε | (2) |
| --- | --- |

where   denotes a suitable norm and   is a small positive constant.  The value of xls is found by solving the parameter extraction problem [4]:

| xls=P(xem)=argminx ∈Xls Rls(x)−Rem(xem) | (3) |
| --- | --- |

The SMSM starts by selecting a set of base pointsBem with m points in the region Regem

| Bem=xem(1),xem(2),...,xem(m). | (4) |
| --- | --- |

Therefore, by parameter extraction it constructs the set of points Bls in the Xls-space

| Bls=xls(1),xls(2),...,xls(m). | (5) |
| --- | --- |

such that

| xls(i)=argmin x∈XlsRls(x)−Rem(xem(i)) , i=1, 2, ..., m. | (6) |
| --- | --- |

The initial mappingP0is constructed so that it maps every element in the set Bem to the corresponding element in the set Bls.  The technique updates the mapping P in an iterative manner by testing the mapping Pj established in the jth iteration on a set of testing points Cem (the set Cem has no common elements with the set Bem).  Then, it constructs using the existing mapping Pj the corresponding set Cls in the Xls-space.  That is, for every element xemÎCemfind the corresponding point xlsÎCls where

| xls=pj(xem) | (7) |
| --- | --- |

It also finds, by parameter extraction, the corresponding points of Cem and put them in the set Dls.

The technique stops if the points in the set Cls are very close to the corresponding points in the set Dls within a certain accuracy.  If not, it augments the set Bem by adding to it the elements of the set Cem and the set Bls by adding to it the elements of Dls obtained by parameter extraction .  It then updates the mapping Pj to get a new mapping Pj+1.  If the number of points in Bem is equal to a predefined number N (to be set by the user) the technique terminates and considers the current mapping as the final required mapping P.  The flow chart of this procedure is shown in Fig. 3.

Construction of the mapping P

The mapping P is constructed in an iterative manner during the running of the SMSM algorithm.  At the jth iteration of the algorithm, assume that the two sets of points Bem and Bls have mj points each.  Therefore, the mapping Pj transforms every point in Bem to its corresponding point in Bls.  The mapping Pj is defined as a linear combination of a predefined and fixed t fundamental functions

| f1(xem),f2(xem),f3(xem),...,ft(xem), | (8) |
| --- | --- |

such that the ith component of xls is represented by

| xlsi=s=1taisfs(xem). | (9) |
| --- | --- |

The number of fundamental functions satisfies the relation mj³t.  In matrix form, (9) is written as

| xls=pj(xem)=Ajf(xem), | (10) |
| --- | --- |

where Aj is an (n x t) matrix (n is the dimension of the column vector xls) and f(xem) is a t-dimensional vector of fundamental functions.  Consider the mapping Pj for all points in Bem and Bls and expand (10) to get

| xls(1)xls(2)...xls(mj)=Ajf(xem(1))f(xem(2))...f(xem(mj)). | (11) |
| --- | --- |

The matrix of constant coefficients Aj can be evaluated using the leastsquares method.  Using the notations:

| V=xls(1)xls(2)...xls(mj)T,S=f(xem(1))f(xem(2))...f(xem(mj))T, | (12) |
| --- | --- |

(11) can be written as

| VT=AjST. | (13) |
| --- | --- |

By taking the transpose of both sides we get

| SAjT=V, | (14) |
| --- | --- |

which is an overdetermined system of linear equations.  The leastsquares solution of (14) is given by

| AjT=(STS)−1STV | (15) |
| --- | --- |

In this report, we are using linear mapping.  Accordingly, the vector of fundamental functions in (10) is given by

| f(xem)=1xem1xem2...xemnT. | (16) |
| --- | --- |

Consequently, the matrix S in (15) is given by

| S=11...1. | (17) |
| --- | --- |

The mapping Pj is fully determined by (10) and (15).

The sets Bem and Cem

The way the two sets Bem and Cem are constructed is crucial to the performance of the SMSM technique.  When constructing these sets, one should compromise between the number of points they contain and the amount of information these points have (i.e., the distribution of these points in the region Regem).  Of course, using a very large number of points in Bem and Cem improves the accuracy of the mapping but it is time extensive.

If the vector xem is a twodimensional vector, then the region Regem is a twodimensional box with dimensions d1 and d2.  The set of base points Bem contains the four vertices of the box and the point at the centre of the box as shown in Fig. 4(a).  At the jth iteration, the set of test points Cem contains the vertices of the box in Fig. 4(b) whose dimensions are (d1−2jh1)and (d2−2jh2) where h1 and h2 are two predefined increments.  During the running time of the algorithm, the maximum number of the sets of test points is given by

| M=mind1−h12h1,d2−h22h2, | (18) |
| --- | --- |

where x is the largest integer less than or equal to x.  The maximum number of points that can be used during the SMSM algorithm is N=5+4∗M.

The way of selecting the sets Bem and Cem in two dimensions can be easily generalized in the n-dimension case, where the region Regem is an n-dimensional box with side lengths d1, d2,..., dn.  The number of the sets of test points is given by

| M=minidi−hi2hi, where i=1,2,...,n | (18) |
| --- | --- |

and the maximum number of points that can be used during the SMSM algorithm is

| N=1+2n(1+M). | (19) |
| --- | --- |

The SMSM Algorithm

The SMSM can be implemented through the following steps.

Step 1	Determine the maximum number of points N in the region Regem using (19).

Step 2	Construct the set Bem.

Step 3	Construct by parameter extraction the set of points Bls and Initialize j=0.

Step 4	Establish the mapping Pj by evaluating the matrix AjT=(STS)−1STVwhere the matrix V is constructed using the points in Bls according to (12) and the matrix S is constructed using the points in Bem according to (17).

Step 5	If the total number of elements in the set Bem equals N, then stop and consider the current mapping Pj as the final mapping P.

Step 6	Construct the set of test points C and find the corresponding elements by using the current mapping Pj.

Step 7	If the mapped elements found in step 6 are very close to the corresponding elements of Cem obtained by parameter extraction, stop and set P=Pj.  Else, augment the set Bem by adding to it the elements of Cem and augment the set Blsby adding to it the points obtained by parameter extraction corresponding to the points in Cem and set j= j+1 and go to step 4.

III.  PARAMETER EXTRACTION

Parameter extraction process is very important to the SMSM technique.  In each iteration of the SMSM we have to perform parameter extraction more than once as presented in Section II.  Recall that we have a microwave element E with physical parameters represented by the vector xem, and we want to find transformed parameters represented by the vector xls such that (1) is satisfied.  The parameter extraction process involves the solution of the optimization problem (3).  The norm mentioned in (3) can be the l1norm, l2norm or Hubernorm [6].  In this report the Hubernorm was chosen since it is robust against gross errors and flexible with respect to small errors [6].

IV.  EXAMPLE

Consider two model of the microstrip line.  The first model is a transmission line model using two parameters, the characteristic impedance Z0 and the line length, see Fig. 4.  The second  model is the empirical model of microstrip line presented by Jansen [2].  The empirical model is considered as the EM model while transmission line model is considered as the linear simulator model.  The parameters of the microstrip empirical model are 40mil≤L≤65mil, 10mil≤W≤35mil, the substrate height H=15 mil and εr= 9.8.  Therefore, the region Regem and the vector xem in the EM space are given by

Regem≃(W,L)| 40 mil≤L≤65 mil and 10 mil≤W≤35 mil xem=[W,L]T

The vector xls to be used in the transmission line model is given by

xls=[W1,L1]T.

Therefore, the parameters used in the transmission line are the length L1 and the characteristic impedance Z0 (which is computed using the quasi-static model in [7] for the microstrip transmission line in terms of the width W1, the substrate height H and the relative dielectric constant εr).

The SMSM algorithm was applied to this problem in order to match the response obtained by the transmission line model with the response obtained by the empirical model implemented in OSA90/hope [3].  The frequency range used is from 40 GHz to 50GHz with a step of 0.5 GHz.  The algorithm terminated after two iterations (13 parameter extraction were needed) and produced the mapping P which is represented by

xls=p(xem)=A 1,

or in other form

W1= 0.4042+0.9074 W−0.0409 LL1=− 0.7487+0.0290 W +1.0960 L

To show the benefit of the SMSM algorithm we performed yield analysis supplied by OSA90/hope [3] in the region Regem.  We generated 50 uniformly distributed random points inside Regem.  The specifications used to obtain the yield are the difference between the real part or the imaginary part of the scattering parameters obtained by the two models is less than (0.04).  Before using the SMSM technique the yield is found to be zero and after using it the yield became 90%.  Figs. 6, 7, 8 and 9 show the difference of the real part and the imaginary part of the scattering parameters obtained by the empirical model and by the transmission line model.  The circuit and the MATLAB [5] files required to implement the SMSM technique for this problem are given in Appendix A.  While the circuit file (accepted by OSA90/hope [3]) to perform yield analysis is given in Appendix B.

V.  CONCLUSIONS

In this report we presented the Space Mapping Super Model (SMSM) technique to enhance the accuracy of microwave empirical models.  For a microwave element, the technique establishes a mapping to transform the parameters of this element to another set of parameters.  These transformed parameters are used by the fast but less accurate empirical model of this element to match its response with that obtained by an accurate but time extensive EM simulator.  The proposed technique was tested in an example and gave good results.

APPENDIX A

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!!This file performs parameter extraction to match the response of the transmission

!!line model with the response of the microstrip empirical model

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

Model

!!!!!!!!!!!! parameters values and frequency range

F_MIN=40ghz;

F_MAX=50ghz;

F_STEP=0.5ghz;

N=13;  ! the number of points we perform parameter extraction up on

k=1;   ! just an index

L[N]=[40mil 40 mil 65mil 65 mil 52.5mil 45mil 45 mil 60mil 60 mil

50mil 50 mil 55mil 55 mil];

L1[N]=[?43.3562mil? ?43.9348mil? ?71.4879mil? ?70.4699mil? ?57.565mil?

?49.0647mil? ?49.2502mil? ?65.9403mil? ?65.4628mil? ?54.7716mil?

?54.8464mil? ?60.3621mil? ?60.2439mil?];

W[N]=[10mil 35mil 35 mil 10mil 22.5mil 15mil 30mil 30 mil 15mil

20mil 25mil 25 mil 20mil];

W1[N]=[?7.82576mil? ?30.8619mil? ?29.4513mil? ?7.68279mil? ?18.2689mil?

?12.0043mil? ?26.8623mil? ?24.9827mil? ?11.7219mil? ?15.9467mil?

?20.6309mil? ?20.5213mil? ?16.0013mil?];

H=15mil;

EPSR=9.8;

!!!!!!! The empirical model for the microstrip!!!!!!!!!!

MSUB EPSR=EPSR H=H;

MSL 1 2  W=W[k] L=L[k];

PORTS 1 0 2 0;

!!!!!!!!!!!!!!!!!!!!!!The TL model!!!!!!!!!!!!!!!!!

!! compute the effective dielectric constant and the characteristic impedance

!! using the quasistatic analysis

!! that is transform W1, H1, EPSR1 to Z0 and the effective dielectric constant

EPSR_E=( EPSR+1)/2+(EPSR-1)/(2 *sqrt(1+12*H/W1[k]));

Z0=if((W1[k]/H)<1)

(60/sqrt(EPSR_E) * log(8*H/W1[k]+W1[k]/(4*H)))

else

(120*pi/(sqrt(EPSR_E)*(W1[k]/H+1.393+0.667*log(W1[k]/H+1.444))));

TRL 3  4 Z=Z0 L=L1[k] K=EPSR_E F=FREQ;

PORTS 3 0 4 0;

CIRCUIT;

!! This is for displaying purposes only

D_RS11=abs(RS11-RS33);

D_RS12=abs(RS12-RS34);

D_IS11=abs(IS11-IS33);

D_IS12=abs(IS12-IS34);

end

Sweep

AC:

k: from 1 to N step=1

FREQ: from F_MIN to F_MAX step=F_STEP

D_PS D_MS D_RS D_IS

end

Spec

AC:

k: from 1 to N step=1

FREQ: from F_MIN to F_MAX step=F_STEP

!!! the goals of the parameter extraction

RS11=RS33

RS12=RS34

IS11=IS33

IS12=IS34;

end

control

optimizer = HUBER;

end

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!! End of file !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

% %%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

% This Matlab file is to establish the mapping P in the Space Mapping Super Model Algorithm

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

function  AJ = P_Linear;

% where AJ is the same matrix mentioned in the report

% n is the dimension of xem and xls, assuming it is the same

Ns= 4;  %this is the number of points in the set of testing points Cem

N=2;   % the maximum number of iterations

[xls1, xls2, xem1, xem2]=get_data;  % get data is the function the extracted parameters values

% compute the matrix V

V1=xls1(1:Ns+1);  % the first vector of V

V2=xls2(1:Ns+1);  % the second vector of V

V=[V1 V2];

%compute the matrix S

S1=xem1(1:Ns+1);  % the first vector of  S

S2=xem2(1:Ns+1);  % the second vector of S

U(1:Ns+1,1)=1;

S=[U S1 S2];

% construct the initial mapping P0

epslon=0.03; % the maximum error

AJ=(inv(S.'*S)*S.'*V).'

for i=1:N

AJ=(inv(S.'*S)*S.'*V).';

Cem1=xem1((2+i*Ns): (1+(i+1)*Ns));

Cem2=xem2((2+i*Ns): (1+(i+1)*Ns));

U1(1:Ns,1)=1;

Cem=[U1 Cem1 Cem2];

Dls = Cem *AJ.';

Cls1=xls1((2+i*Ns): (1+(i+1)*Ns));

Cls2=xls2((2+i*Ns): (1+(i+1)*Ns));

Cls=[Cls1 Cls2];

Error=max(max(abs((Dls-Cls)./Cls)))

if(Error<= epslon)

break;

end

V1=[V1 ; Cls1];

V2=[V2 ; Cls2];

V=[V1 V2];

S1=[S1;Cem1];

S2=[S2;Cem2];

U(1: (1+(i+1)*Ns),1)=1;

S=[U S1 S2];

AJ=(inv(S.'*S)*S.'*V).'

end

%%%%%%%%%%%%%%%%%%%%% End of the file %%%%%%%%%%%%%%%%%%%%%%%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

% This function returns the values of the set of base points Bem and the set of test points Cem

%obtained by parameter extraction

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

function [Xls1, Xls2, Xem1, Xem2]=get_data;

Xls1=[11.0158   38.3204  38.4779  10.9578  24.087

16.4918  32.8907  32.9397  16.4927

21.8889   27.3376  27.3844  21.9269 ];

Xls2=[39.3525  39.0836  63.6283    63.7337  50.6077

43.9241   43.7737  58.4877     58.551

48.6866  48.6413  53.5232   53.5666 ];

Xem1=[10; 35; 35; 10; 22;

15; 30; 30; 15;

20; 25; 25; 20];

Xem2=[40; 40; 65; 65; 52;

45; 45; 60; 60;

50; 50; 55; 55];

Appendix B

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!! In this file we do montcarlo analysis to evaluate the Space Mapping Super Model used to

!! match the transmission line model response with the microstrip empirical model response

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

Model

!!!!!!! this is for montcarlo analysis!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!! define L as a uniform random variable between 40 mil and 65 mil

L_: 40 {Uniform TOL=25 LOW=0.0 HIGH=1.0};

!! define W as a uniform random variable between 10 mil and 35 mil

W_: 10 {Uniform TOL=25 LOW=0.0 HIGH=1.0};

!!!!!!!!!!!!!!!!!!!!!!!!!! circuit definition!!!!!!!!!

!! define the parameters values and the frequency range

F_MIN=40ghz;

F_MAX=50ghz;

F_STEP=0.5ghz;

L=L_*1mil;

W=W_*1mil;

H=15mil;

EPSR=9.8;

! Without SMSM

!W1=W;

!L1=L;

!!( L1,W1)=P(L,W) and it should be replaced by L1=L and W1=W if the SMSM is not used

!W1=( 0.4042 + 0.9074 * W_ - 0.0409 *L_) *1mil;

!L1=(-0.7487 + 0.0290 * W_ + 1.0960 *L_) * 1mil;

!!!!!!!!!!!!!!!! The empirical model for the microstrip TL !!!!!!!!!!

MSUB EPSR=EPSR H=H;

MSL 1 2  W=W L=L;

PORTS 1 0 2 0;

!!!!!!!!!!!!!!!!!!!!!!The TL physical model!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

EPSR_E=( EPSR+1)/2+(EPSR-1)/(2 *sqrt(1+12*H/W1));  !!! compute the effective dielectric constant

Z0=if((W1/H)<1)

(60/sqrt(EPSR_E) * log(8*H/W1+W1/(4*H)))

else

(120*pi/(sqrt(EPSR_E)*(W1/H+1.393+0.667*log(W1/H+1.444))));

TRL 3  4 Z=Z0 L=L1 K=EPSR_E F=FREQ;

PORTS 3 0 4 0;

CIRCUIT;

!! define the goal as the differences between the corresponding responses

D_PS11_1=abs(PS11-PS33);

D_RS11=abs(RS11-RS33);

D_RS12=abs(RS12-RS34);

D_IS11=abs(IS11-IS33);

D_IS12=abs(IS12-IS34);

D_RS[1,2]=[D_RS11  D_RS12];

D_IS[1,2]=[D_IS11  D_IS12];

end

Sweep

AC: FREQ: from F_MIN to F_MAX step =F_STEP

D_PS D_MS D_RS D_IS

end

MonteCarlo

AC:  FREQ: from F_MIN to F_MAX step =F_STEP

N_Outcomes=500

D_RS11<0.04

D_RS12<0.04

D_IS11<0.04

D_IS12<0.04;

end

## REFERENCES

[1]	emTM, Sonnet Software, Inc., 1020 Seventh North Street, Suite 210, Liverpool, NY13088.

[2]	M. Kirschning and R. Jansen, “Accurate model for effective dielectric constant of microstrip with validity up to millimeter-wave frequencies,” Electronics Letters, vol. 18, 1982, pp. 272273.

[3]	OSA90/hopeTM, Optimization Systems Associate Inc., Dundas, Ontario, Canada, 1997.

[4]	J.W. Bandler, R.M. Biernacki, S.H. Chen, P.A. Grobelny and R.H. Hemmers, “Space mapping 	technique for electromagnetic optimization,” IEEE Trans. Microwave Theory Tech., vol. 42, 	1994, pp. 25362544.

[5]	MATLAB, The MathWorks, Inc., Simon & Schuster/A Viacom Company Upper Saddle River, 	New Jersey 07458, 1996.

[6]	J.W. Bandler, S.H. Chen, R.M. Biernacki, Li Gao, K. Madsen and H. Yu, “Huber optimization 	of circuits: a robust approach,” IEEE Trans. Microwave Theory Tech., vol. 41, 1993, pp. 	22792287.

[7]	D.M. Pozar, Microwave Engineering, New York: Addison-Wesley, 1990.

Fig. 1.	The magnitude of the reflection coefficient S11of a microstrip analyzed by using the

Sonnet’s em simulator [1] () and by using the Jansen’s empirical model [2](---).

Fig. 2.	The mapping P which transforms any element in Regem to the corresponding

element in Reglssuch that (2) is satisfied.

Fig. 3.	The flow chart of the SMSM technique.

Fig. 4	For twodimensional region Regem: (a) the points in the set Bem; (b) the points

in the set Cem.

Fig . 5.	(a) microstrip line and the corresponding transmission line (b).

(a)

(b)

Fig. 6.	The difference between the real part of S11 obtained by the empirical model of the microstrip line and by the transmission line model before applying the SMSM technique (a) and after applying it (b).

(a)

(b)

Fig. 7.	The difference between the real part of S21 obtained by the empirical model of the microstrip line and by the transmission line model before applying the SMSM technique (a) and after applying it (b).

(a)

(b)

Fig. 8.	The difference between the imaginary part of S11 obtained by the empirical model of the microstrip line and by the transmission line model before applying the SMSM technique (a) and after applying it (b).

(a)

(b)

Fig. 9.	The difference between the imaginary part of S21 obtained by the empirical model of the microstrip line and by the transmission line model before applying the SMSM technique (a) and after applying it (b).

 	This work was supported by the Natural Sciences and Engineering Research Council of Canada under Grants OGP0007239, STP0201832 and through the Micronet Network of Centers of Excellence.