OPTIMAL MICROSTRIP BENDS

J.W. Bandler and T. Chen

SOS-98-17-R

August 1998

 J.W. Bandler and T. Chen 1998

No part of this document may be copied, translated, transcribed or entered in any form into any machine without written permission.  Address inquiries in this regard to Dr. J.W. Bandler.  Excerpts may be quoted for scholarly purposes with full acknowledgment of source.  This document may not be lent or circulated without this title page and its original cover.

OPTIMAL MICROSTRIP BENDS

J. W. Bandler, T. Chen

Simulation Optimization Systems Research Laboratory

and Department of Electrical and Computer Engineering

McMaster University, Hamilton, Canada L8S 4L7

Tel 905 525 9140 Ext. 24971

Fax 905 523 4407

Email tongch@pc-unsavory.sos.mcmaster.ca

Abstract	In this work, we discuss two compensation techniques [1] for a 90 right angle microstrip bend.  A broad frequency range is considered.  The microstrip bends are optimized to achieve the possible minimum reflection at each port.

- INTRODUCTION

In this report, we discuss two configurations to compensate the discontinuities of a microstrip

bend.  The microstrip bends concerned are symmetric and are connected to 50 Ω impedance at each port. A bend with a slit as shown in Fig. 1(a) is the first configuration.  The second compensation technique is to miter the bend as illustrated in Fig. 1(b).  The optimization variable for the bend in Fig. 1(a) is d1 with d2 equal to 1 mil, 3 mil, 4 mil.  It is noted that d1and d2 represent the length and the width of the slit, respectively.  For the configuration in Fig. 1(b), the optimization variable is d.

II.	DESIGN SPECIFICATIONS

The two configurations in Fig. 1 are optimized to achieve the possible minimum reflection at each port.  Therefore the design specifications is given as

S11=0

in the frequency range from 18 to 30 GHz.  The width w, the height h and the relative dielectric constant εr are fixed during optimization.  Their values are

w=15 mil	h=15 mil	εr=9.8

Three CAD packages are used for the simulation and optimization, Sonnet em [2] to compute the scattering parameters of the microstrip bends, OSA90/hope [3] to construct the circuit file and perform the optimization and Empipe [3] to parameterize the geometry of the bends.

III.	OPTIMIZATION RESULTS

The microstrip bend in Fig. 1(a) is optimized to satisfy the design specifications.  The length of

the slit d1 with the width of the slit d2 equal to 1 mil and 2 mil is considered.  We optimize the structure with respect to d1 at each value of d2.  The optimal values of d1 are given in Table I.  S11 at the optimal value of d1 for d2equal to 1 mil and 2 mil is shown in Fig. 2.  S11 of the basic 90 right angle microstrip bend without any compensation is also shown in Fig. 2 for the purpose of comparison.  It is noted that the optimal responses at d2 equal to 1 mil and 2 mil are good and very close to each other.

The compensation technique in Fig. 1(b) is to miter the basic 90 right angle bend.  The mitered

bend is optimized with respect to d.  The solution is unique and is given in Table II.  The optimal response of S11 is shown in Fig. 3 together with S11 of the basic bend for comparison.

Fig. 4 shows S11 in the frequency range 18 GHz to 30 GHz for the optimal structures of the configurations in Fig. 1 and the basic bend.  It is noted that the optimal bends give good response.

CONCLUSION

In this report, we presented two compensation techniques for a 90 right angle microstrip bend.  The two structures in Fig. 1 were proposed in literature [1] and they are optimized here.  Optimization is carried out with respect to proper dimensions to reach a possible minimum reflection at each port.  Sonnet em simulator as well as the robust optimizer in OSA90/hope have been used to perform direct optimization.  The optimization results indicate that the two compensation techniques are effective to reduce the effect of the discontinuities in a microstrip bend.

REFERENCES

[1]	E. H. Fooks, R. A. Zakarevicius, “Microwave Engineering Using Microstrip Circuits”, Prentice Hall, 1990

[2]	emTM and xgeomTM, Sonnet Software, Inc., 135 Old Cove Road, Suite 203, Liverpool, NY, 13090-3774, USA

[3]	OSA90hopeTM, EmpipeTM and Empipe3DTM, Optimization Systems Associates Inc., P.O. Box 8083, Dundas, Ontario, Canada L9H 5E7, 1994

TABLE I

MINIMAX OPTIMIZATION RESULTS OF THE BEND WITH A SLIT

| CAD Software | d2 (mil) | Optimal Value of d1 (mil) |
| --- | --- | --- |
| Sonnet em, Empipe, OSA90/hope | d2=1 | 8.11025 |
|  | d2=2 | 6.17696 |

TABLE II

MINIMAX OPTIMIZATION RESULTS OF THE MITERED BEND

| CAD Software | Optimal Solutions (mil) |
| --- | --- |
| Sonnet em, Empipe, OSA90/hope | d=18.8257 |

(a)								(b)

Fig. 1	The illustration of two compensation techniques

Fig. 2	S11 for the basic 90 right angle microstrip bend and the optimal bend

in Fig. 1(a) with d2 equal to 1mil and 2 mil

Fig. 3	S11 for the basic 90 right angle microstrip bend and

the optimal mitered bend in Fig. 1(b)

Fig. 4	S11 for the basic 90 right angle microstrip bend and

the optimal bends shown in Fig. 1 (a) and (b)