OPTIMAL MICROSTRIP BENDS

J.W. Bandler and T. Chen

SOS-98-18-V

August 1998

 J.W. Bandler and T. Chen 1998

No part of this document may be copied, translated, transcribed or entered in any form into any machine without written permission.  Address inquiries in this regard to Dr. J.W. Bandler.  Excerpts may be quoted for scholarly purposes with full acknowledgment of source.  This document may not be lent or circulated without this title page and its original cover.

OPTIMAL MICROSTRIP BENDS

# J.W. Bandler and T. Chen

Simulation Optimization Systems Research Laboratory

and Department of Electrical and Computer Engineering

McMaster University, Hamilton, Canada L8S 4L7

presented at

SOS Research Laboratory Meeting, Hamilton, August 19, 1998.

Introduction

this work presents two compensation techniques for the discontinuities of a 90 right angle microstrip bend

direct optimization is applied to achieve an optimal structure, which minimizes the effect of the discontinuities

in this work, the goal of optimization is to minimize the reflection at each port

the bends considered here are symmetrical and are connected to 50 ohm impedance at each port

OSA90/hope, Empipe and Sonnet em are used as the optimization and simulation tools

## Introduction

the configurations in Fig. (1) and Fig. (2) present two compensation techniques (E. H. Fooks, R. A. Zakarevicius, “Microwave Engineering Using Microstrip Circuits”, Prentice Hall, 1990)

w=15 mil, h (height of substrate)=15 mil and εr=9.8

### Fig. 1	a mitered bend

## Introduction

### Fig. 2	a bend with a slit

##### Design Specifications

zero is the ideal value of magnitude of the reflection coefficient at each port

the design specification used here is

MS11=0

From 18 GHZ to 30 GHZ, step=3 GHZ

## Optimization Results

for the compensation technique shown in Fig. 1, d is the optimization variable

the optimal solution is d=1.3103w

the optimal response of S11 is shown in Fig. 3 together with S11 of the basic 90º right angle bend for comparison

Fig. 3	S11 for the basic 90 right angle microstrip bend and

the optimal mitered bend in Fig. 1

## Optimization Results

for the compensation technique shown in Fig. 2, the optimization variable is d1 at d2 equal to 1 mil, 2 mil

the optimal values of d1 were given in Table I

S11 at the optimal value of d1 for d2equal to 1 mil and 2 mil is shown in Fig. 4

S11 of the basic 90 right angle microstrip bend without any compensation is also shown in Fig. 4 for comparison

### Table I

| d2 (mil) | The optimal value of d1 (mil) |
| --- | --- |
| 1 | 8.11025 |
| 2 | 6.17696 |

## Optimization Results

Fig. 4	S11 for the basic 90 right angle microstrip bend

and the optimal bend in Fig. 2 with d2 equal to 1mil and 2 mil

## Optimization Results

Fig. 5 shows S11 in the frequency range 18 GHz to 30 GHz for the optimal structures of the two configurations and the basic 90 right angle microstrip bend

it is noted that the optimal bends give good response

Fig. 5	S11 for the basic 90 right angle microstrip bend and

the optimal bends shown in Fig. 1 and Fig. 2

## Conclusions

##### two compensation techniques are applied to a 90 right angle microstrip bend

the performance of the two techniques is optimized by direct optimization with Sonnet em and Empipe

both of the two techniques can noticeably reduce the reflection at each port