A SUGGESTED ALGORITHM FOR MULTI-POINT
PARAMETER EXTRACTION

M.H. Bakr and J.W. Bandler

SOS-98-25-V

August 1998

 M.H. Bakr and J.W. Bandler 1998

No part of this document may be copied, translated, transcribed or entered in any form into any machine without written permission.  Address inquiries in this regard to Dr. J.W. Bandler.  Excerpts may be quoted for scholarly purposes with full acknowledgment of source.  This document may not be lent or circulated without this title page and its original cover.

A SUGGESTED ALGORITHM FOR MULTI-POINT
PARAMETER EXTRACTION

# M.H. Bakr and J.W. Bandler

Simulation Optimization Systems Research Laboratory

and Department of Electrical and Computer Engineering

McMaster University, Hamilton, Canada L8S 4L7

presented at

SOS Research Laboratory Meeting, Hamilton, August 24, 1998.

Classification of the Solution of Parameter Extraction

a solution xose of the parameter extraction problem is labeled locally unique if there exists an open neighborhood of xose containing no other pointxossuch that R(xos)=R(xose), where R is the vector of matched coarse model responses

it can be shown that the local uniqueness condition is equivalent to the condition that the Jacobian of the vector of matched coarse model responses R has a rank n, where n is the number of optimizable parameters.

(a) non locally unique                        b) locally unique

Problem Definition

## assume that multi-point parameter extraction is carried out at the fine model point xemusing Ne fine model points

it follows that the vector of matched coarse model responses R is given by

R=Ros(xos)Ros(xos+Δx(1))..Ros(xos+Δx(Ne−1))

where Δx(i)∈Vp; the set of utilized perturbations

it is required to find the perturbation Δx that can be added to the set VPand maximizes the improvement of the uniqueness of the extraction step

Problem Assumptions

the coarse model is assumed to be much faster than the fine model

few extra coarse model simulations add negligible overhead to the computational time of the problem

it is also assumed that first and second order derivatives of the coarse model responses can be obtained

in the absence of information about the mapping between the two spaces we take B=I

the mapping B can be easily integrated with the suggested algorithm if it is available

A Suggested Method: the Non Locally Unique Case

assume that the rank of the Jacobian of matched coarse model responses at the point xose is k < n

we impose the condition that the gradients of (nk) of the responses generated by the new coarse model point xose+Δx be normal to the gradients of a linearly independent set of gradients of cardinality k of the responses in the vector R

define the set of linearly independent gradients by

S=g(1),...,g(k)

the gradient of each of the (nk) selected responses can be approximated by

ga(i)=g(i)+G(i)Δx,  i=k+1, . ., n

where g(i)is the gradient of the ith response at the point xose and G(i)is the corresponding Hessian

the perturbation Δx that satisfies the orthogonality condition is obtained by solving

Suggested Method: the Non Locally Unique Case

ATΔx=−c

where

A=G(k+1)g(1).  .G(n)g(1).....G(n)g(k)

and

c=g(k+1)Tg(1).g(n)Tg(1).g(n)Tg(k)

this system of linear equations may be under-determined, over-determined or well-determined

the pseudoinverse of the matrix AT is used to find the solution of minimum length in all cases

the perturbation Δx is rescaled to satisfy a certain trust region condition

rescaling implies that the gradients might not be orthogonal but the independency property is retained

A Suggested Method: the Locally Unique Case

assume that a solution for the multi-point parameter extraction xos,1e was obtained and that the rank of the Jacobian of matched coarse model responses is k=n

a perturbation Δx is sought that distinguishes this locally unique minimum from other minima that may exist

assume the presence of another locally unique minimum xos,2e

the quadratic models for the objective function in the neighborhood of these two minima are given by, respectively

q1(Δx)=f1+0.5ΔxTH1Δx

and

q2(Δx)=f2+0.5ΔxTH2Δx

where f1 and f2are the values of the objective function at the points xos,1e and xos,2e, respectively and H1 and H2 are the corresponding Hessian matrices

it can be shown that the direction that maximizes the difference between the two quadratic models is an eigenvector for the matrix (H1 – H2)

Suggested Method: the Locally Unique Case

the matrix H2 is assumed to be the identity as there is no available information about the other minima that may exist

it follows that the direction of Δx is an eigenvector for the matrix H1

the obtained direction of Δx supplies a direction with a high rate of change of the coarse model responses and is a characteristic of the minimum xos,1e

proper scaling is applied to the obtained perturbation Δx to satisfy a certain trust region condition

a scheme was developed to select one of the eigenvectors of the matrix H1 on a sequential basis

if all the perturbations with certain trust region size are exhausted the size of the trust region is increased

An Alternative Perturbation for the Locally Unique Case without Using Second Order Derivatives

a perturbation of Δx results in a perturbation of the coarse model responses at the two minima by

ΔR1=Jos(xos,1e)Δx

and

ΔR2=Jos(xos,2e)Δx

we impose the condition that the difference between the ℓ2norms of these two response perturbations be maximized subject to certain trust region size

it follows that the following Lagrangian can be formed

L(Δx,λ)=ΔxTJos(xos,1e)TJos(xos,1e)Δx−ΔxTJos(xos,2e)TJos(xos,2e)Δx                                                                              +λ(ΔxTΔx−δ2)

it can be shown that the perturbation Δx is an eigenvector for the matrix Jos(xos,1e)TJos(xos,1e)−Jos(xos,2e)TJos(xos,2e)

the perturbation is then scaled to satisfy the length condition

again we make the assumption that Jos(xos,2e)TJos(xos,2e)=I because of the lack of information about other minima

The Algorithm Flowchart

Given xem,   and n.

Initialize V=xem and i=1

J(xose) has rank n ?

No

Is V=1?

Yes

No

Is xose approaching a limit ?

Obtain a new perturbation Δxusing second method and add the pointxem+Δxto the set V.  Set i=i+1.

Apply multi-point parameter extraction using the points in the set V to get xose

Obtain a new perturbation Δxusing first method and add the pointxem+Δxto the set V.  Set i=i+1.

Yes

Yes

Stop

No

Example 1: the Rosenbrock Function

the coarse model is

Ros=100(u2−u12)2+(1−u1)2

the fine model is

Rf=100((u2+0.2)−(u1−0.2)2)2+(1−(u1−0.2))2

it is required to extract the vector of coarse model parameters at the point [1.0    1.0]T

three fine model points were needed for the algorithm to terminate

the variation of the extracted parameters with the number of fine model points is shown in the following table

| Number of Points | xos,1e | xos,2e |
| --- | --- | --- |
| 1 | 1.21541 | 0.91728 |
| 2 | 0.80008 | 1.20012 |
| 3 | 0.80008 | 1.20014 |

The Contours of the L2 Objective Function for the Rosenbrock Function

(a) single-point extraction

(b) two-point extraction

The Contours of the L2 Objective Function for the Rosenbrock Function

(c) three-point extraction

Example 2: a Quadratic Function

the coarse model for this problem is given by

Ros=x12+x22

the fine model for this problem is given by

Rem=(0.9x1+0.1x2)2+(0.1x1+0.9x2)2

it is required in this problem to extract the coarse model parameters corresponding to the fine model parameters

xem=[2.0    1.0]T

four fine model points were needed to ensure the uniqueness of the extracted parameters

The Variation of the Extracted Parameters with the Number of Fine Model Points for the Quadratic Function

the following table shows the variation of the extracted coarse model parameters with the number of fine model points used in the multi-point parameter extraction

| Number of Points | xos,1 | xos,2 |
| --- | --- | --- |
| 1 | 1.95724 | 0.99458 |
| 2 | 2.10283 | 0.63094 |
| 3 | 1.92787 | 1.05337 |
| 4 | 1.89571 | 1.10868 |

the exact solution for the parameter extraction problem is

xos=[1.9      1.1]T

The Contours of the L2 Objective Function for the Quadratic Function

(a) single-point extraction

(b) two-point extraction

The Contours of the L2 Objective Function for the Quadratic Function

(c) three-point extraction

(d) four-point extraction

Example 3: the 10:1 Impedance Transformer

the parameters for this problem are the characteristic impedances of the two transmission lines

the lengths of both lines are kept fixed at their optimal values (quarter wave length)

the coarse model is an ideal 10:1 impedance transformer

the fine model scales each of the two impedances by 1.6

the responses of both models at 11 different frequencies in the frequency band  0.5 GHz  f  1.5 GHz were used to match the two models

it is required to extract the coarse model parameters corresponding to the fine model point [2.2628   4.5259]T

three fine model points were needed to improve the uniqueness of the problem

The Matched Responses for the 10:1 Transformer (Single-Point Extraction)

the set of fine model points utilized in parameter extraction is

V=2.262774.52592

the extracted coarse model parameters are

xose=3.620437.24147

the matched responses are shown in the following figure

The Contours of the L2 Objective Function for the 10:1 Transformer (Single-Point Extraction)

The Matched Responses for the 10:1 Transformer (Two-Point Extraction)

the set of fine model points utilized in parameter extraction is

V=2.262774.52592,1.499754.76634

the extracted coarse model parameters are

xose=3.47167.43214

the matched responses of every fine model point are shown

The Contours of the L2 Objective Function for the 10:1 Transformer (Two-Point Extraction)

The Matched Response for the 10:1 Transformer (Three-Point Extraction)

the set of used fine model points is

V=2.262774.52592,1.499754.76634,3.020244.26855

the extracted coarse model parameters are xose=3.603577.35052

the matched responses of every fine model point are shown

The Contours of the L2 Objective Function for the 10:1 Transformer (Three-Point Extraction)