AGGRESSIVE SPACE MAPPING WITH MULTIPLE POINT

PARAMETER EXTRACTION: A STEP BY

STEP ILLUSTRATION USING TWO

# ROSENBROCK FUNCTIONS

J.W. Bandler and J.E. Rayas-Sánchez

SOS-98-26-R

August 1998

 J.W. Bandler and J.E. Rayas-Sánchez 1998

No part of this document may be copied, translated, transcribed or entered in any form into any machine without written permission.  Address inquiries in this regard to Dr. J.W. Bandler.  Excerpts may be quoted for scholarly purposes with full acknowledgment of source.  This document may not be lent or circulated without this title page and its original cover.

AGGRESSIVE SPACE MAPPING WITH MULTIPLE POINT PARAMETER EXTRACTION: A STEP BY STEP ILLUSTRATION USING

TWO ROSENBROCK FUNCTIONS

J.W. Bandler and J.E. Rayas-Sánchez

Simulation Optimization Systems Research Laboratory

and Department of Electrical and Computer Engineering

McMaster University, Hamilton, Canada L8S 4L7

Tel 905 525 9140  Ext. 24305

Fax 905 523 4407

Email: rayassje@mcmaster.ca

Abstract  The Aggressive Space Mapping (ASM) algorithm employing multiple point parameter extraction is illustrated in this report, making use of two analytical Rosenbrock functions as fine and coarse models. The aggressive Space Mapping concept is reviewed.  The improvement in the uniqueness of the parameter extraction due to multiple point matching is graphically illustrated.  The ASM algorithm is shown in a step by step fashion.  The corresponding space mapped solution is obtained and verified with the direct fine solution.

I.  Introduction

Space Mapping (SM) is a novel concept for circuit design and optimization that combines the computational efficiency of coarse models with the accuracy of fine models.  The coarse models are typically empirical circuit-equivalent engineering models, which are computationally very efficient but often have a limited validity range for its parameters, beyond which the simulation results become very coarse.  On the other hand, the fine models may be provided by an electromagnetic simulator, or even by direct lab measurements; they are very accurate but demand considerable resources.

The SM technique establishes a mathematical link between the coarse and the fine models, and directs the bulk of CPU intensive optimization to the coarse model, while preserving the accuracy and confidence offered by the fine model.  The SM technique was originally developed by Bandler et al. [].

The Aggressive Space Mapping (ASM) algorithm using multiple point parameter extraction is illustrated in this report making use of two analytical Rosenbrock functions.  The original Rosenbrock function is taken as the coarse model, and a perturbed Rosenbrock function as the fine model. The aggressive Space Mapping concept is reviewed.  The improvement in the uniqueness of parameter extraction due to multiple point matching is graphically illustrated.  The ASM algorithm is shown in a step by step fashion.  The corresponding space mapped solution is obtained and verified with the direct fine solution.

II.  AGGRESSIVE SPACE MAPPING ALGORITHM

Let the vectors xc and xf represent the design parameters of the coarse and fine models, respectively, and Rc(xc) and Rf(xf) the corresponding model responses.  Rcis much faster to calculate but less accurate than Rf.

SM optimization consists in the generation of an appropriate mapping, P, from the fine model parameter space xf to the coarse model parameter space xc

| xc = P(xf) | (1) |
| --- | --- |

such that

| Rc(P(xf)) ≈ Rf(xf) | (2) |
| --- | --- |

The mapping is established iteratively.  In the original work [Error: Reference source not found], the initial mapping is established by performing upfront fine model analysis at a number of base points.  The aggressive SM strategy minimizes the upfront effort by targeting every fine simulation at optimizing the design and progressively refining the mapping [].

The aggressive SM concept can be illustrated by the use of the diagram in Fig. 1.  We initially perform conventional optimization using the coarse model to obtain the optimal coarse solution xc. Assuming that xf and xchave the same dimension, we choose the coarse optimal solution as a starting point for the fine model

| xf = xc | (3) |
| --- | --- |

and calculate the corresponding fine response Rf(xf).  Then we perform parameter extraction to find the optimal value of the coarse model parameters xc, such that the coarse response sufficiently match the fine response calculated,

| Rc(xc)»Rf(xf) | (4) |
| --- | --- |

If the extracted parameters xc are approximately the same as the optimal coarse solution xc, then we have the space-mapped solution xf and the optimization ends, otherwise, a new value of xf is generated using Broyden’s formula, the mapping is updated, and the optimization continues by calculating the corresponding new fine response (see Fig. 1).  To find the space-mapped solution xf means to find the fine model parameters xf whose response Rf(xf) matches the optimal coarse response Rc(xc).

Following [2], the ASM algorithm can be implemented as follows

Step 0.		Initialize xf(1) = xc, B(1) = 1, j = 1.

Step 1.		Evaluate Rf(xf(1)).

Step 2.		Extract xc(1) such that Rc(xc(1)) ≈ Rf(xf(1)).

Step 3.		Evaluate f(1) = xc(1) − xc.  Stop if f(1) ≤ η.

Step 4.		Solve B(j) h(j) = −f(j) for  h(j).

Step 5.		Set xf(j+1) = xf(j) +h(j).

Step 6.		Evaluate Rf(xf(j+1)).

Step 7.		Extract xc(j+1) such that Rc(xc(j+1)) ≈ Rf(xf(j+1)).

Step 8.		Evaluate f(j+1) = xc(j+1) − xc.  Stop if f(j+1) ≤ η.

Step 9.		Update B(j+1) = B(j)+f(j+1)h(j)Th(j)Th(j).

Step 10.		Set j = j+1; go to Step 4.

It is clear that the matrix B(j) obtained from Broyden’s formula represents the most up-to-date approximation to the mapping between the two spaces around the optimal coarse solution xc.  That is

| B(j)(xf(j+1)−xf(j)) = −(xc(j) − xc) | (5) |
| --- | --- |

Thus, when the space-mapped solution is reached, xf(j+1) = xf, the mapping can be obtained from (5) using the final value of B, as follows

| xc = P (xf) =B (xf−xf) + xc | (6) |
| --- | --- |

Using (6) the fine model could be replaced by the coarse model for fast response evaluations.

III.  coarse and fine models

Consider a Rosenbrock function as a coarse model, Rc(x), given by

| Rc (x) = 100 (x2 − x12)2 + (1−x1)2 | (7) |
| --- | --- |

where x = x1x2  are the design parameters.  The-3-D plot and the contours of the coarse model Rc(x) are illustrated in Fig. 2, which were obtained using MatLab [].  The well-known coarse optimal solution is at point xc = 11  and the coarse optimal value is Rc = Rc (xc) = 0.

A perturbed Rosenbrock function will be considered as a fine model, Rf(x), given by

| Rf (x) = 100 (u2 − u12)2 + (1−u1)2 | (8) |
| --- | --- |

where  u = u1u2 = x + −0.2+0.2.  The 3-D plot and the corresponding contours of the fine model are illustrated in Fig. 3.

IV.  MULTIPLE POINT PARAMETER EXTRACTION

According to the step 2 of the ASM algorithm previously described, we want to extract the corresponding parameters for the coarse model, x, such that Rc (x) ≈ Rf (xc).  Instead of minimizing

| Rc (x) − Rf (xc)p | (9) |
| --- | --- |

we will perform a multiple point parameter extraction to obtain a sharper objective function and improve the uniqueness of the parameter extraction, at the expense of more fine model evaluations.  The multiple point parameter extraction approach was introduced by Bandler et al.  [].  The new objective function considering five matching points is

| Rc (x+B(j)Δxi) − Rf (xc+Δxi)p | (10) |
| --- | --- |

where Δx1 = 00 , Δx2 = 00.5 , Δx3 = 0.50 , Δx4 = 0.5−0.5  and Δx5 = −0.5−0.5 were chosen arbitrary.

The points pi = xc+Δxi in the fine parameter space are being indicated in Fig. 3b.  We are considering that, at the jth iteration of the ASM algorithm, a perturbation in the fine parameter space Δxi corresponds to a perturbation B(j)Δxi in the coarse parameter space, as proposed in [].

Since the matrix B(j) = 1 for the first parameter extraction optimization, the corresponding five error functions Ei are given by

| Ei =Rc (x+Δxi) − Rf (xc+Δxi) | (11) |
| --- | --- |

If we consider only l1and l2 norms, the corresponding objective functions are

| ABSn = E1 + E2 + … +En | (12) |
| --- | --- |

and

| SQRn = (E1)2 + (E2)2 + … +(En)2 | (13) |
| --- | --- |

The l1 objective functions for the single point parameter extraction problem (ABS1) and for the multiple point parameter extraction problem taking four additional points (ABS5) are illustrated in Fig. 4, from where it can clearly be noticed the improvement in the uniqueness of the parameter extraction solution.  A similar result is illustrated in Fig. 5 for the l2 objective functions SQR1 and SQR5.  It can be visualized from the plots of ABS5 and SQR5 that the solution for the parameter extraction problem is x = 0.81.2 .  It can also be verified that

| Rc ( 0.81.2+Δx1) = Rf (xc+Δx1) = 31.4, Rc ( 0.81.2+Δx2) = Rf (xc+Δx2) = 112.4, Rc ( 0.81.2+Δx3) = Rf (xc+Δx3) = 24.1 , Rc ( 0.81.2+Δx4) = Rf (xc+Δx4) = 98.1, and Rc ( 0.81.2+Δx5) = Rf (xc+Δx5) = 37.7 . | (14a-e) |
| --- | --- |

V.  SPACE MAPPED SOLUTION

Now we will apply the ASM algorithm described in Section II to the coarse and fine models defined in Section III.

Step 0.		xf(1) = 11, B(1) = 1, j = 1

Step 1.		From (14a), Rf(xf(1)) = 31.4

Step 2.		From Figs. 4 and 5, when xc(1) = 0.81.2, Rc(xc(1)) = Rf(xf(1))

Step 3.		f(1) = 0.81.2 − 11 = −0.20.2

Step 4.		Since B(1) = 1,  h(1) = 0.2−0.2

Step 5.		Set xf(2) = 11 + 0.2−0.2 = 1.20.8

Step 6.		Rf(xf(2)) = 0

Step 7.		xc(2) = 11, because we knowRc(xc(2)) = 0

Step 8.		Since f(2) = xc(2) − xc = 00, the algorithm ends, and xf = xf(2) = 1.20.8.

For this particular problem, which involves two optimization variables, the Aggressive Space Mapping algorithm takes only one iteration.  It can be verified that the space-mapped solution obtained is equal to the exact fine solution (see Fig. 3b).

The mapping between the coarse and the fine parameter spaces can be obtained using (6) as follows

| xc = P (xf) = 1001 (xf−1.20.8) + 11 = xf+−0.20.2, | (15) |
| --- | --- |

which corresponds to the original perturbation that relates the coarse and the fine models (8).  Therefore, the mapping obtained is valid for the whole range of parameter values.

VI.  Conclusions

The Aggressive Space Mapping (ASM) algorithm with multiple point parameter extraction was illustrated in this report making use of two analytical Rosenbrock functions.  The original Rosenbrock function was taken as the coarse model, and a perturbed Rosenbrock function as the fine model.  The improvement in the uniqueness of the parameter extraction due to multiple point matching was graphically illustrated.  The ASM algorithm was executed in a step by step fashion, and the corresponding space mapped solution was accomplished after only one iteration.  The mapping calculated by the ASM algorithm is equal to the original linear mapping between both parameter spaces.

References

[?] 	J.W. Bandler, R.M. Biernacki, S.H. Chen, P.A. Grobelny and R.H. Hemmers, “Space mapping technique for electromagnetic optimization,” IEEE Trans. Microwave Theory Tech., vol. 42, 1994, pp. 2536-2544.

[?] 	J.W. Bandler, R.M. Biernacki, S.H. Chen, R.H. Hemmers and K. Madsen, “Electromagnetic optimization exploiting aggressive space mapping,” IEEE Trans. Microwave Theory Tech., vol. 43, 1995, pp. 2847-2882.

[?] 	MatLab, The MathWorks, Inc., 24 Prime Parkway, Natick MA 01760-9889, 1997.

[?] 	J.W. Bandler, R.M. Biernacki, and S.H. Chen, “Fully automated space mapping optimization of 3-D structures,” IEEE MTT-S Int. Microwave Symp. Dig. (San Francisco, CA), 1996, pp. 753-756.

[?] 	M.H. Bakr, J.W. Bandler, R.M. Biernacki, S.H. Chen and K. Madsen, “A trust region aggressive space mapping algorithm for EM optimization,” IEEE MTT-S Int. Microwave Symp. Dig. (Baltimore, MD), 1997, pp. 1759-1762.

Fig. 1.  Space Mapping concept.

(a)

(b)

Fig. 2.  Original Rosenbrock function or coarse response, Rc:  (a) surface plot;

(b) contours for levels 0.0001, 0.7, 7, 70, 200, 350, 550, 700.

(a)

(b)

Fig. 3.  Perturbed Rosenbrock function or fine response, Rf:  (a) surface plot;

(b) contours for levels 0.0001, 0.7, 7, 70, 200, 350, 550, 700.

(a)

(b)

Fig. 4.  l1 objective function for the parameter extraction problem: (a) single point parameter

extraction; (b) multiple point parameter extraction (with 4 additional points).

(a)

(b)

Fig. 4.  l2 objective function for the parameter extraction problem: (a) single point parameter

extraction; (b) multiple point parameter extraction (with 4 additional points).

  This work was supported in part by the Natural Sciences and Engineering Research Council of Canada under Grants OGP0007239, OGP0042449 and STP0201832 and through the Micronet Network of Centres of Excellence. J.E. Rayas-Sánchez is funded by CONACYT (Consejo Nacional de Ciencia y Tecnología, Mexico), as well as by ITESO (Instituto Tecnológico y de Estudios Superiores de Occidente, Mexico).