# A generalized space mapping TABLEAU approach

# to microwave device modeling

M.A. Ismail

SOS-98-43-V

December 1998

 M.A. Ismail 1998

No part of this document may be copied, translated, transcribed or entered in any form into any machine without written permission.  Address inquiries in this regard to Dr. J.W. Bandler.  Excerpts may be quoted for scholarly purposes with full acknowledgement of source.  This document may not be lent or circulated without this title page and its original cover.

A GENERALIZED SPACE MAPPING APPROACH TO MICROWAVE CIRCUITS AND DEVICE MODELING

M.A. Ismail

Simulation Optimization Systems Research Laboratory

and Department of Electrical and Computer Engineering

McMaster University, Hamilton, Canada L8S 4L7

presented at

SOS Research Laboratory Meeting, Hamilton, December, 1998

Basic Concepts

Space Mapping (SM) is a powerful tool for circuit design and optimization

two model types are usually defined in the SM: a “coarse” model, typically an empirical model, and a “fine” model, typically a full wave electromagnetic simulator

Generalized Space Mapping (GSM)

GSM is a comprehensive framework to engineering device modeling

GSM exploits the Space Mapping (SM), the Frequency Space Mapping (FSM) and the Multiple Space Mapping (MSM) concepts to build a new engineering device modeling framework

two cases are considered: the basic Space Mapping Super Model (SMSM) concept and the Frequency-Space Mapping Super Model (FSMSM) concept

Space Mapping Super Model (SMSM)

Frequency-Space Mapping Super Model (FSMSM)

Multiple Space Mapping (MSM) Concept

##### MSM for different responses

Multiple Space Mapping (MSM) Concept

##### MSM for different frequency ranges

Mathematical Formulation for GSM

the kth mapping targeting the sub-response Rkor the response R in the kth frequency sub-range is given by

(xck,ωck)=Pk(xf,ω)

or, in matrix form, assuming a linear mapping,

xck

the mapping parameters ck, Bk,sk,tk σk, δk can be evaluated, directly or indirectly, by solving the optimization problem

minck, Bk,sk,tk σk, δk  [ek1Tek2T⋯ekmT]T

where m is the number of base points selected in the fine model space and ekj is an error vector given by

ekj = Rf(xf(j),ω)−Rc(xck(j),ωck) , j=1,2,...,m

an important variation of the mapping is to use the inverse of the frequency variable (which is proportional to the wavelength) instead of the frequency itself

An Implementation of SMSM and FSMSM

the SMSM for the kth mapping can be evaluated through the following steps

choose a set of m base points xf(j), j=1,2,...,m in the fine model space

for each base point xf(j) the corresponding point xc(j)in the coarse model space can be obtained by solving a parameter extraction problem

the kth mapping parameters ck, Bk can then be evaluated by least-squares

in FSMSM we evaluate ck, Bk as in SMSM and then obtain  σk, δk by solving

min sk,tk,σk, δk  [ek1Tek2T⋯ekmT]T

considering ck, Bk fixed

FSMSM for Microstrip Right Angle Bend

| Parameter | Minimum value | Maximum value |
| --- | --- | --- |
| W | 20 mil | 30 mil |
| H | 8 mil | 16 mil |
| εr | 8 | 10 |

the fine model is analyzed by Sonnet’s em and the “coarse” model is Jansen empirical model

the fine model parameters are given by

xf=[ W H εr]T

the coarse model parameters are given by

xc=[W1 H1 εr1]T

FSMSM for the Right Angle Bend

(before applying SMSM or FSMSM)

| (after applying SMSM) | (after applying FSMSM) |
| --- | --- |

MSM-FSMSM for the Right Angle Bend

| Frequency range | Bk | ck | k | k |
| --- | --- | --- | --- | --- |
| 1 GHz to 11 GHz | 1 | 0 | 1 | 0 |
| 11 GHz to 21 GHz | 1.81627 0.81981 0.91555 0.44352 0.30230 0.02315 | -22.7 -3.822 | 1.297 | -4.765 |
| 21 GHz to 31 GHz | 2.33221 1.98365 2.17360 0.53802 0.54226 0.45435 | -49.41 - 9.324 | 1.452 | -10.964 |
| 31 GHz to 35 GHz | 2.77533 3.06006 2.92395 0.69924 0.81860 0.59300 | -71.5 -15.2 | 1.446 | -15.765 |
| 35 GHz to 41 GHz | 3.94219 4.61833 4.95400 1.12763 1.33404 1.46545 | -125.4 -36.13 | 1.977 | -36.335 |

MSM-FSMSM for the Right Angle Bend

the error in S11 and S21 before applying MSM-FSMSM

the error in S11 and S21 after applying MSM-FSMSM

Conclusions

we introduce a comprehensive framework called Generalized Space Mapping (GSM) to engineering device modeling

In GSM we utilize a few relevant full-wave EM simulations to match the responses of the fine model and the coarse model over a designable region of parameters and frequency

GSM generalizes the Space Mapping (SM), the Frequency Space Mapping (FSM) and the Multiple Space Mapping (MSM) concepts to build a new engineering device modeling framework

two fundamental concepts are presented:  one is a basic Space Mapping Super Model (SMSM) and the other is a basic Frequency-Space Mapping Super Model (FSMSM)

MSM can be combined with SMSM and FSMSM to provide a powerful and reliable modeling tool for microwave devices

more research is being carried out to efficiently evaluate the mappings defined in GSM and to apply the generic GSM concept to enhance the accuracy of microwave circuits