A HYBRID AGGRESSIVE SPACE MAPPING

ALGORITHM FOR EM OPTIMIZATION

M.H. Bakr, J.W. Bandler, N. Georgieva and K. Madsen

SOS-99-9-R

March 1999

 M.H. Bakr, J.W. Bandler, N. Georgieva and K. Madsen 1998

No part of this document may be copied, translated, transcribed or entered in any form into any machine without written permission.  Address inquiries in this regard to Dr. J.W. Bandler.  Excerpts may be quoted for scholarly purposes with full acknowledgment of source.  This document may not be lent or circulated without this title page and its original cover.

A HYBRID AGGRESSIVE SPACE MAPPING

ALGORITHM FOR EM OPTIMIZATION

Mohamed H. Bakr, Student Member, IEEE, John W. Bandler, Fellow, IEEE,

Natalia Georgieva, Member, IEEE, and Kaj Madsen

Abstract	We propose a novel Hybrid Aggressive Space Mapping (HASM) optimization algorithm.  HASM exploits both the Trust Region Aggressive Space Mapping (TRASM) strategy and direct optimization.  Severe differences between the coarse and fine models and nonuniqueness of the parameter extraction procedure may cause the TRASM algorithm to be trapped in local minima.  The HASM algorithm is based on a novel lemma that enables smooth switching from the TRASM optimization to direct optimization if the TRASM algorithm is not converging.  It also enables switching back from direct optimization to the TRASM algorithm in a smooth way.  The uniqueness of the extraction step is improved by utilizing a good starting point.  The algorithm does not assume that the final space-mapped design is the true optimal design and is robust against severe misalignment between the coarse and fine models.  The new algorithm has been tested on designs of several microwave filters and transformers.  The examples include a three-section and a seven-section waveguide transformer as well as the design of an H-plane waveguide filter and a double-folded stub filter.

I. INTRODUCTION

In this work a Hybrid Aggressive Space Mapping (HASM) algorithm is presented.  Space Mapping (SM) optimization [1-4] assumes that the circuit under consideration can be simulated using two models: a fine model and a coarse model.  The fine model is accurate but is computationally intensive, e.g., a full-wave EM simulator.  The coarse model is assumed to be fast but not very accurate.  SM optimization directs most of the optimization computational effort towards the coarse model while maintaining the accuracy of the fine model.  The overall computational effort needed is much smaller than that for direct optimization.

Parameter extraction is crucial to the Aggressive Space Mapping (ASM) technique [4].  Here, a coarse model point whose response matches a given fine model response is obtained.  This is essentially an optimization problem.  The nonuniqueness of the extracted parameters may lead to divergence or oscillation of the iterations [2].  To alleviate this problem the TRASM algorithm was introduced [3].  TRASM integrates a trust region methodology [5] with the ASM technique.  Also, it utilizes a recursive multi-point parameter extraction in order to improve the uniqueness of the extraction step.

In this work we address the convergence behavior of the TRASM algorithm.  We discuss the effect of a severely misaligned coarse model.  We show that in this case TRASM optimization may be trapped in local minima.

The design obtained by pure SM optimization in most cases is very near optimal.  However, the optimality of the final design can not be guaranteed.  This is because the final space-mapped response matches the optimal coarse model response, which may be different from the optimal fine model response obtained by solving the original design problem in the fine model space.

Our HASM algorithm is designed to overcome these limitations.  The algorithm switches smoothly between SM optimization and direct optimization and vice versa.  A novel lemma enables such a switch.  The algorithm also integrates a novel prediction of the starting point for the parameter extraction problem to enhance the uniqueness.

We start by giving a brief review of SM algorithms in Section II.  Some properties of SM optimization that justify utilizing a hybrid algorithm are discussed in Section III.  The novel lemma and the prediction approach are introduced in Sections IV and V, respectively.  The HASM algorithm is presented in Section VI.  The algorithm is applied successfully to the design of microwave transformers and filters.  The examples include the design of a three-section waveguide transformer, a seven-section waveguide transformer, an H-plane waveguide filter and a double-folded stub microstrip filter.  These examples are given in Section VII.  Finally, the conclusions are given in Section VIII.

II. SPACE MAPPING: A BRIEF REVIEW

We refer to the vectors of “fine” model parameters and “coarse” model parameters as xemand xos, respectively.  The first step in any SM algorithm is to obtain the optimal design of the coarse model xos.  The corresponding response is denoted by Ros.  ASM aims at establishing a mapping P between the two spaces [4]

xos=P(xem)                                                                    (1)

such that

Rem(xem)−Ros(xos)≤ε                                                        (2)

where Rem is the vector of fine model response, Ros is the vector of coarse mode response and  is a suitable norm.  We define the error function

f=P(xem)−xos                                                                 (3)

The final fine model design is obtained and the mapping is established by solving the nonlinear system

f(xem) = 0                                                                        (4)

Let xem(i) be the ith iterate in the solution of (4).  In the ASM technique, the next iterate xem(i+1) is found by a quasi-Newton iteration

xem(i+1)=xem(i)+h(i)                                                                 (5)

where h(i) is obtained from

B(i)h(i)=−f(xem(i))                                                               (6)

and B(i)is an approximation to the Jacobian of the vector f with respect to xem at the ith iteration.  The matrix B is updated at each iteration using Broyden’s update [6].

Vector f is obtained by evaluating P(xem), which is done indirectly through parameter extraction.  The nonuniqueness of this problem may lead to divergence or oscillation of the ASM technique.  The TRASM algorithm [3] was designed to overcome this problem.  At the ith iteration, the residual vectorf(i)=P(xem(i))−xos defines the difference between the vector of extracted coarse model parameters xos(i)=P(xem(i)) and the optimal coarse model design.  The value f(i) serves as a measure of the misalignment between the two spaces in the ith iteration.  The TRASM algorithm aims at minimizing f(i).  The ith iteration of the algorithm is given by

(B(i)TB(i)+λI)h(i)=−B(i)Tf(i)                                                   (7)

Parameter  is selected such that the step obtained satisfies h(i)≤δ, where   is the size of the trust region.

III. SOME PROPERTIES OF SPACE MAPPING

ASM and TRASM are efficient algorithms.  The number of fine model simulations needed to obtain the space-mapped design is of the order of the problem dimensionality.  However, both algorithms depend on the existence of a coarse model that is fast and has enough accuracy.

# If the coarse model is bad (i.e., very different from the fine model) space mapping might not work.  To illustrate this we consider the Rosenbrock function [7].  We form an artificial problem in which the “coarse” model is given by

Ros=100(x2−x12)2+(1−x1)2                                                         (8)

and the “fine” model by

Rem=100((x2+α2)−(x1+α1)2)2+(1−(x1+α1))2                                        (9)

where α1and α2 are constant shifts.  The target of the direct optimization problem is to minimize Rem.  Considering (8) and (9) we notice that xos = [1.0   1.0]T and xem = (xos ) where   = [α1   α2]T.  The misalignment between the two models is thus given by the two shifts α1and α2.

We discuss two sets of values for the shifts.  First, we consider  = [0.1   0.1]T.  Using (8) and (9) we notice that the coarse model point whose response matches the fine response at a point xem is xos= (xem+ ).  It follows that the mapping between the two spaces is given by P(xem)=xem+α.  The contours of xem+α−xos22 are shown in Fig. 1(a).  The mapping P(xem) is then approximated through multi-point parameter extraction [3].  Only one perturbed fine model point is utilized.  The contours of P(xem)−xos22 obtained in this manner are shown in Fig. 1(b).  Figs. 1(a) and 1(b) show that f22 has a single minimum which is the solution that would have been obtained by direct optimization.  The differences between the two plots are attributed to the nonuniqueness of the parameter extraction process.  Taking the point xos as the initial solution of the fine model, the TRASM algorithm is expected to converge to xem.  The corresponding contours of Rem are shown in Fig. 1(c), whose minimum value is at [1.1   1.1]T as expected.

The same steps are repeated for the case   = [1.5   1.5]T.  The contour plot of xem+α−xos22 is shown in Fig. 2(a).  The contour plot of P(xem)−xos22 obtained using parameter extraction is shown in Fig. 2(b).  Fig. 2(b) illustrates the existence of a minimum of P(xem)−xos22 other than xem which is closer to the starting point of the TRASM algorithm xos.  It follows that the TRASM algorithm is unlikely to converge to xem.  The corresponding contours of Rem for this case are shown in Fig. 2(c), whose minimum value is at [2.5   2.5]T.

The solution obtained using SM optimization is very near optimal if Ros is similar to the optimal fine model response Rem.  However, this can not be guaranteed.  For example, consider

Rem=100(x2−x12)2+(1−x1)2                                                     (10)

Assume also that

Ros=Rem+ε                                                                 (11)

where  > 0.  It is clear that Ros is equal to  while Rem is zero.  It follows that space mapping may converge to a solution other than xem.

###### IV. SPACE MAPPING AND DIRECT OPTIMIZATION

The properties of space mapping suggest that a hybrid algorithm be used.  This algorithm exploits the efficiency of space mapping and defaults to direct optimization when space mapping fails.  Our HASM algorithm utilizes a novel lemma that enables smooth switching between the TRASM algorithm and direct optimization and vice versa.

Lemma Assume that xos corresponds to xem through a parameter extraction process.  Then the Jacobian Jemof the fine model response at xem and the Jacobian Jos of the coarse model response at xos are related by

Jem=JosB                                                                                                        (12)

where B is a valid mapping between the two spaces at xos and xem.

Proof

As the points xem and xoscorrespond to each other, it follows that their responses match, i.e.,

Rem(xem)=Ros(xos)                                                                                         (13)

Now define a new fine model point xn=xem+Δxem where Δxemis a small perturbation.  The response at this new point is perturbed from the response at the point xem by

ΔR=JemΔxem                                                                                                    (14)

The point xncorresponds to a coarse model point xos+Δxos that satisfies

ΔR=JemΔxem=JosΔxos                                                                                   (15)

Also, by definition of the mapping B the two perturbations ΔxemandΔxosare related by

BΔxem=Δxos                                                                                                   (16)

multiplying both sides of (16) by Joswe get

JosBΔxem=JosΔxos                                                                                            (17)

Comparing (17) with (15) we conclude that

Jem=JosB                                                                                                    (18)

Relation (18) is interesting.  It shows that by having the matrix B and the coarse model Jacobian Joswe are able to obtain a good estimate of the Jacobian of the fine model response without any further fine model simulations.  It follows that when SM optimization is not converging smoothly we can switch to direct optimization and supply it with the available first order derivatives given by (18).

Another relationship which can be easily obtained from (18) is

B=Jos TJos - 1Jos TJem                                                             (19)

Relation (19) assumes that Josis  full rank and m  n, where n is the number of parameters and m is the dimensionality of both Rem and Ros.  It is used for switching back from direct optimization to SM optimization.  Fig. 3 illustrates the switching between SM optimization and direct optimization.

We illustrate the lemma as follows.  Consider

Rem=(0.9x1+0.1x2)2+(0.1x1+0.9x2)2                                                 (20)

and

Ros=x12+x22                                                                      (21)

Take xem=[2.0    1.0]T.  Here Rem = 4.82.  The solution for the parameter extraction problem is xos=[1.90    1.10]T.  The Jacobian Josat xos is

Jos=3.82.2                                                                 (22)

From (20) and (21) it is seen that

B=0.90.10.10.9                                                                 (23)

It follows that Jemat xemis estimated by

Jem=3.82.20.90.10.10.9=3.642.36                                        (24)

which is the exact Jacobian of the fine model response.

###### V. SELECTION OF THE STARTING POINT

The discussion in Section III reveals how the nonuniqueness of the parameter extraction process can affect the convergence of SM optimization.  The uniqueness of this procedure can be improved by utilizing a good starting point.  In the first iteration of the algorithm there is no available information about the mapping between the two spaces.  A reasonable assumption is to take xos as the starting point for the parameter extraction optimization problem.  As the algorithm proceeds the matrix B(i)approximates the mapping between the two spaces.  A prediction of the extracted parameters in the ith iteration is given by

xos(p)=xos(i)+B(i)xem(i+1)−xem(i)                                                  (25)

This predicted point is then taken as a starting point for the parameter extraction optimization problem.  It supplies a good starting point provided that xos(i) is a valid solution to the parameter extraction in the previous iteration and B(i)approximates the mapping between the two spaces.

###### VI. THE HASM ALGORITHM

The HASM algorithm exploits SM when effective, otherwise it defaults to direct optimization.  Two objective functions are utilized by the algorithm.  The first objective function is

f22=P(xem)−xos22                                                           (26)

while the second function is

g22=Rem(xem)−Ros(xos)22                                                     (27)

While (26) aims at matching the extracted coarse model parameters to xos in the parameter space, (27) aims at matching the same points mapped through the appropriate responses in the response space.

The HASM algorithm consists of two phases: the first phase follows the TRASM strategy while the second phase exploits direct optimization.  It utilizes (18) and (19) for switching between phases as dictated by the smoothness of convergence.

In the ith iteration we assume the existence of a trusted extracted coarse model parameters   xos(i)=P(xem(i)).  The step taken in this iteration is given by (7) where xem(i+1)=xem(i)+h(i).  Single-point parameter extraction is then applied at the point xem(i+1)to get f(i+1)=P(xem(i+1))−xos.

The first phase utilizes two success criteria related to the reduction in (26) and (27).  The SM success criterion is defined as

f(i)−f(i+1)>0.01f(i)−f(i)+B(i)h(i)                                       (28)

which indicates that the actual reduction in the objective function (26) should be greater than a certain fraction of the predicted reduction.  The direct optimization success criterion is

g(i+1)<g(i)                                                                   (29)

which implies that the new iterate xem(i+1) is a descent iterate of (27).

The new point xem(i+1)is accepted and the first phase resumes if this point satisfies both (28) and (29).  B(i)is then updated.  The vector f(i+1) satisfying (28) is either obtained through single-point extraction or through recursive multi-point extraction [3] that approaches a limit satisfying (28).  We denote by x'emand R'em the solution obtained at the end of the first phase and the corresponding fine model response, respectively.

Switching to the second phase takes place in two cases.  The first case is that (29) is not satisfied which means that we have to reject the new point xem(i+1).  The Jacobian of the fine model response at the point xem(i)is then evaluated.  This is done by first evaluating the Jacobian of the coarse model response Jos(i) at the previously extracted coarse model point xos(i)=P(xem(i)).  Jem(i) is then approximated using (18).  The second phase is then supplied by xem(i), Jem(i) and f(i).

The second case occurs when the new point xem(i+1) satisfies (29) but does not satisfy (28).  In this case the point xem(i+1) is better than the previous point xem(i) and is accepted.  As the vector of extracted parameters does not satisfy (28), the vectorf(i+1)can not be trusted.  In order to trust this vector, recursive multi-point parameter extraction is applied at the point xem(i+1) until eitherf(i+1)approaches a limiting value or the number of additional points used for multi-point parameter extraction reaches n.  Iff(i+1)approaches a limit that does not satisfy (28), B(i+1) is updated, Jos(i+1)at the extracted coarse model point xos(i+1)=P(xem(i+1)) is evaluated and Jem(i+1) is then approximated using (18).  Otherwise, Jem(i+1) is approximated using the n+1 fine model points used for multi-point parameter extraction.  The second phase is then supplied by the point xem(i+1), f(i+1) and the Jacobian estimateJem(i+1), which is either calculated using (18) or through finite differences.

The second phase utilizes the first-order derivatives supplied by SM to carry out a number of successful iterations.  By a successful iteration we mean an iteration that satisfies the success criterion

g(k)−g(k+1)>0.01g(k)−g(k)+Jem(k)h(k)                                              (30)

which indicates that the actual reduction in the objective function (27) should be greater than a certain fraction of the predicted reduction.  Notice that the superscript k is used as an index for the successful iterates of the direct optimization phase.  At the end of each successful iteration parameter extraction is applied at the new iterate xem(k) and is used to check whether a switch back to the first phase can take place.  The criterion for such a switch is

f(k+1)<f(k)                                                                   (31)

If it is satisfied Jos(k+1) is evaluated at the point xos(k+1)=P(xem(k+1)), B is reevaluated using (19) and the algorithm switches back to the first phase.  Otherwise, the second phase continues.  We denote by x''emand R''emthe solution obtained at the end of the second phase and the corresponding fine model response, respectively.

For any iteration i  0, the two phases are given by the following steps.

Phase 1

Step 0	Given xem(i),f(i),B(i)and δ(i).  Set δ(i+1)=δ(i).

Comment  δ(i) is the utilized trust region size.

Step 1	Obtain h(i)by solving (7) with  =δ(i+1).  Let δ(i+1)=h(i)2.

Step 2	Evaluate xem(i+1)using (5) and set V=xem(i+1).

Comment  V is the set of fine model points utilized in the multi-point extraction.

Step 3	Apply multi-point parameter extraction using the points in the set V to obtain f(i+1).

Comment  The prediction given in (25) is used as a starting point for the multi-point parameter extraction.

Step 4	If both (28) and (29) are satisfied update the matrix B(i)toB(i+1)using Broyden’s formula [6] and update .  Go to Step 10.

Comment  The trust region size  is updated based on how the predicted reduction in f2 agrees with the actual reduction [3].

Step 5	If (29) is not satisfied, obtain Jos(i) and evaluate Jem(i)=Jos(i)B(i).  Switch to the second phase.

Comment  The second phase takes as arguments xem(i), f(i)and Jem(i) and returns xem(k), B(k)andf(k).  It should be clear that several iterations might be executed in the second phase before switching back to the first phase at Step 10.

Step 6	If V is equal to one, go to Step 9.

Comment  V denotes the cardinality of the set V.

Step 7	Compare f(i+1)obtained using V fine model points with that previously obtained using V1 fine model points.  If f(i+1) is approaching a limit, update the matrix B(i)to get B(i+1), obtain Jos(i+1), evaluate Jem(i+1)=Jos(i+1)B(i+1) and switch to the second phase.

Step 8	If V is equal to n+1, obtain the matrix Jem(i+1) by finite differences using the set V.  Switch to the second phase.

Step 9	Obtain a temporary point xt = xem(i+1)+ht, where

(B(i)TB(i)+λI)ht=−B(i)Tf(i+1)

and ht≤δ(i+1).  Add this point to the set V and go to Step 3.

Step 10	Let i=i+1.  Go to Step 0.

The second phase can be summarized in the following steps.

Phase 2

Step 0	Given the current iterate of the space mapping technique xem(k), the corresponding Jacobian matrix Jem(k) and f(k).

Step 1	Obtain a successful iterate xem(k+1) by solving

(Jem(k)TJem(k)+λI)Δx=−Jem(k)Tg(k)

for a suitable value of λ that satisfies the direct optimization success criterion.

Step 2	Update Jem(k) to Jem(k+1).

Step 3	Apply parameter extraction at xem(k+1) to getf(k+1).

Step 4	If (31) is satisfied obtain Jos(k+1) at the point xos(k+1)=P(xem(k+1)), evaluate the matrix B=Jos(k+1)TJos(k+1)-1Jos(k+1)TJem(k+1) and switch to the first phase.

Step 5	If the termination condition is satisfied invoke the minimax optimizer else set k=k+1 and go to Step 1.

A flow chart of the first phase of the HASM algorithm is shown in Fig. 4.

To ensure optimality, direct optimization is used to solve the original design problem using a minimax optimizer [8] starting from x''em.  The current implementation of the HASM algorithm is in MATLAB [9].

###### VII. EXAMPLES

Three-Section Waveguide Transformer

We consider the design of a three-section waveguide transformer [10].  The design constraints are

VSWR  1.04  for 5.7 GHz  f  7.2 GHz                                          (32)

The designable parameters are the heights of the waveguide sections b1, b2 and b3 and the lengths of waveguide sections L1, L2 and L3.  The fine model exploits HP HFSS [11] through HP Empipe3D [12].  The coarse analytical model does not take into account the junction discontinuity effects [10].

The vector xos is taken as the initial fine model design (Fig. 5).  The HASM algorithm switched to the second phase after two iterations of the first phase, which required 4 fine model simulations.  The response R'em is shown in Fig. 6.  The second phase carried out only one iteration which required 2 fine model simulations.  The response R''emis shown in Fig. 7.

Direct minimax optimization is then applied to the original design problem.  The optimal fine model response Rem is shown in Fig. 8.  Figs. 6 and 8 show that in this example R'em is different from Rem.  The designs xos,x'em,x''emand xemare shown in Table I.

###### Six-Section H-Plane Waveguide Filter

We consider a six-section H-plane waveguide filter [13, 14].  Design specifications are taken as

|S11|  0.16  for   5.4 GHz  f  9.0 GHz                                          (33)

|S11|  0.85  for f  5.2 GHz  and |S11|  0.5  for 9.5 GHz  f                             (34)

A waveguide with a cross-section of 1.372 inches by 0.622 inches (3.485 cm by 1.58 cm) is used.  The six sections are separated by seven H-plane septa, which have a finite thickness of 0.02 inches (0.508 mm).  The filter is shown in Fig. 9.

The optimizable parameters are the four septa widths W1, W2, W3 and W4 and the three waveguide-section lengths L1, L2 and L3.  The coarse model consists of lumped inductances and dispersive transmission line sections.  It is simulated using OSA90/hope [15].  There are various approaches to calculate the equivalent inductive susceptance corresponding to an H-plane septum.  We utilize a simplified version of a formula due to Marcuvitz [16] in evaluating the inductances.  The coarse model is shown in Fig. 10.  The fine model exploits HP HFSS [11] through HP Empipe3D [12].

The fine model response at the starting point xos is shown in Fig. 11.  The first phase required 4 iterations to reach the design x'em.  A total of 5 fine model simulations were needed.  The second phase did not carry out any successful iteration.  The response R''emis shown in Fig. 12.

The response Remis obtained through direct minimax optimization (see Fig. 13).  The different fine model designs are given in Table II.  It is clear that the convergence of the first phase is smooth as R'emRemRos.

Seven-Section Waveguide Transformer

The design of a seven-section waveguide transformer is also considered.  The transformer is shown in Fig. 14.  This example is a classical microwave circuit design problem [10].  The fine model is simulated using HP HFSS [11] through HP Empipe3D [12].  The coarse model is an analytical model which neglects the junction discontinuity [10].  The design specifications are taken as

VSWR  1.01  for  1.06 GHz  f  1.8 GHz                                               (35)

The designable parameters for this problem are the height and length of each waveguide section.  The fine model response at xos is shown in Fig. 15.  The first phase executed three successful iterations that required six fine model simulations.  The response R'em is shown in Fig. 16.  The second phase executed four iterations (see Fig. 17).  The response Rem is shown in Fig. 18.  Table III shows the different designs.

Double-Folded Stub Filter

We consider the design of the double-folded stub (DFS) microstrip structure (Bandler et al. [1]).  Folding the stubs reduces the filter area w.r.t. the conventional double stub structure (Rautio [17]).  The filter is characterized by five parameters : W1, W2, S, L1 and L2 (see Fig. 19).  L1, L2 and S are chosen as optimization variables.  W1 and W2 are fixed at 4.8 mil.  The design specifications are

S21 3 dB  for  f  9.5 GHz  and 16.5 GHz  f                                     (36)

S21 30 dB  for  12 GHz  f  14 GHz                                          (37)

The fine model is the structure simulated by HP HFSS [11] through HP Empipe3D [12].  The coarse model exploits the microstrip line and microstrip T-junction models available in OSA90/hope [15].  The coupling between the folded stubs and the microstrip line is simulated using equivalent capacitors.  The values of these capacitors are determined using Walker’s formulas [18].  Jansen’s microstrip bend model [19] is used to model the folding effect of the stub.  The coarse model is shown in Fig. 20.

The fine model response at xos is shown in Fig. 21.  This figure shows a big shift between the optimal coarse response and the initial fine response.  This signals considerable misalignment between the two models.

The first phase successfully carried out eight iterations that required twelve fine model simulations.  The response R'em is shown in Fig. 22.  Fig. 23 shows a coarse contour plot of f22 in the neighborhood of the point of x'em.  It is clear from this figure that the first phase was trapped in a local minimum.  The mapping established in the first phase is utilized to get a good estimate of Jem and a switch to the second phase is carried out.  The response R''em (Fig. 24) shows a significant improvement in the response.  The design x''em is then taken as the starting point for the minimax optimizer.  The response Rem is shown in Fig. 25.  The designs are given in Table IV.

###### VIII. CONCLUSIONS

We present a novel Hybrid Aggressive Space Mapping (HASM) optimization algorithm.  The algorithm is designed to handle severely misaligned cases.  It enables smooth switching from SM optimization to direct optimization if SM fails.  The direct optimization phase utilizes all the available information accumulated by SM optimization about the mapping between the coarse and fine model spaces.  The algorithm also enables switching back from direct optimization to space mapping if SM is converging smoothly.  The connection between SM and direct optimization is based on a novel lemma.  An original approach for the prediction of the starting point of the parameter extraction optimization problem is also utilized.  This approach improves the uniqueness of the extraction step and consequently enhances the convergence of the algorithm.  The algorithm is successfully demonstrated through the design of waveguide transformers and filters.

ACKNOWLEDGEMENT

The authors thank HP EEsof, Santa Rosa, CA, for making HP HFSS and HP Empipe3D available for this work.

###### REFERENCES

[1]	J.W. Bandler, R.M. Biernacki, S.H. Chen, P.A. Grobelny and R.H. Hemmers, “Space mapping technique for electromagnetic optimization,” IEEE Trans. Microwave Theory Tech., vol. 42, 1994, pp. 2536-2544.

[2]	J.W. Bandler, R.M. Biernacki and S.H. Chen, “Fully automated space mapping optimization of 3D structures,” IEEE MTT-S Int. Microwave Symp. Dig. (San Francisco, CA), 1996, pp. 753-756.

[3]	M.H. Bakr, J.W. Bandler, R.M. Biernacki, S.H. Chen and K. Madsen, “A trust region aggressive space mapping algorithm for EM optimization,” IEEE Trans. Microwave Theory Tech., vol. 46, 1998, pp. 2412-2425.

[4]	J.W. Bandler, R.M. Biernacki, S.H. Chen, R.H. Hemmers and K. Madsen, “Electromagnetic optimization exploiting aggressive space mapping,” IEEE Trans. Microwave Theory Tech., vol. 43, 1995, pp. 2874-2882.

[5]	J.J. Moré and D.C. Sorenson, “Computing a trust region step,” SIAM J. Sci. Stat. Comp., vol. 4, 1983, pp. 553-572.

[6]	C.G. Broyden, “A class of methods for solving nonlinear simultaneous equations,” Math. Comp., vol. 19, 1965, pp. 577-593.

[7]	R. Fletcher, Practical Methods of Optimization, New York: Wiley, Second Edition, 1987.

[8]	J.W. Bandler, W. Kellermann and K. Madsen, “A superlinearly convergent minimax algorithm for microwave circuit design,” IEEE Trans. Microwave Theory Tech., vol. MTT-33, 1985, pp. 1519-1530.

[9]	MATLAB Version 5.0, The Math. Works, Inc., 24 Prime Park Way, Natick, MA 01760, 1997.

[10]	J.W. Bandler, “Computer optimization of inhomogeneous waveguide transformers,” IEEE Trans. Microwave Theory Tech., vol. MTT-17, 1969, pp. 563-571.

[11]	HP HFSS Version 5.2, HP EEsof Division, 1400 Fountaingrove Parkway, Santa Rosa, CA 95403-1799, 1998.

[12]	HP Empipe3D Version 5.2, HP EEsof Division, 1400 Fountaingrove Parkway, Santa Rosa, CA 95403-1799, 1998.

[13]	L. Young and B.M. Schiffman, “A useful high-pass filter design,” Microwave J., vol. 6, 1963, pp. 78-80.

[14]	G.L. Matthaei, L. Young and E.M. T. Jones, Microwave Filters, Impedance-Matching Network and Coupling Structures.  New York: McGraw-Hill, First Edition, 1964.

[15]	OSA90/hope Version 4.0, formerly Optimization Systems Associates Inc., P.O. Box 8083, Dundas, ON, Canada, L9H 5E7, 1997, now HP EEsof Division, 1400 Fountaingrove Parkway, Santa Rosa, CA 95403-1799.

[16]	N. Marcuvitz, Waveguide Handbook.  New York: McGraw-Hill, First Edition, 1951, p. 221.

[17]	J.C. Rautio, Sonnet Software, Inc., 1020 Seventh North Street, Suite 210, Liverpool, NY 13088, Private Communication, 1992.

[18]	C.S. Walker, Capacitance, Inductance and Crosstalk Analysis, Norwood, MA: Artech House, 1990.

[19]	M. Kirschning, R. Jansen and N. Koster, “Measurement and computer-aided modeling of microstrip discontinuities by an improved resonator method,” IEEE MTT-S Int. Microwave Symp. Dig. (Boston, MA), 1983, pp. 495-497.

TABLE I

THE OPTIMAL COARSE MODEL DESIGN AND THE DESIGNS

OBTAINED DURING DIFFERENT PHASES OF THE HASM ALGORITHM FOR

THE THREE-SECTION WAVEGUIDE TRANSFORMER

| Parameter | xos | x'em | x''em | xem |
| --- | --- | --- | --- | --- |
| b1 | 0.90318 | 0.90331 | 0.90114 | 0.90549 |
| b2 | 1.37093 | 1.36436 | 1.35687 | 1.35777 |
| b3 | 1.73609 | 1.73208 | 1.72470 | 1.71866 |
| L1 | 1.54879 | 1.46991 | 1.47203 | 1.47008 |
| L2 | 1.58375 | 1.56402 | 1.56521 | 1.57587 |
| L3 | 1.64590 | 1.79666 | 1.77744 | 1.78286 |
| All values are in cm |  |  |  |  |

TABLE II

THE OPTIMAL COARSE MODEL DESIGN, THE FINAL SPACE-MAPPED

AND THE OPTIMAL FINE MODEL DESIGNS FOR THE

SIX-SECTION H-PLANE WAVEGUIDE FILTER

| Parameter | xos | x'em, x''em | xem |
| --- | --- | --- | --- |
| W1 | 0.48583 | 0.51326 | 0.51344 |
| W2 | 0.43494 | 0.47379 | 0.47396 |
| W3 | 0.40433 | 0.45091 | 0.45100 |
| W4 | 0.39796 | 0.44675 | 0.44664 |
| L1 | 0.65585 | 0.63701 | 0.63695 |
| L2 | 0.65923 | 0.63954 | 0.63977 |
| L3 | 0.67666 | 0.65704 | 0.65694 |
| All values are in inches |  |  |  |

TABLE III

THE OPTIMAL COARSE MODEL DESIGN AND THE DESIGNS

OBTAINED DURING DIFFERENT PHASES OF THE HASM ALGORITHM FOR

THE SEVEN-SECTION WAVEGUIDE TRANSFORMER

| Parameter | xos | x'em | x''em | xem |
| --- | --- | --- | --- | --- |
| b1 | 7.86732 | 7.84126 | 7.84321 | 7.84319 |
| b2 | 6.61888 | 6.56661 | 6.56753 | 6.56746 |
| b3 | 4.68540 | 4.63369 | 4.63275 | 4.63267 |
| b4 | 2.91987 | 2.88266 | 2.88266 | 2.88268 |
| b5 | 1.81412 | 1.79307 | 1.79273 | 1.79272 |
| b6 | 1.27658 | 1.26697 | 1.26721 | 1.26723 |
| b7 | 1.06847 | 1.06475 | 1.06477 | 1.06474 |
| L1 | 7.10588 | 7.27059 | 7.27141 | 7.27145 |
| L2 | 7.12201 | 7.03866 | 7.04043 | 7.04047 |
| L3 | 7.11760 | 6.89568 | 6.89549 | 6.89552 |
| L4 | 7.12331 | 6.89253 | 6.89192 | 6.89189 |
| L5 | 7.12815 | 6.98273 | 6.97985 | 6.98000 |
| L6 | 7.12154 | 7.03160 | 7.03020 | 7.03023 |
| L7 | 7.12945 | 7.02606 | 7.02503 | 7.02509 |
| All values are in cm |  |  |  |  |

TABLE IV

THE OPTIMAL COARSE MODEL DESIGN, THE FINAL SPACE-MAPPED

AND THE OPTIMAL FINE MODEL DESIGNS FOR THE

DFS FILTER

| Parameter | xos | x'em | x''em | xem |
| --- | --- | --- | --- | --- |
| L1 | 66.727 | 72.454 | 73.869 | 78.964 |
| L2 | 60.228 | 72.728 | 82.939 | 81.210 |
| S | 9.592 | 7.621 | 8.170 | 7.901 |
| All values are in mil |  |  |  |  |

x1

-1.0

-0.5

0

0.5

1.0

1.5

2.0

2.5

3.0

0

0.5

1.0

1.5

2.0

2.5

3.0

x2

(a)

x1

x1

(b)                                                                                          (c)

Fig. 1.	Different contour plots for the Rosenbrock problem for the case  = [0.1   0.1]T; (a) the contour plot of xem+α−xos22, (b) the contour plot of P(xem)−xos22 obtained through parameter extraction and (c) contours of the fine model Rosenbrock function.

x1

-1.0

-0.5

0

0.5

1.0

1.5

2.0

2.5

3.0

0

0.5

1.0

1.5

2.0

2.5

3.0

x2

(a)

-1.0

-0.5

0

0.5

1.0

1.5

2.0

2.5

3.0

0.5

1.0

2.0

0

1.5

2.5

3.0

x2

x1

x1

Space Mapping

x, f, B

Direct Optimization

x, Jem

Fig. 3.  Illustration of the connection between SM optimization and direct optimization.

(b)                                                                                            (c)

Fig. 2.  Different contour plots for the Rosenbrock problem for the case  = [1.5   1.5]T; (a) the contour plot of xem+α−xos22, (b) the contour plot of P(xem)−xos22 obtained through parameter extraction and (c) contours of the fine model Rosenbrock function.

Given
,
,
and
 Set
=

Evaluate 
using space mapping and set

Obtain
using the points in V

Both success

criteria satisfied?

No

Direct optimization success criterion is satisfied ?

Evaluate
 from
and

No

Yes

V=1?

No

Yes

Update B to get 
and obtain 
 using 
and

V=n+1?

No

Obtain a temporary point 
and set

Obtain 
 using the points in V

Yes

No

Phase 2

Update 
and

Set i=i+1

Yes

Yes

Parameter extraction

is approaching a limit ?

Fig. 4.  A flow chart of the first phase of the proposed algorithm.

1.20

1.15

1.10

1.05

1.00

VSWR

5.3

5.5

5.7

5.9

6.1

6.3

6.5

6.7

6.9

7.1

7.3

frequency (GHz)

Fig. 5.  The coarse response 
 () and the fine response 
 () for the three-section waveguide transformer.

1.20

1.15

1.10

1.05

1.00

VSWR

5.3

5.5

5.7

5.9

6.1

6.3

6.5

6.7

6.9

7.1

7.3

frequency (GHz)

Fig. 6.The coarse response 
 () and the fine response 
 () for the three-section waveguide transformer.

1.20

1.15

1.10

1.05

1.00

VSWR

5.3

5.5

5.7

5.9

6.1

6.3

6.5

6.7

6.9

7.1

7.3

frequency (GHz)

Fig. 7.  The coarse response 
 () and the fine response 
 () for the three-section waveguide transformer.

1.20

1.15

1.10

1.05

1.00

VSWR

5.3

5.5

5.7

5.9

6.1

6.3

6.5

6.7

6.9

7.1

7.3

frequency (GHz)

Fig. 8.  The coarse response 
 () and the fine response 
 () for the three-section waveguide transformer.

Fig. 9.  The fine model of the six-section H-plane waveguide filter [13, 14].

Fig. 10.  The coarse model of the six-section H-plane waveguide filter [16].

5.20

6.06

6.92

7.78

8.64

9.50

frequency (GHz)

0

0.2

0.4

0.6

0.8

1.0

S11

Fig. 11.  The coarse response Ros () and the fine response Rem(xos) () for the six-section H-plane waveguide filter.

5.20

6.06

6.92

7.78

8.64

9.50

0

0.2

0.4

0.6

0.8

1.0

frequency (GHz)

S11

Fig. 12.	The coarse response Ros () and the fine response R''em () for the six-section H-plane waveguide filter.

5.20

6.06

6.92

7.78

8.64

9.50

frequency (GHz)

0

0.2

0.4

0.6

0.8

1.0

S11

Fig. 13.	The coarse response Ros () and the fine response Rem () for the six-section H-plane waveguide filter.

Fig. 14.  The seven-section waveguide transformer [10].

1.0

1.1

1.2

1.3

1.4

1.5

1.6

1.7

1.8

1.9

frequency (GHz)

1.00

1.01

1.02

1.03

1.04

1.05

VSWR

Fig. 15.	The coarse response Ros () and the fine response Rem(xos) () for the seven-section waveguide transformer.

1.0

1.1

1.2

1.3

1.4

1.5

1.6

1.7

1.8

1.9

frequency (GHz)

1.00

1.01

1.02

1.03

1.04

1.05

VSWR

Fig. 16.	The coarse response Ros () and the fine response R'em () for the seven-section waveguide transformer.

1.0

1.1

1.2

1.3

1.4

1.5

1.6

1.7

1.8

1.9

frequency (GHz)

1.00

1.01

1.02

1.03

1.04

1.05

VSWR

Fig. 17.	The coarse response Ros () and the fine response R''em () for the seven-section waveguide transformer.

1.0

1.1

1.2

1.3

1.4

1.5

1.6

1.7

1.8

1.9

frequency (GHz)

1.00

1.01

1.02

1.03

1.04

1.05

VSWR

Fig. 18.	The coarse response Ros () and the fine response Rem () for the seven-section waveguide transformer.

S

W2

L1

L2

W1

Fig. 19.  The DFS filter [1, 17].

Fig. 20.  The coarse model of the DFS filter.

8

11

14

17

20

5

frequency (GHz)

-70

-60

-50

-40

-30

-20

-10

0

S21dB

-80

Fig. 21.  The coarse response Ros () and the fine response Rem(xos) () for the DFS filter.

S21dB

11

14

17

20

5

frequency (GHz)

-70

-60

-50

-40

-30

-20

-10

0

-80

8

Fig. 22.  The coarse response Ros () and the fine response R'em () for the DFS filter.

72.40

72.45

72.50

72.55

72.60

72.65

72.70

72.75

72.80

72.4

72.5

72.6

72.7

72.8

72.9

72.3

L1

L2

Fig. 23.	A coarse contour plot of P(xem)−xos22 in the neighborhood of x'em for the DFS filter.

8

11

14

17

20

5

frequency (GHz)

-70

-60

-50

-40

-30

-20

-10

0

S21dB

-80

Fig. 24.	The coarse response Ros () and the fine response R''em () for the DFS filter.

8

11

14

17

20

5

frequency (GHz)

-70

-60

-50

-40

-30

-20

-10

0

S21dB

-80

Fig. 25.	The coarse response Ros () and the fine response Rem () for the DFS filter.

This work was supported in part by the Natural Sciences and Engineering Research Council of Canada under Grants OGP0007239, STP0201832 and through the Micronet Network of Centres of Excellence.  N. Georgieva is supported by an NSERC Postdoctorate Fellowship and M.H. Bakr by an Ontario Graduate Scholarship.

M.H. Bakr, J.W. Bandler and N. Georgieva are with the Simulation Optimization Systems Research Laboratory and the Department of Electrical and Computer Engineering, McMaster University, Hamilton, Ontario, Canada, L8S 4K1.

J.W. Bandler is also with Bandler Corporation, P.O. Box 8083, Dundas, Ontario, Canada L9H 5E7.

K. Madsen is with the Department of Mathematical Modelling, Technical University of Denmark, DK-2800 Lyngby, Denmark.