###### AN AGGRESSIVE APPROACH TO PARAMETER EXTRACTION

M.H. Bakr, J.W. Bandler and N. Georgieva

SOS-99-10-R

March 1999

 M.H. Bakr, J.W. Bandler and N. Georgieva 1999

No part of this document may be copied, translated, transcribed or entered in any form into any machine without written permission.  Address inquiries in this regard to Dr. J.W. Bandler.  Excerpts may be quoted for scholarly purposes with full acknowledgment of source.  This document may not be lent or circulated without this title page and its original cover.

AN AGGRESSIVE APPROACH TO PARAMETER EXTRACTION

Mohamed H. Bakr, Student Member, IEEE, John W. Bandler, Fellow, IEEE,

and Natalia Georgieva, Member, IEEE

#### Abstract	A novel Aggressive Parameter Extraction (APE) algorithm is presented.  Our APE algorithm addresses the optimal selection of parameter perturbations used to increase trust in parameter extraction uniqueness.  The uniqueness of the parameter extraction problem is crucial especially in the Space Mapping (SM) approach to circuit design.  We establish an appropriate criterion for the generation of these perturbations.  The APE algorithm classifies possible solutions for the parameter extraction problem.  Two different approaches for obtaining subsequent perturbations are utilized based on a classification of the extracted parameters.  The algorithm is demonstrated through parameter extraction of microwave filters and transformers.  The examples include the parameter extraction of a decomposed electromagnetic model of an HTS filter.  Also, the parameter extraction of an empirical model of a DFS filter is carried out.

I. INTRODUCTION

Parameter extraction is important in device modeling and characterization.  It also plays a crucial role in Space Mapping (SM) technology [13].  Optimization approaches to parameter extraction often yield nonunique solutions.  In SM optimization this nonuniqueness may lead to divergence or oscillatory behavior.

We present an “aggressive” approach to parameter extraction.  While generally applicable, the new algorithm is discussed here in the context of SM technology.  We assume the existence of a “fine” model that generates the target response and a “coarse” model whose parameters are to be extracted.

Several authors have addressed nonuniqueness in parameter extraction.  For example, Bandler et al. [4] proposed the idea of making unknown perturbations to a certain system whose parameters are to be extracted.  Later Bandler et al. [5] suggested that Multi-Point Extraction (MPE) be used to match the first-order derivatives of the two models to ensure a global minimum.  The perturbations used in that approach are predefined and arbitrary.  The optimality of the selection of those perturbations was not addressed.  Recently, a recursive MPE technique was suggested by Bakr et al. [3].  This approach employs a mapping between the two models to enhance uniqueness.

Our algorithm aims at minimizing the number of perturbations used in the MPE process by utilizing perturbations that significantly improve the uniqueness in each iteration.  Consequently, we designate this as an Aggressive Parameter Extraction (APE) algorithm.  Each perturbation requires an additional fine model simulation which could be very CPU intensive.  We classify the different solutions returned by the MPE process and, based on this classification, a new perturbation that is likely to sharpen the result is suggested.

II. PARAMETER EXTRACTION

The objective of parameter extraction is to find a set of parameters of a model whose response matches a given set of measurements.  It can be formulated as

xose=argminxosRm−Ros(xos)                                                    (1)

where Rm is the vector of given measurements, Ros is the vector of circuit response and xose is the vector of extracted parameters.  In the context of SM the fine model response Rem, typically from an electromagnetic simulator, at a certain point xem supplies the target response Rm.  Fig. 1 illustrates the Single Point Extraction (SPE) for the two dimensional case.  An MPE procedure [5] was suggested to improve the uniqueness of the step.  The vector of extracted coarse model parameters xose should satisfy

xose=argminxos  [e0Te1T⋯eNpT]T                                            (2)

where

e0=Ros(xos)−Rem(xem)                                                              (3)

and

ei=Ros(xos+Δxos(i))−Rem(xem+Δxem(i))                                                (4)

The set of perturbations in the coarse model space is represented by Δxos(i)∈Vp, where i=1, 2, .., Np and Vp=Np.  Δxem(i) is the corresponding perturbation in the fine model space.  The perturbations Δxos(i) and Δxem(i) in this MPE procedure are related by

Δxos(i)=Δxem(i)                                                                     (5)

It follows that its solution simultaneously matches the responses of a set of corresponding points in both spaces.  The number of fine model points needed for this process is arbitrary.  There is no clear way of how to select the set of perturbations.  Also, the available information about the mapping between the two spaces was not utilized.

Bakr et al. [3] suggested that the perturbations utilized in (2)-(4) should satisfy

Δxos(i)=BΔxem(i)                                                                   (6)

The matrix B approximates the mapping between the two spaces.  In the context of space mapping (6) is superior to (5) as it integrates the available mapping B into the MPE procedure.  It is also suggested in [3] that the parameter extraction step terminates if the vector of extracted parameters approaches a limit.  The algorithm generates the perturbations used for the MPE process.

The perturbations used in [3] are not guaranteed to result in significant improvement in the uniqueness of the extracted parameters.  A large number of additional fine model simulations may be needed to ensure the uniqueness of the step.

For both (5) and (6) the set V of fine model points utilized in MPE is

V=xem∪xem+Δxem(i)|∀Δxos(i)∈Vp                                               (7)

Fig. 2 illustrates the MPE procedure.

III. THE SELECTION OF PERTURBATIONS

The vector of coarse model responses R used to match the two models is given by

R=Ros(xos)Ros(xos+Δxos(1))..Ros(xos+Δxos(Np))                                                                 (8)

The dimensionality of R is mp, where mp=(Np+1)m and m is the dimensionality of both Ros and Rem.  Vector xose is labeled locally unique [6] if there exists an open neighborhood of xose containing no other point xos such that R(xos)=R(xose).  Otherwise, it is labeled locally nonunique.  It was shown in [6] that the local uniqueness condition is equivalent to the condition that the Jacobian of R has rank n where n is the number of parameters.

To achieve local uniqueness, it was suggested in the context of system identification [4] that increasing the number of perturbations enhances the possibility that the Jacobian matrix J of R becomes full rank.  The perturbations suggested by Daijavad [4] were unidentified perturbations and thus result in an increase in the number of the optimizable parameters.  However, it was pointed out that the improvement in the rank of J outweighs the increase in the number of parameters.

In a later work, the idea of using known perturbations to achieve global uniqueness of parameter extraction was introduced [5].  By global uniqueness we mean that there exists only one minimum xose for the MPE problem.  The existence of more than one locally unique solution for the parameter extraction procedure may result in the divergence or oscillation of the SM optimization algorithm.  It was also pointed out in [5] that using MPE is equivalent to matching the first-order derivatives of the coarse and fine models.

We suggest two different types of perturbation depending on whether the solution of the MPE is locally unique or locally nonunique.  If the solution obtained is locally nonunique we choose a perturbation that is likely to make the new extracted parameters locally unique.

Assume that a locally nonunique minimum xose is obtained using the current set of coarse model perturbations Vp.  Here, the rank of the Jacobian J of R is k where k < n and n is the number of parameters.  We suggest a perturbation Δx that attempts to increase the rank of the Jacobian of the responses corresponding to the augmented set Vp∪Δx at xose by at least one.  This is achieved by imposing the condition that the gradients of nk of the coarse model responses in the new response vector Ros(xose+Δx) be normal to a linearly independent set of gradients of cardinality k of the responses in the vector R at the point xose.  We denote the set of linearly independent gradients by S where

S=g(1),...,g(k)                                                               (9)

We denote the set of the gradients of the newly selected nk responses in Ros(xose+x) by Sa, where

Sa=ga(k+1),... ,gan                                                            (10)

Each of these gradients is approximated by

ga(i)=g(i)+G(i)Δx,       i=k+1, ..., n                                               (11)

where g(i)is the gradient of the ith response at the point xose and G(i)is the corresponding Hessian.  The imposed condition on the perturbation is that

ga(i)Tg(j)=0    ∀g(j)∈S  and   ∀ga(i)∈Sa                                         (12)

Using (11) and (12), the perturbation Δxis obtained by solving the system of linear equations

ATΔx=−c                                                              (13)

where the matrix A is given by

A=[G(k+1)g(1)...  G(n)g(1)...  G(n)g(k)]                                        (14)

and the vector c is given by

c=g(k+1)Tg(1).g(n)Tg(1).g(n)Tg(k)                                                           (15)

A complete derivation of (13) is given in Appendix A.  It should be noted that the system of linear equations (13) may be an over-determined, under-determined or well-determined system of equations depending on k and n.  The pseudoinverse of the matrix AT obtains the solution with minimum ℓ2 norm in all cases.  The fact that this solution is a minimum length solution is of importance since (13) is based on a linear approximation of the gradients which can only be trusted within a certain trust region.  If the perturbation Δx is outside this trust region, it is rescaled.

If the minimum obtained by the MPE is locally unique we still have to ensure that this is the true solution to the extraction problem.  The following lemma leads to a robust way to weaken any other existing locally unique minimum.

Lemma

Assume that there exist two locally unique minima xose,1 and xose,2 for the MPE problem obtained using least squares optimization and a set of perturbations Vp.  A possible perturbation x that can be added to the set Vp and can be used to weaken one of these minima as a solution for the MPE is in the direction of an eigenvector for the matrix H1 – H2 where H1 and H2 are the Hessian matrices for the ℓ2 objective function at the points xose,1 and xose,2, respectively.

Proof

We denote by Q(x,V) the value of the ℓ2 objective function of the MPE problem at a coarse model point x using a set of fine model points V, where V is given by (7).  The quadratic approximations of Q(x,V) in a neighborhood centered at the two locally unique minima xose,1 and xose,2, respectively are given by

q1(Δx,V)=Q(xose,1,V)+0.5ΔxTH1Δx                                                   (16)

q2(Δx,V)=Q(xose,2,V)+0.5ΔxTH2Δx                                                  (17)

The perturbation Δxthat results in the maximum difference between the two quadratic models (16) and (17) for a specific trust region   is obtained by formulating the Lagrangian

L(x,λ)=(Q(xose,2,V)−Q(xose,1,V))+0.5ΔxT(H2−H1)Δx+λ(ΔxTΔx−δ2)                  (18)

Taking the derivative with respect to x gives

(H1−H2)Δx=2λΔx                                                               (19)

It follows that Δxis an eigenvector of the matrix H1 – H2.  Δxprovides a direction that maximizes the difference between the quadratic models.  In other words, it provides a perturbation that maximizes the contrast between the changes of the coarse model responses at these two minima.  It follows that the true minimum is the one whose response changes match better the changes of the fine model responses obtained using the perturbationΔx.

A similar result to that obtained in (19) can be obtained using a different approach.  Assume that a perturbation of Δx is sought.  This perturbation results in a perturbation of the coarse model responses at the two minima by

ΔR1=Jos(xose,1)Δx                                                             (20)

and

ΔR2=Jos(xose,2)Δx                                                             (21)

Where Jos(xos) is the Jacobian of the coarse model response Ros.  We impose the condition that the difference between the ℓ2 norms of these two response perturbations be maximized subject to certain trust region size.  Therefore, the following Lagrangian can be formed

L(Δx,λ)=ΔxTJos(xose,1)TJos(xose,1)Δx−ΔxTJos(xose,2)TJos(xose,2)Δx+λ(ΔxTΔx−δ2)      (22)

Using a similar approach to that used in deriving (19) it can be shown that the perturbation Δx is obtained by solving the eigenvalue problem

(Jos(xose,1)TJos(xose,1)−Jos(xose,2)TJos(xose,2))Δx=λΔx                                         (23)

The two perturbations (19) and (23) can be shown to be almost identical by writing the Hessian matrix of Q(x,V) in terms of the Jacobian of the coarse model responses [7].  However, the perturbation calculated in (23) is more computionally efficient than that of (19).

There is one substantial difficulty in the exact evaluation of the perturbation given by (23).  Once a locally unique minimum is reached the Hessian of Q at this point can be obtained while no information is available about the Hessian at the other locally unique minima that may exist.  In such a case, a reasonable assumption is to take H2=I, the identity matrix or alternatively take Jos(xose,2)TJos(xose,2) as the identity matrix in (23).  This assumption implies that no information is available about the curvature of the objective function at the other minima.  It follows that Δxis an eigenvector of the matrix Jos(xose,1)TJos(xose,1).

The perturbation given by (23) is a suggested perturbation in the coarse model space.  The new fine model point that should be added to the set V is xem+Δxemwhere Δxemobtained by solving the system of linear equations

Δx=BΔxem                                                                  (24)

The relation (24) is used if some information is available about the mapping between the two spaces.  However, in most cases we make the assumption that B=I.

The scheme that we utilized for the selection of points in (23) is as follows.  First, the eigenvalue problem is solved.  The eigenvector v(1) with the largest eigenvalue in modulus is initially selected as the candidate eigenvector.  The suggested perturbation in this case is

Δxos=δv(1)v(1)                                                                (25)

where δis the current size of the trust region.  This perturbation is tested to see whether it belongs to the current set of perturbations.  It follows that xos is rejected if the condition

ΔxosTΔxos(i)Δxos2>(1−ε)                                                            (26)

is satisfied for a perturbation Δxos(i)∈Vp, where  > 0 is a small number.  In this case the alternative perturbation

Δxos=−δv(1)v(1)                                                                     (27)

is tested against the condition (26).  If it also fails, we switch to the eigenvector with second largest eigenvalue in modulus and repeat steps (25) – (27).  This is repeated until either a perturbation is found such that condition (26) is not satisfied or all the eigenvectors are exhausted for perturbations of length .  In this case the trust region size  is scaled by  where  >1.0.  The perturbation is then taken in the direction of eigenvector with largest eigenvalue in modulus.

### IV. THE AGGRESSIVE PARAMETER EXTRACTION (APE) ALGORITHM

In this section we present the APE algorithm for the MPE process.  This algorithm is based on the two methods discussed in the previous section.  It is given by the following steps.

Step 0	Given xem,   and n.  Initialize V(1)=xem(1), where xem(1)=xem and set i=1.

Comment  The set V(i) contains the points used for the MPE in the ith iteration.  The index i is equal to V(i), the cardinality of V(i).

Step 1	Apply MPE using the set V(i) to get xose (i).

Comment  The point xose (i)is the solution to the MPE problem obtained using the set V(i).

Step 2	If the Jacobian of R at xose (i) has full rank, go to Step 4.

Step 3	Obtain a new perturbation Δxusing (13), use (24) to get Δxem and let V(i+1)=V(i)∪xem(i+1), where xem(i+1)=xem+Δxem.  Set i=i+1 and go to Step 1.

Comment  The perturbation Δxis rescaled to satisfy the trust region condition Δx=δ.

Step 4	If V(i)is equal to one, go to Step 6.

Step 5	If xose (i) is approaching a limit, stop.

Step 6	Obtain a new perturbation Δxusing (23) and use (24) to get Δxem.  Update  and let V(i+1)=V(i)∪xem(i+1), where xem(i+1)=xem+Δxem.  Set i=i+1 and go to Step 1.

Comment  In Step 6 the eigenvalue problem is solved and the perturbationΔxis selected according to the scheme discussed in the previous section.  This scheme may result in updating the trust region size.  The algorithm terminates if the vector of extracted coarse model parameters obtained using i fine model points is close enough in terms of some norm to the vector of extracted parameters obtained using i1 fine model points.

Fig. 3 illustrates the relationship between the generated sets V(i), the fine model points xem(i) and the extracted coarse model points xose (i).  A flowchart of the APE algorithm is shown in Fig. 4.  The current implementation of the algorithm is in MATLAB [8].

V. EXAMPLES

The Rosenbrock Function

The first example utilizes the famous Rosenbrock function [7].  The coarse model for this problem is given by

Rc=100(u2−u12)2+(1−u1)2                                                     (28)

The fine model is another “Rosenbrock” function but with a shift applied to the parameters

Rf=100((u2+0.2)−(u1−0.2)2)2+(1−(u1−0.2))2                                   (29)

It is required to extract the coarse model parameters corresponding to the fine model point [1.0    1.0]T.  The result of the SPE at this point is xose (1)=[1.21541   0.91728]T.  The contours of Q(x,V(1)) are shown in Fig. 5.  It is clear from the contour plot that the minimum obtained is a locally nonunique minimum.  The algorithm detects this and generates a perturbation that attempts to improve the rank of the Jacobian of R in the Double Parameter Extraction (DPE) using (13).  Utilizing a trust region size of 0.25 the set V(2)is given by

V(2)=1.01.0 T,0.76431.0833 T                                                (30)

The contours of Q(x,V(2)) are shown in Fig. 6.  It is clear that by using only one additional fine model simulation the uniqueness of the problem has improved dramatically.  Actually, the only existing minimum is a global unique minimum.  To ensure uniqueness a third point is generated by solving the eigenvalue problem (23).  Thus we have

V(3)=1.01.0 T,0.76431.0833 T,0.81850.8281 T                              (31)

The contours of Q(x,V(3)) are shown in Fig. 7.  The algorithm then terminates as it detects that the extracted parameters are approaching a limit.  It returns the last set of extracted parameters as the solution for the MPE problem.  The variation of the extracted parameters obtained using the ℓ2 optimizer with the number of fine model points used is shown in Table I.

A 10:1 Impedance Transformer

The second example is the well-known 10:1 impedance transformer [9].  The parameters for this problem are the characteristic impedance of the two transmission lines Z1 and Z2 while the two lengths of the transmission lines are kept fixed at their optimal values (quarter wave length).  The coarse model utilizes nonscaled parameters while a “fine” model scales each of the two impedances by a factor of 1.6.

It is required in this synthetic problem to extract the coarse model parameters whose response matches the fine model response at the point [2.2628   4.5259]T.  This point is the optimal coarse model design according to the specifications in [9].

The two models are matched using the reflection coefficients at eleven equally spaced frequencies in the frequency range 0.5 GHz  f  1.5 GHz.  The fine model response at xem(1) and the coarse model response at the point xose (1) are shown in Fig. 8.  The contours of Q(x,V(1)) are shown in Fig. 9.  It is clear from this figure that there exist three locally unique minima for the extraction problem.  The algorithm then generates a second perturbation using (23).  The set V(2) is given by

V(2)=2.262774.52592 T,1.499754.76634 T                                  (32)

The fine model response for every point in V(2) and the coarse response at the corresponding extracted coarse model point are shown in Fig. 10.  The corresponding contours of Q(x,V(2)) are shown in Fig. 11.  It is clear that there still exist two locally unique minima.  Using (23) we have

V(3)=2.262774.52592 T,1.499754.76634 T,3.020244.26855T                (33)

The fine model response for each point in the set V(3) and the coarse model response at the corresponding extracted coarse model point are shown in Fig. 12.  The contours of Q(x,V(3)) are shown in Fig. 13.  The algorithm terminates as the termination condition is satisfied.  The variation of the extracted coarse model point with V(i) is given in Table II.

The HTS Filter [10]

The fine model for the HTS filter (Fig. 14) is simulated as a whole using Sonnet’s em [11].  The “coarse” model is a decomposed Sonnet version of the fine model.  This model exploits a coarser grid than that used for the fine model.  The physical parameters of the coarse and fine models are given in Table III.

It is required to extract the coarse model parameters corresponding to the fine model parameters given in Table IV.  The values in this table are the optimal coarse model design obtained using the minimax optimizer in OSA90/hope [12] according to specifications given in [10].  We utilize the responses at 15 discrete frequencies in the range [3.967 GHz, 4.099 GHz] in the parameter extraction process.

The algorithm first started by applying SPE where V(1) contains only the point given in Table IV.  The point xose (1) is given in Table VI.  Fig. 15 shows the fine model response at xem(1) and the coarse model response at xose (1).

The algorithm detected that this extracted point is a locally unique minimum.  A new fine model point is then generated by solving the eigenvalue problem (23).  A DPE step is then carried out.  The set V(2) includes the points given in the second and third columns of Table V.  The point xose (2) is given in Table VI.  Fig. 16 shows the fine model responses at the two utilized fine model points and the responses at the corresponding extracted coarse model points, respectively.  Again the algorithm detected that the extracted point is locally unique and a new fine model point is generated and added to the set of points.  The same steps were then repeated for three-point and four point parameter extraction.  The points utilized are given in Table V.  The results are shown in the fourth and fifth columns of Table VI.  It is clear that the extracted parameters are approaching a limit.  The fine model responses and the responses at the corresponding extracted coarse model points for the last two iterations are shown in Figs. 17 and 18, respectively.  Fig. 18(a) demonstrates that a good match between the responses of both models over a wider range of frequencies than that used for parameter extraction is achieved.

Double-folded Stub Filter [1]

We consider the design of the double-folded stub (DFS) microstrip structure shown in Fig. 19 (Bandler et al. [1]).  Folding the stubs reduces the filter area w.r.t. the conventional double stub structure (Rautio [13]).  The filter is characterized by five parameters : W1 , W2 , S, L1 and L2.  L1, L2 and S are chosen as optimization variables.  W1 and W2 are fixed at 4.8 mil.  The fine model is simulated by HP HFSS [14] through HP Empipe3D [15].  The coarse model exploits the microstrip line and microstrip T-junction models available in OSA90/hope [12].  The coupling between the folded stubs and the microstrip line is simulated using equivalent capacitors.  The values of these capacitors is determined using Walker’s formulas [16].  Jansen’s microstrip bend model [17] is used to model the folding effect of the stub.  The coarse model is shown in Fig. 20.

It is required in this example to extract the coarse model parameters corresponding to the fine model parameters given in Table VII.  This vector is the optimal design of the coarse model obtained by minimax optimization.  The optimal coarse model response and the fine model response at the optimal coarse model design are shown in Fig. 21.  This figure shows clearly the large misalignment between the two models which implies nonuniqueness of the extracted parameters.

The algorithm started by applying SPE using the fine model point given in Table VII.  Fig. 22 shows the fine model response at xem(1) and the coarse model response at the point xose (1).  The algorithm detected that the extracted parameters are locally unique.  A new fine model point is generated using (23) and added to the set of fine model points used for the MPE.  The algorithm needed nine iterations to trust the extracted coarse model parameters.  The fine model points utilized are given in Table VIII and the extracted coarse model points are given in Table IX.  Fig. 23 shows the fine model response at xem(1) and the coarse model response at the point xose (9).

Table IX shows the large relative change in parameter values between the first set of extracted parameters xose (1) and the trusted set of parameters xose (9).  If the step taken by any SM optimization algorithm utilizes xose (1), the algorithm would have probably failed.  The contours of Q(x,V(1)) in the neighborhood of xose (1) and in the neighborhood of xose (9) are shown in Fig. 24 (a) and (b), respectively.  Also, The contours of Q(x,V(9)) in the neighborhood of xose (1) and in the neighborhood of xose (9) are shown in Fig. 24 (c) and (d), respectively.  It is clear from this figure that the point xose (1) has been weakened as a possible solution for the parameter extraction problem.

Fig. 25 shows the change of Q(xose (1),V(i)) and of Q(xose (9),V(i)) with V(i).  The value of Q(xose (9),V(i)) remains almost constant and small in value.  On the other hand the value of Q(xose (1),V(i))  increases significantly with each new point added to the set of utilized fine model points signaling a false minimum.

VI. CONCLUSIONS

An Aggressive Parameter Extraction (APE) algorithm is proposed.  Our APE algorithm addresses the optimal selection of parameter perturbations used to improve the uniqueness of a multi-point parameter extraction procedure.  The nonuniqueness of the parameter extraction problem may lead to the divergence or oscillation of the SM approach to circuit design.  New parameter perturbations are generated based on the nature of the minimum reached in the previous iteration.  We consider possibly locally unique and locally nonunique minima.  The suggested perturbations in each of these two cases are obtained either by solving a system of linear equations or by solving an eigenvalue problem.  The APE algorithm continues until the extracted coarse model parameters can be trusted.  The algorithm is successfully demonstrated through the parameter extraction of microwave filters and impedance transformers.

ACKNOWLEDGEMENT

The authors thank Sonnet Software, Inc., Liverpool, NY for making em available for this work.  They also thank HP EEsof, Santa Rosa, CA, for making HP HFSS and HP Empipe3D available.

###### REFERENCES

[1] 	J.W. Bandler, R.M. Biernacki, S.H. Chen, P.A. Grobelny and R.H. Hemmers, “Space mapping technique for electromagnetic optimization,” IEEE Trans. Microwave Theory Tech., vol. 42, 1994, pp. 2536-2544.

[2]	J.W. Bandler, R.M. Biernacki, S.H. Chen, R.H. Hemmers and K. Madsen, “Electromagnetic optimization exploiting aggressive space mapping,” IEEE Trans. Microwave Theory Tech., vol. 43, 1995, pp. 2874-2882.

[3]	M.H. Bakr, J.W. Bandler, R.M. Biernacki, S.H. Chen and K. Madsen, “A trust region aggressive space mapping algorithm for EM optimization,” IEEE Trans. Microwave Theory Tech., vol. 46, 1998, pp. 2412-2425.

[4]	J.W. Bandler, S.H. Chen and S. Daijavad, “Microwave device modeling using efficient ℓ1optimization: a novel approach,” IEEE Trans. Microwave Theory Tech., vol. MTT-34, 1986, pp. 1282-1293.

[5]	J.W. Bandler, R.M. Biernacki and S.H. Chen, “Fully automated space mapping optimization of 3D structures,” IEEE MTT-S Int. Microwave Symp. Dig. (San Francisco, CA), 1996, pp. 753-756.

[6]	J.W. Bandler and A.E. Salama, “Fault diagnosis of analog circuits,” Proc. IEEE, vol. 73, pp. 1279-1325, 1985.

[7]	R. Fletcher, Practical Methods of Optimization. New York: Wiley, Second Edition, 1987.

[8]	MATLAB Version 5.0, The Math. Works, Inc., 24 Prime Park Way, Natick, MA 01760, 1997.

[9]	J.W. Bandler, “Optimization methods for computer-aided design,” IEEE Trans. Microwave Theory Tech., vol. MTT-17, 1969, pp. 533-552.

[10]	J.W. Bandler, R.M. Biernacki, S.H. Chen, W.J. Gestinger, P.A. Grobelny, C. Moskowitz and S.H. Talisa, “Electromagnetic design of high-temperature superconducting filters,” Int. J. Microwave and Millimeter-Wave Computer-Aided Engineering, vol. 5, 1995, pp. 331-343.

[11]	em , Sonnet Software, Inc., 1020 Seventh North Street, Suite 210, Liverpool, NY 13088, 1997.

[12]	OSA90/hope Version 4.0, formerly Optimization Systems Associates Inc., P.O. Box 8083, Dundas, ON, Canada, L9H 5E7, 1997, now HP EEsof Division, 1400 Fountaingrove Parkway, Santa Rosa, CA 95403-1799.

[13]	J.C. Rautio, Sonnet Software, Inc., 1020 Seventh North Street, Suite 210, Liverpool, NY 13088, Private Communication, 1992.

[14]	HP HFSS Version 5.2, HP EEsof Division, 1400 Fountaingrove Parkway, Santa Rosa, CA 95403-1799, 1998.

[15]	HP Empipe3D Version 5.2, HP EEsof Division, 1400 Fountaingrove Parkway, Santa Rosa, CA 95403-1799, 1998.

[16]	C.S. Walker, Capacitance, Inductance and Crosstalk Analysis, Norwood, MA: Artech House, 1990.

[17]	M. Kirschning, R. Jansen and N. Koster, “Measurement and computer-aided modeling of microstrip discontinuities by an improved resonator method,” IEEE MTT-S Int. Microwave Symp. Dig. (Boston, MA), 1983, pp. 495-497.

TABLE I

THE VARIATION OF THE EXTRACTED PARAMETERS

FOR THE ROSENBROCK FUNCTION WITH THE

NUMBER OF POINTS USED FOR EXTRACTION

| Parameter | xose (1) | xose (2) | xose (3) |
| --- | --- | --- | --- |
| u1 | 1.21541 | 0.80008 | 0.80008 |
| u2 | 0.91728 | 1.20012 | 1.20014 |

TABLE II

THE VARIATION OF THE EXTRACTED PARAMETERS

FOR THE 10:1 IMPEDANCE TRANSFORMER WITH THE

NUMBER OF POINTS USED FOR EXTRACTION USING

| Parameter | xose (1) | xose (2) | xose (3) |
| --- | --- | --- | --- |
| Z1 | 3.62043 | 3.47160 | 3.60357 |
| Z2 | 7.24147 | 7.43214 | 7.35052 |

TABLE III

MATERIAL AND PHYSICAL PARAMETERS

FOR THE COARSE AND FINE MODELS OF THE HTS FILTER

| Model Parameter | Coarse Model | Fine Model |
| --- | --- | --- |
| substrate dielectric constant | 23.425 | 23.425 |
| substrate thickness (mil) | 19.9516 | 19.9516 |
| shielding cover height (mil) | 100 | 250 |
| conducting material thickness | 0 | 0 |
| substrate dielectric loss tangent | 0 | 0 |
| resistivity of metal (m) | 0 | 0 |
| magnetic loss tangent | 0 | 0 |
| surface reactance (/sq) | 0 | 0 |
| x-grid cell size (mil) | 2.00 | 1.00 |
| y-grid cell size (mil) | 1.75 | 1.75 |

TABLE IV

THE OPTIMAL COARSE MODEL DESIGN

FOR THE HTS FILTER

| Parameter | Value |
| --- | --- |
| L1 | 181.00 |
| L2 | 201.59 |
| L3 | 180.97 |
| S1 | 20.12 |
| S2 | 67.89 |
| S3 | 66.85 |
| all values are in mils |  |

TABLE V

THE FINE MODEL POINTS USED IN THE APE

ALGORITHM FOR THE HTS FILTER

| Parameter | xem(1) | xem(2) | xem(3) | xem(4) |
| --- | --- | --- | --- | --- |
| L1 | 181.00 | 182.55 | 181.34 | 179.86 |
| L2 | 201.59 | 205.64 | 205.38 | 197.74 |
| L3 | 180.97 | 183.36 | 184.20 | 178.08 |
| S1 | 20.12 | 20.05 | 20.07 | 20.46 |
| S2 | 67.89 | 68.40 | 68.08 | 67.35 |
| S3 | 66.85 | 67.25 | 66.98 | 66.46 |
| all values are in mils |  |  |  |  |

TABLE VI

THE VARIATION IN THE EXTRACTED PARAMETERS

FOR THE HTS FILTER WITH THE NUMBER OF

FINE MODEL POINTS

| Parameter | xose (1) | xose (2) | xose (3) | xose (4) |
| --- | --- | --- | --- | --- |
| L1 | 188.31 | 179.99 | 176.67 | 178.50 |
| L2 | 197.69 | 204.52 | 208.52 | 206.78 |
| L3 | 189.72 | 181.23 | 178.00 | 179.09 |
| S1 | 19.34 | 17.13 | 17.21 | 18.99 |
| S2 | 52.67 | 63.44 | 56.52 | 57.99 |
| S3 | 52.06 | 53.18 | 53.47 | 56.77 |
| all values are in mils |  |  |  |  |

TABLE VII

THE OPTIMAL COARSE MODEL DESIGN

FOR THE DFS FILTER

| Parameter | Value |
| --- | --- |
| L1 | 66.73 |
| L2 | 60.23 |
| S | 9.59 |
| all values are in mils |  |

TABLE VIII

THE FINE MODEL POINTS USED IN THE APE

ALGORITHM FOR THE DFS FILTER

| Parameter | xem(1) | xem(2) | xem(3) | xem(4) | xem(5) | xem(6) | xem(7) | xem(8) | xem(9) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L1 | 66.73 | 67.72 | 67.32 | 66.15 | 70.60 | 67.66 | 62.82 | 65.80 | 66.57 |
| L2 | 60.23 | 63.58 | 64.13 | 56.33 | 59.48 | 64.10 | 60.88 | 56.36 | 59.85 |
| #### S | 9.59 | 9.27 | 9.48 | 9.71 | 9.71 | 9.66 | 9.50 | 9.52 | 10.26 |
| all values are in mils |  |  |  |  |  |  |  |  |  |

TABLE IX

THE VARIATION IN THE EXTRACTED PARAMETERS

FOR THE DFS FILTER WITH THE NUMBER OF

FINE MODEL POINTS

| Parameter | xose (1) | xose (2) | xose (3) | xose (4) | xose (5) | xose (6) | xose (7) | xose (8) | xose (9) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L1 | 58.01 | 67.05 | 66.11 | 64.36 | 56.46 | 66.10 | 56.50 | 56.39 | 56.59 |
| L2 | 38.40 | 40.47 | 40.40 | 43.28 | 42.94 | 42.02 | 42.81 | 43.00 | 43.02 |
| #### S | 3.24 | 6.86 | 6.64 | 8.83 | 18.10 | 7.99 | 18.25 | 17.93 | 17.87 |
| all values are in mils |  |  |  |  |  |  |  |  |  |

APPENDIX A

Let the two sets S and Sa be defined as in (9) and (10), respectively.  We impose the condition that every gradient on the set Sa should be orthogonal to all gradients in the set S.  It follows that

ga(i)Tg(j)=0    ∀g(j)∈S  and   ∀ga(i)∈Sa                                         (34)

But each gradient in the set Sa  can be approximated by

ga(i)=g(i)+G(i)Δx,       i=k+1, ..., n                                                (35)

where g(i)is the gradient of the ith response at the pointxose and G(i)is the corresponding Hessian.  It follows that the condition (34) can be restated as

g(j)Tg(i)=−g(j)TG(i)Δx,    j=1, ..., k and i=k+1, ..., n                                 (36)

Equation (36) is a linear equation in n unknowns (the components of x).  There are k(nk) such linear equations.  Putting these equations into a matrix form we have

Bx = c                                                                              (37)

where the mth row of the matrix B is

B(m)T=g(j)TG(i),       i=k+1, ..., n and j=1, ..., k                                          (38)

where m=(i1)(nk)+j.  Similarly the mth component of the vector c is

cm,1=g(j)Tg(i),      i=k+1, ..., n and j=1, ..., k                                           (39)

Thus (13) follows.

    

PE

PE

PE

Fig. 3.Illustration of the relationship between the generated sets 
, the fine model points 
 and the extracted coarse model points 
generated by the APE algorithm.

xem,1

xem,2

xos,1

xos,2

Ros

Rem

xem

Fig. 1.  Illustration of the SPE procedure.

xem,1

xem,2

xos,1

xos,2

xem

V

B

Fig. 2.  Illustration of the MPE procedure.

Given 
,   and n.

Initialize i=1 and

J(
) has rank n ?

No

Obtain a new perturbation 
using first method.  Set  
 and i=i+1.

Is 
=1?

Yes

No

Is 
 approaching a limit ?

Obtain a new perturbation 
using second method. Set 
 and i=i+1.

Stop

Yes

No

Yes

Apply multi-point parameter extraction using the points in the set 
 to get

Fig. 4.  The flow chart of the APE algorithm.

Fig. 5.  The contours of Q(x,V(1)) for the Rosenbrock function.

Fig. 6.  The contours of Q(x,V(2)) for the Rosenbrock function.

Fig. 7.  The contours of Q(x,V(3)) for the Rosenbrock function.

0.5

0.6

0.7

0.8

0.9

1.0

1.1

1.2

1.3

1.4

1.5

0

0.1

0.2

0.3

0.4

0.5

0.6

frequency (GHz)

0.7

S11

Fig. 8.	The responses of the given fine model point () and the coarse model response () at the point xose (1) for the 10:1 impedance transformer.

Fig. 9.  The contours of Q(x,V(1)) for the 10:1 impedance transformer.

0.5

0.6

0.7

0.8

0.9

1.0

1.1

1.2

1.3

1.4

1.5

0

0.1

0.2

0.3

0.4

0.5

0.6

frequency (GHz)

0.7

S11

0.5

0.6

0.7

0.8

0.9

1.0

1.1

1.2

1.3

1.4

1.5

0

0.1

0.2

0.3

0.4

0.5

0.6

frequency (GHz)

0.7

S11

(b)

Fig. 10.	The fine model response () and the corresponding coarse model response (); (a) at the first point and (b) at the second point utilized in the DPE for the 10:1 impedance transformer.

Fig. 11.  The contours of Q(x,V(2)) for the 10:1 impedance transformer.

0.5

0.6

0.7

0.8

0.9

1.0

1.1

1.2

1.3

1.4

1.5

0

0.1

0.2

0.3

0.4

0.5

0.6

frequency (GHz)

0.7

S11

0.5

0.6

0.7

0.8

0

.9

1.0

1.1

1.2

1.3

1.4

1.5

0

0.1

0.2

0.3

0.4

0.5

0.6

frequency (GHz)

0.7

S11

0.5

0.6

0.7

0.8

0.9

1.0

1.1

1.2

1.3

1.4

1.5

0

0.1

0.2

0.3

0.4

0.5

0.6

frequency (GHz)

0.7

(a)																				(b)

(c)

Fig. 12.	The fine model response () and the corresponding coarse model response () ; (a) at the first point, (b) at the second point and (c) at the third point utilized in the three-point parameter extraction for the 10:1 impedance transformer.

1

2

3

4

5

6

7

1

2

3

4

5

6

7

8

9

10

Z1

Z2

Fig. 13.	The contours of Q(x,V(3)) for the 10:1 impedance transformer.

L0

L1

L2

L3

L2

L1

L0

S1

S2

S3

S2

S1

W

x

y

First Section

Second Section

Third

Section

Fourth Section

Fifth Section

Fig. 14.  The HTS filter [10].

3.867

3.929

3.992

4.054

4.117

4.179

frequency (GHz)

-60

-50

-40

-30

-20

-10

0

S21 dB

(a)

Fig. 15.	The fine model response () and the corresponding coarse model response () at the point utilized in the SPE for the HTS filter.  Note that only points in the range 3.967 GHz to 4.099 GHz were actually used.

3.867

3.929

3.992

4.054

4.117

4.179

frequency (GHz)

-60

-50

-40

-30

-20

-10

0

S21 dB

3.867

3.929

3.992

4.054

4.117

4.179

frequency (GHz)

-60

-50

-40

-30

-20

-10

0

S21 dB

(a)	(b)

Fig. 16.	The fine model response () and the corresponding coarse model response (), (a) at the first point, and (b) at the second point utilized in the DPE for the HTS filter.  Note that only points in the range 3.967 GHz to 4.099 GHz were actually used.

3.867

3.929

3.992

4.054

4.117

4.179

frequency (GHz)

-60

-50

-40

-30

-20

-10

0

S21 dB

3.867

3.929

3.992

4.054

4.117

4.179

frequency (GHz)

-60

-50

-40

-30

-20

-10

0

S21 dB

3.867

3.929

3.992

4.054

4.117

4.179

frequency (GHz)

-60

-50

-40

-30

-20

-10

0

S21 dB

(a)	(b)

(c)

Fig. 17.	The fine model response () and the corresponding coarse model response (), (a) at the first point, (b) at the second point, and (c) at the third point utilized in the three-point parameter extraction for the HTS filter.  Note that only points in the range 3.967 GHz to 4.099 GHz were actually used.

3.867

3.929

3.992

4.054

4.117

4.179

frequency (GHz)

-60

-50

-40

-30

-20

-10

0

S21 dB

3.867

3.929

3.992

4.054

4.117

4.179

frequency (GHz)

-60

-50

-40

-30

-20

-10

0

S21 dB

3.867

3.929

3.992

4.054

4.117

4.179

frequency (GHz)

-60

-50

-40

-30

-20

-10

0

S21 dB

3.867

3.929

3.992

4.054

4.117

4.179

frequency (GHz)

-60

-50

-40

-30

-20

-10

0

S21 dB

(a)	(b)

(c)	(d)

Fig. 18.	The fine model response () and the corresponding coarse model response (), (a) at the first point, (b) at the second point, (c) at the third point, and (d) at the fourth point utilized in the four-point parameter extraction for the HTS filter.  Note that only points in the range 3.967 GHz to 4.099 GHz were actually used.

S

W2

L1

L2

W1

Fig. 19.  The DFS filter [1].

Fig. 20.  The coarse model of the DFS filter.

8

10

12

14

16

18

20

0

0.25

0.50

0.75

1.00

S21

frequency (GHz)

Fig. 21.	The optimal coarse model response () and the fine model response () at the optimal coarse model design for the DFS filter.

8

10

12

14

16

18

20

0

0.25

0.50

0.75

1.00

S21

frequency (GHz)

Fig. 22.	The fine model response () and the corresponding coarse model response () at the point xose (1) for the DFS filter.

8

10

12

14

16

18

20

0

0.25

0.50

0.75

1.00

S21

frequency (GHz)

Fig. 23.	The fine model response () and the corresponding coarse model response () at the point xose (9) for the DFS filter.

45

50

55

60

65

25

30

35

40

45

L1

L2

45

50

55

60

65

35

40

45

50

55

L1

L2

(a)                                                                                    (b)

45

50

55

60

65

25

30

35

40

45

L1

L2

45

50

55

60

65

35

40

45

50

55

L1

L2

(c)                                                                                     (d)

Fig. 24.	Contour plots of the MPE problem for the DFS filter; (a) the contours of Q(x,V(1)) in the neighborhood of xose (1), (b) the contours of Q(x,V(1)) in the neighborhood of xose (9), (c) the contours of Q(x,V(9)) in the neighborhood of xose (1) and (d) the contours of Q(x,V(9)) in the neighborhood of xose (9).

1

2

3

4

5

6

7

8

9

0

0.1

0.2

0.3

0.4

0.5

0.6

0.7

0.8

0.9

1.0

i

Q(x,V(i))

Fig. 25.	The variation of Q(x,V(i)) for the DFS filter at the point xose (1) (  ) and at the point xose (9) (  ) with the number of points utilized for parameter extraction.

This work was supported in part by the Natural Sciences and Engineering Research Council of Canada under Grants OGP0007239, STP0201832 and through the Micronet Network of Centres of Excellence.  N. Georgieva is supported by an NSERC Postdoctorate Fellowship and M.H. Bakr by an Ontario Graduate Scholarship.

M.H. Bakr, J.W. Bandler and N. Georgieva are with the Simulation Optimization Systems Research Laboratory and the Department of Electrical and Computer Engineering, McMaster University, Hamilton, Ontario, Canada L8S 4K1.

J.W. Bandler is also with Bandler Corporation, P.O. Box 8083, Dundas, Ontario, Canada L9H 5E7.

