Space Mapping Based Neuromodeling

of High Frequency Circuits

J.W. Bandler, J.E. Rayas-Sánchez and Q.J. Zhang

SOS-99-12-V

April 1999

 J.W. Bandler, J.E. Rayas-Sánchez and Q.J. Zhang 1999

No part of this document may be copied, translated, transcribed or entered in any form into any machine without written permission.  Address enquiries in this regard to Dr. J.W. Bandler.  Excerpts may be quoted for scholarly purposes with full acknowledgement of source.  This document may not be lent or circulated without this title page and its original cover.

###### Space Mapping Based Neuromodeling

###### of High Frequency Circuits

J.W. Bandler, J.E. Rayas-Sánchez and Q.J. Zhang*

Simulation Optimization Systems Research Laboratory

and Department of Electrical and Computer Engineering

McMaster University, Hamilton, Canada L8S 4K1

* Department of Electronics

Carleton University, Ottawa, Canada K1S 5B6

bandler@mcmaster.ca

www.sos.mcmaster.ca

presented at

1999 Micronet Annual Workshop, Ottawa, ON, April 26, 1999

###### Space Mapping Based Neuromodeling

###### of High Frequency Circuits

J.W. Bandler, J.E. Rayas-Sánchez and Q.J. Zhang*

Simulation Optimization Systems Research Laboratory

and Department of Electrical and Computer Engineering

##### McMaster University, Hamilton, Canada L8S 4K1

* Department of Electronics

##### Carleton University, Ottawa, Canada K1S 5B6

bandler@mcmaster.ca

www.sos.mcmaster.ca

# Abstract

We present neuromodeling of microwave and high frequency circuits based on Space Mapping (SM) technology.  Neuromo-deling of microwave circuits is briefly reviewed.  The aim of SM is described.  An innovative scheme to combine SM techno-logy and ANN for modeling is proposed.  SM based neuromo-dels decrease the cost of training, improve generalization ability and reduce the complexity of the ANN topology w.r.t. classical neuromodeling.  Huber optimization is proposed to train the neuro-space-mapping (NSM).  Space-Mapped Neuromodeling (SMN) is illustrated by a microstrip line with high dielectric constant.

The Aim of Space Mapping

(Bandler et al., 1994-)

Artificial Neural Network (ANN) Modeling

Artificial Neural Networks can model high-dimensional and highly nonlinear problems

ANN models are computationally efficient and can be more accurate than empirical models

the size of an ANN does not grow exponentially with dimension

(White et al., 1992)

in theory, it can approximate any degree of nonlinearity to any desired level of accuracy, provided a deterministic relationship between input and target exists (White et al., 1992)

ANNs that are too small cannot approximate the desired input-output relationship

ANNs with too many internal parameters perform correctly in the learning set, but give poor generalization ability

ANNs are suitable models for microwave circuit optimization and statistical design (Zaabab, Zhang and Nakhla, 1995, Gupta et al., 1996, Burrascano and Mongiardo, 1998, 1999)

Classical Neuromodeling of Microwave Components

## large amount of training data is usually needed to ensure model accuracy

the number of learning samples needed to approximate a function grows exponentially with the ratio between the dimensionality  and its degree of smoothness (Stone, 1982)

even with sufficient training data, the reliability of MLP for extrapolation may be very poor

Neural Space Mapping

using a three layer perceptron (3LP)

Space Mapped Neuromodeling (SMN) Concept

once the ANN is trained

## Starting Point and Learning Samples

the starting point for the optimization problem is chosen such that xc ≈ xf

to keep a reduced set of learning data samples, we consider an n-dimensional star distribution for the learning base points (Bandler et al., 1989)

the number of learning base points for a microwave circuit with n design parameters is Bp = 2n + 1

Training the ANN for SMN

the neuromapping can be found by solving the optimization problem

minw  [e1Te2T⋯elT] T

w contains the internal parameters of the ANN (weights, bias, etc.) selected as optimization variables

l is the total number of learning samples

ek is the error vector given by

ek = Rf(xfi, freqj) − Rc(xc, freqj)

xc = P (xfi)

with

i = 1,…, Bp

j = 1,…, Fp

k=j+Fp(i−1)

Microstrip Line with High Dielectric Constant

region of interest

5mil  W  9mil

15mil  H  25mil

40mil  L  60mil

20  r  25

27GHz  freq  30GHz.

“coarse” model: Pozar’s formulas (Pozar, 1998)

“fine” model: Sonnet’s em

learning set: 9 base points with “star” distribution

testing set: 50 random base points in the region of interest

Microstrip Line Response Errors

comparison before neuromodeling between em and Pozar’s model at 50 random test points

## SMN Model for the Microstrip Line (3LP:4-3-4)

## SMN Model Results for the Microstrip Line

comparison between em and the SMN model

Other SM Based Neuromodeling techniques

four new SM based neuromodeling techniques have been developed

Frequency-Dependent Space Mapped Neuromodeling (FDSMN)

Frequency Space Mapped Neuromodeling (FSMN)

Frequency Mapped Neuromodeling (FMN)

Frequency Partial-Space Mapped Neuromodeling (FPSMN)

they make even better use of the implicit knowledge of the coarse model

these techniques have been applied to a microstrip right angle bend and to an HTS filter, with excellent results

New Results

J.W. Bandler, M.A. Ismail, J.E. Rayas-Sánchez and Q.J. Zhang, “Neuromodeling of microwave circuits exploiting space mapping technology,” IEEE MTT-S Int. Microwave Symp. (Anaheim, CA), June 1999.

J.W. Bandler and J.E. Rayas-Sánchez, “Circuit CAD and modeling through space mapping,” IEEE MTT-S Int. Microwave Symp., Workshop WSFD (Anaheim, CA), June 1999.

J.W. Bandler, M.A. Ismail, J.E. Rayas-Sánchez and Q.J. Zhang, “New directions in model development for RF/microwave components utilizing artificial neural networks and space mapping,” IEEE AP-S Int. Symp. (Orlando, FL), July 1999.

J.W. Bandler, J.E. Rayas-Sánchez and Q.J. Zhang, “Neural modeling and space mapping: two approaches to circuit design,” XXVI URSI General Assembly (Toronto, ON), August 1999.

Conclusions

we present novel applications of Space Mapping technology to the neuromodeling of microwave circuits

Space Mapped Neuromodels (SMN) are described and illustrated

these techniques

exploit the vast set of empirical models already available

decrease the fine model evaluations needed for training

improve generalization ability

reduce complexity of the ANN topology

w.r.t. the classical neuromodeling approach

Huber optimization efficiently trains the neuromappings, exploiting its robust characteristics for data fitting