REALIZATIONS OF Space Mapping BASED NEUROMODELS OF MICROWAVE COMPONENTS

J.W. Bandler, J.E. Rayas-Sánchez, F. Wang* and Q.J. Zhang*

Simulation Optimization Systems Research Laboratory

and Department of Electrical and Computer Engineering

McMaster University, Hamilton, Canada L8S 4K1

* Department of Electronics

Carleton University, Ottawa, Canada K1S 5B6

bandler@mcmaster.ca

www.sos.mcmaster.ca

presented at

AP2000 Millennium Conference on Antennas and Propagation, Davos, Switzerland, April 13, 2000

Artificial Neural Networks (ANN) in Modeling

Artificial Neural Networks are suitable in modeling high-dimensional and highly nonlinear problems

ANN models are computationally efficient and can be more accurate than empirical models

multilayer feedforward networks can approximate any measurable function to any desired level of accuracy, provided a deterministic relationship between input and target exists

(White et al., 1992)

ANNs that are too small cannot approximate the desired input-output relationship

ANNs with too many internal parameters perform correctly in the learning set, but give poor generalization ability

ANNs are suitable models for microwave circuit optimization and statistical design (Zaabab, Zhang and Nakhla, 1995, Gupta et al., 1996, Burrascano and Mongiardo, 1998, 1999)

Space Mapping-based neuromodeling techniques significantly decrease the number of EM simulations needed for training

(Bandler et al., 1999)

Conventional Neuromodeling of Microwave Components

## many learning samples are usually needed to ensure model accuracy

the number of learning samples needed to approximate a function grows exponentially with the ratio of the dimensionality to the function’s degree of smoothness

(Stone, 1982)

even with sufficient training data, the reliability of MLPs for extrapolation may be very poor

The Aim of Space Mapping

(Bandler et al., 1994-)

Neural Space Mapping

using a three layer perceptron (3LP)

Space Mapped Neuromodeling (SMN) Concept

once the ANN is trained

Frequency-Dependent Space Mapped Neuromodeling (FDSMN) Concept

once the ANN is trained

Frequency Space Mapped Neuromodeling (FSMN) Concept

once the ANN is trained

Frequency Mapped Neuromodeling (FMN) Concept

once the ANN is trained

Frequency Partial-Space Mapped Neuromodeling

(FPSMN) Concept

once the ANN is trained

Training the ANN

the neuromapping can be found by solving the optimization problem

minw  [e1Te2T⋯elT] T

w contains the internal parameters of the ANN (weights, bias, etc.) selected as optimization variables

l is the total number of learning samples

ek is the error vector given by

for SMN

ek = Rf(xfi, freqj) − Rc(xc, freqj)

xc = P (xfi)

for FDSMN

ek = Rf(xfi, freqj) − Rc(xc, freqj)

xc = P (xfi, freqj)

for FSMN

ek = Rf(xfi, freqj) − Rc(xc, fc)

Training the ANN (continued)

xcfc = P (xfi, freqj)

for FMN

ek = Rf(xfi, freqj) − Rc(xfi, fc)

fc = P (xfi, freqj)

for FPSMN

ek = Rf(xfi, freqj) − Rc(xfsi, xcs, fc)

xcsfc = P (xfi, freqj)

with

i = 1,…, Bp

j = 1,…, Fp

k=j+Fp(i−1)

## Starting Point and Learning Samples

we chose a unit mapping (xc  x f and fc  freq) as the starting point for the optimization problem

to keep a reduced set of learning data samples, we consider an n-dimensional star distribution for the learning base points (Bandler et al., 1989)

the number of learning base points for a microwave circuit with n design parameters is Bp = 2n + 1

Microstrip Right Angle Bend

region of interest

20mil  W  30mil

8mil  H  16mil

8  r  10

1GHz  freq  41GHz

“coarse” model: Gupta model (Gupta, Garg and Bahl, 1979)

“fine” model: Sonnet’s em

learning set: 7 base points with “star” distribution

testing set: 50 random base points in the region of interest

Microstrip Right Angle Bend Response Errors

comparison before neuromodeling between em and Gupta model at 50 random test points

FSMN Model for the Right Angle Bend (3LP:4-8-4)

strategy implemented in NeuroModeler

FSMN Model for the Right Angle Bend (3LP:4-8-4)

implementation in NeuroModeler Version 1.2b (1999)

Implementation in NeuroModeler

layer one, in green, has the input parameters of the neuromapping (W, H, r, and freq) scaled to 1

layer two corresponds to the hidden layer of the ANN implementing the mapping (8 hidden neurons with sigmoid non-linearities)

layer three is linear and contains the coarse design parameters xc and the mapped frequency fc before de-scaling

layer four de-scales the parameters

Gupta’s formulas to calculate L and C are programmed as the internal analytical functions of the fifth hidden layer, using the built-in MultiSymbolicFixed function

the output layer, in blue, contains a simple internal circuit simulator that computes the real and imaginary parts of S11 and S21 for the lumped LC equivalent circuit (this layer uses the built-in CktSimulatorPS function)

FSMN Model Results for the Right Angle Bend

errors in the learning set after training (w.r.t. em)

FSMN Model Results for the Right Angle Bend

errors in the testing set after training (w.r.t. em)

FSMN Model Results for the Right Angle Bend

comparison between em and the FSMN model

Conclusions

we present novel realizations of Space Mapping based neuromodels of practical passive microwave components using available software

five powerful SM based neuromodeling techniques are described

these techniques

exploit the vast set of empirical models already available

decrease the fine model evaluations needed for training

improve generalization ability

reduce complexity of the ANN topology

w.r.t. the classical neuromodeling approach

frequency-sensitive neuromappings expand the usefulness of empirical quasi-static models

an SM based neuromodel of a microstrip right angle bend is implemented using NeuroModeler Version 1.2b (1999)

this model can be entered into Agilent ADS Version 1.1 (1999) as a library component through an ADS plugin module