# SPACE MAPPING OPTIMIZATION OF MICROWAVE CIRCUITS EXPLOITING SURROGATE MODELS

M.H. Bakr, J.W. Bandler, K. Madsen, J.E. Rayas-Sánchez and J. Søndergaard

McMaster University, Hamilton, Canada L8S 4K1

www.sos.mcmaster.ca

### Abstract

A powerful new Aggressive Space Mapping optimization algorithm is presented.  It draws upon recent developments in both surrogate-based optimization and microwave device neuromodeling.  Our surrogate formulation (new to microwave engineering) exploits, in a novel way, a linear frequency-space mapping.  This is a powerful approach to severe response misalignments.

Introduction

We present a novel Aggressive Space Mapping algorithm for microwave circuit optimization.  It integrates, for the first time, two distinct optimization approaches: Space Mapping (SM) optimization [1] and surrogate-based opti-mization [2].  Both approaches aim at efficiently optimizing an accurate and time-intensive “fine” model, e.g., a full-wave electromagnetic simu-lator.  SM exploits the existence of a less accurate but fast “coarse” model.  It formulates the design problem as a system of nonlinear equations.  On the other hand, surrogate-based optimization, new to the microwave arena, exploits an approximate model iteratively in solving the original design problem.

Our algorithm combines both approaches.  The original design problem is solved using an approximate surrogate model.  This model is a convex combination of a mapped coarse model and a linearized fine model.  The accuracy of the surrogate model is improved in every iteration.

Recent developments in Space Mapping-based Neuromodeling (SMN) [3] exploit frequency-sensitive mappings.  We integrate this concept, in a novel way, with SM optimization.  In each iteration, a linear frequency-space mapping is exploited in constructing the mapped coarse model.  During the optimization iterates, the coarse and fine models are simulated over different sets of frequencies.  The algorithm is demonstrated through the design of a high temperature superconducting (HTS) filter [4].

### Space Mapping Optimization vs. Optimization via Surrogates [1, 2]

We denote the fine model responses at a point xf∈ℜn×1 and frequency  by Rf(xf,ω)∈ ℜNr×1.  These responses may include the real and imaginary parts of S11, etc.  The response vector Rf(xf)∈ℜm×1 denotes the responses over all the Nω simulation frequencies where m=NrNω.  The original design problem is

xf=argminxf U(Rf(xf))           (1)

where U is the objective function and xf is the optimal fine model design.  Solving (1) using direct optimization methods can be prohibitive due to the model’s intensive simulation time.

SM optimization exploits the existence of a fast but less accurate “coarse” model of the circuit.  We denote by xc∈ℜn×1 and Rc(xc)∈ℜm×1 a coarse model point and the corresponding response, respectively.  SM establishes a mapp-ing xc=P(xf) between the two spaces such that Rf(xf)≈Rc(xc)[1].  The space-mapped design xf is a solution of the nonlinear system

f(xf)=P(xf)−xc=0              (2)

where P(xf) is approximated through Parameter Extraction (PE) and xc is the optimal coarse design.

Previous SM-based optimization algorithms solve (2) iteratively.  Let xf(i) be the ith iterate in the solution of (2).  The original ASM algorithm utilizes a quasi-Newton step.  Our TRASM algorithm minimizes f(xf(i+1)) using least squares within a trust region.

Here, a time-intensive model is optimized using a “surrogate” model.  We denote the surrogate model in the ith iteration by Rs(i)(xf)∈ℜm×1.  The step taken is obtained by solving

h(i)=argminh(i) U(Rs(i)(xf(i)+h(i))), h(i)≤δ(i)

(3)

where δ(i) is the trust region size.  The point xf(i)+h(i) is then validated using fine model simulation.  It is accepted if it improves the desired fine model objective function.  Other-wise, the accuracy of Rs(i)(xf) should be improved using fine model validations.  Additional fine model simulations may be generated to improve the surrogate model in certain directions of the parameter space.

### OUR SURROGATE MODEL

In the ith iteration, our algorithm utilizes a surrogate model expressed as a convex combination between a linearized fine model (LFM) and a mapped coarse model (MCM) Rm(i)(xf).  It is given by

Rs(i)(xf)=λ(i)Rm(i)(xf)+(1−λ(i))(Rf(xf(i))+Jf(i)Δxf),λ(i)∈0,1    (4)

where Jf(i)∈ℜm×n is an approximation to the Jacobian of fine model responses at xf(i) and Δxf= xfxf(i).  The parameter λ(i) determines which of the models LFM or MCM is favored.

The MCM utilizes the linear frequency-space mapping

| Rf(xf,ωj)≈Rm(i)(xf,ωj)=Rc(P(i)(xf,ωj),Pω(i)(xf,ωj)), j=1, 2, , Nω | (5) |
| --- | --- |

where

P(i)(xf,ωj)Pω(i)(xf,ωj)=B(i)s(i)t(i)Tσ(i)Δxfωj+c(i)γ(i), (6)

The parameters B(i)∈ℜn×n, s(i)∈ℜn×1, t(i)∈ℜn×1, c(i)∈ℜn×1, σ(i)∈ℜ1×1 and γ(i)∈ℜ1×1 are the mapping parameters.  ωj is the jth sim-ulation frequency, j=1, 2, , Nω.  Here, a fine model point xf and frequency ωj correspond to a coarse point P(i)(xf,ωj) and frequency Pω(i)(xf,ωj).

The mapping parameters of (6) are obtained so that the MCM approximates the fine model over a region of fine model parameters and frequency.  They are obtained by the optimization procedure

[B(i),s(i),t(i),σ(i), c(i),γ(i)]=argminB,s,t, σ, c, γ  [e1Te2T⋯eNpT]T   (7)

ek=Rm(i)(xf(k))−Rf(xf(k)) xf(k)∈V(i)        (8)

where V(i) is a set of fine model points whose cardinality is V(i)=Np and ek∈ℜm×1.  V(i) is mainly composed of a subset of previously simulated points.  It contains points that are within an -neighborhood of xf(i) and sufficient-ly cover this neighborhood.  Additional points may be simulated by the algorithm.  This occurs if the algorithm fails to make a successful iteration and the previously simulated fine points do not adequately cover the -neighborhood.

### THE ALGORITHM

The ith iteration of the algorithm proceeds as follows.  First, the set V(i) is constructed.  The mapping parameters are then estimated using the optimization procedure (7)-(8).  The step h(i) is obtained by solving (3), where the surrogate model is given by (4).  Notice that (3) utilizes only coarse model simulations and can be solved by traditional optimization methods.

h(i) is accepted if it improves the objective function.  Otherwise, it is rejected.  The parameters Jf(i), δ(i) and λ(i) are updated in each iteration.  Broyden’s formula [5] is used to update Jf(i).  Initially, we set Jf(1)=Jc, the Jacobian of the coarse model response at xc.  The trust region δ(i) is updated based on how the actual reduction ra in U matches the predicted reduction rp.  The ratio

ρ=rarp=U(Rf(xf(i)))−U(Rf(xf(i)+h(i)))U(Rs(i)(xf(i)))−U(Rs(i)(xf(i)+h(i)))  (9)

is thus evaluated at the end of each iteration.  If ρ≥0.75, the surrogate model has good accuracy and we set δ(i+1)=π1δ(i), π1>1.0.  If ρ≤0.10, we set δ(i+1)=π2δ(i), 0<π2<1.0.  Otherwise, we set δ(i+1)=δ(i).  λ(i) is updated to favor the more accurate model, either the LFM or the MCM.  It is initialized by λ(1)=1.  The actual update utilized is

λ(i+1)=El(i)El(i)+Em(i)              (10)

where Em(i)=Rm(i)(xf(i)+h(i))−Rf(xf(i)+h(i)) and El(i)=Rf(xf(i))+Jf(i)h(i)−Rf(xf(i)+h(i)) define the prediction error using the MCM and the LFM, respectively.  The algorithm terminates if n+1 consecutive unsuccessful iterations are carried out or if h(i) becomes sufficiently small.  One iteration of the algorithm is shown in Fig. 1.



rp

ra

0

1

Fig. 1. Illustration of the ith iteration of the algorithm.

The HTS Filter

We consider the design of an HTS filter [4] (see Fig. 2).  The design specifications are

S21  0.05 for   3.967 GHz and

4.099 GHz       (11)

S21  0.95 for 4.008 GHz    4.058 GHz

(12)

The design parameters are L1, L2, L3, S1, S2 and S3.  We take L0 = 50 mil and W = 7 mil.  The coarse model exploits the empirical models of microstrip lines, coupled lines and open stubs available in OSA90/hope [6].  The fine model employs Sonnet’s em [7] through Empipe [6].

The fine model is simulated at 16 frequency points.  We utilize the real and imaginary parts of

both S11 and S21 in the optimization procedure (7)-(8).  The initial trust region is δ(1)=0.20xc∞.  The interpolation option of

Empipe is disabled to make the simulation time per sweep feasible.  The grid dimensions in the x and y directions are 1.0 and 1.75 mils, respectively.  The substrate dielectric constant is 23.425.  The metal and the substrate are both assumed lossless.

Starting from the snapped optimal coarse design [L1, L2, L3, S1, S2, S3]= [188, 198, 189, 22.75, 99.75, 112] mils, the final design [188, 192, 187, 22.75, 78.75, 91] mils is reached in only 7 iterations, comprising only 7 fine model sweeps.  The initial and final responses are shown in Fig 3.

L0

L1

L2

L3

L2

L1

L0

S1

S2

S3

S2

S1

W

x

y

Fig. 2. The HTS filter [4].

conclusions

3.887

3.965

4.043

4.121

4.199

frequency (GHz)

-100

-80

-60

-40

-20

0

S21dB

Fig. 3.Results for the HTS filter; The optimal coarse response (), the initial fine response () and the final fine model design ().

We present a breakthrough algorithm for efficient optimization of microwave circuits.  The algorithm integrates, for the first time, SM optimization with optimization via surrogates.  It exploits a novel surrogate formulation in the form of a convex combination of a mapped coarse model and a linearized fine model.  The MCM model utilizes, in a novel way, a frequency-space mapping.  During optimization, the coarse and fine models are simulated over different frequency ranges.  The algorithm is successfully illustrated through the design of a microwave filter.

### References

[1]	M.H. Bakr, J.W. Bandler, N. Georgieva and K. Madsen, “A hybrid aggressive space mapping algorithm for EM optimization,” IEEE Trans. Microwave Theory Tech., vol. 47, 1999, pp. 2440-2449.

[2]	A.J. Booker, J.E. Dennis, Jr., P.D. Frank, D. B. Serafini, V. Torczon and M.W. Trosset, “A rigorous framework for optimization of expensive functions by surrogates,” Structural Optimization, vol. 17, 1999, pp. 1-13.

[3]	J.W. Bandler, M.A. Ismail, J.E. Rayas-Sánchez and Q.J. Zhang, “Neuromodeling of microwave circuits exploiting space mapping technology,” IEEE Trans. Microwave Theory Tech., vol. 47, 1999, pp. 2417-2427.

[4]	J.W. Bandler, R.M. Biernacki, S.H. Chen, W.J. Gestinger, P.A. Grobelny, C. Moskowitz and S.H. Talisa, “Electromagnetic design of high-temperature superconducting filters,” Int. J. Microwave and Millimeter-Wave Computer-Aided Engineering, vol. 5, 1995, pp. 331-343.

[5]	C.G. Broyden, “A class of methods for solving nonlinear simultaneous equations,” Math. Comp., vol. 19, 1965, pp. 577-593.

[6]	OSA90/hope and Empipe, Version 4.0, formerly Optimization Systems Associates Inc., P.O. Box 8083, Dundas, ON, Canada, L9H 5E7, 1997, now Agilent Technologies, 1400 Fountaingrove Parkway, Santa Rosa, CA 95403-1799.

[7]	em, Sonnet Software, Inc., 1020 Seventh North Street, Suite 210, Liverpool, NY 13088, 1997.

This work was supported in part by the Natural Sciences and Engineering Research Council of Canada under Grants OGP0007239, STP0201832, Com Dev, and through the Micronet Network of Centres of Excellence.  M.H. Bakr is supported by an Ontario Graduate Scholarship.  J.E. Rayas-Sánchez is funded by CONACYT (Consejo Nacional de Ciencia y Tecnología, Mexico), as well as by ITESO (Instituto Tecnológico y de Estudios Superiores de Occidente, Mexico).

J.W. Bandler is also with Bandler Corporation, P.O. Box 8083, Dundas, Ontario, Canada L9H 5E7.

K. Madsen and J. Søndergaard are with the Department of Mathematical Modelling, Technical University of Denmark, DK-2800 Lyngby, Denmark.