# Broadband physics-based modeling of microwave passive devices through frequency mapping

J.W. Bandler, M.A. Ismail and J.E. Rayas-Sánchez

McMaster University, Hamilton, Canada L8S 4K1

www.sos.mcmaster.ca

Abstract

We present a new methodology to develop physics-based models for passive components.  We coherently integrate full-wave EM simula-tions, artificial neural networks, multivariable rational functions, dimensional analysis and frequency mapping.  We consider frequency-independent and frequency-dependent mod-els.  Various examples include a microstrip right angle bend and a CPW short-circuit stub.

### Introduction

- We present a new computer-aided modeling methodology to develop physics-based empirical models (“coarse” models) for microwave passive components.  We consider frequency-independ-ent empirical models (FIEM) and frequency-dependent empirical models (FDEM).  In the FDEM we use the frequency mapping approach [1] to introduce frequency dependency into the model elements.  We also exploit the odd property of the frequency mapping, that is the transformed or “coarse” model frequency must be an odd function of frequency.  ANNs or MRFs [2] are used to approximate the model elements as well as the frequency mapping.  MRFs enable us to transform a simple FDEM to an equivalent FIEM.  This transformation can be expedited by impedance synthesis [3] as we will see in the examples.  Dimensional analysis [4,5] reduces the number of input parameters to the ANN or the MRF.  We illustrate the process through various examples, including a microstrip right angle bend and a CPW short-circuit stub.

### Frequency-Independent Empirical Models (FIEM)

Consider a microwave component modeled by a fine model (typically a suitable full-wave EM simulator) and a coarse, equivalent circuit (empirical) model.  We assume that the model topology is known but empirical formulas are to be determined.  Let xf be an n-dimensional vector representing the parameters of the component, Rf  is a vector representing the fine model responses, e.g., the scattering parameters,  is the frequency and Rc is a vector representing the coarse model responses.  The development of the FIEM is carried out by evaluating an l-dimensional vector y which represents the empirical formulas.  Applying dimensional analysis y becomes a function of an nr-dimensional vector xr (nr < n), which we call the reduced input parameter vector.  Through ANNs or MRFs [2] we approximate y in a region of parameters and frequency as

| y≈Q(xr,w) | (1) |
| --- | --- |

where w is the set of parameters of the ANN or the MRF.  This set w is evaluated by solving

| min w [e11T⋯e1MT e21T⋯e2MT⋯eN1T⋯eNMT]T | (2) |
| --- | --- |

where  is a suitable norm, N is the total number of training points, M is the number of frequency points per frequency sweep and eij is an error vector given by

| eij = Rf(xfi,ωj)−Rc(Q(xr i,w), ωj) | (3) |
| --- | --- |

Training points are selected according to the Central Composite Design (CCD) [6].  More points are added if necessary.

### Frequency-Dependent Empirical Models (FDEM)

Two approaches can be used to introduce frequency dependency to the elements of the FDEM.  One approach is to make the reduced vector xr and hence y depend on frequency as well as other physical parameters.  The second approach exploits the frequency mapping concept [1], where we simulate the coarse model at a different frequency from the fine model.  We call this frequency the coarse model frequency c.  Frequency transformations (mappings) have roots in classical filter design, for example, low-pass to band-pass transformations [7].  Dimensional analysis is also applied to determine the dependency of c on  as well as the physical parameters.  Both y and c can be approximated by an ANN or an MRF as

| y≈Q(xr,w1), ωc≈Ω(xr,ω,w2) | (4) |
| --- | --- |

where w1 and w2 are the parameters of the ANN or MRF.  These parameters are evaluated by solving an optimization problem similar to (2).

### Properties of the Frequency Mapping

Simulating the coarse model at a different frequency from that of the fine model implicitly introduces frequency dependency to the coarse model.  For example, if the device is lossless the model contains only lossless lumped-elements (inductors and capacitors).  In this case, an FDEM simulated at c and with a circuit element vector y is equivalent to an FDEM simulated at  and circuit element vector y1=(c/)y.  Further-more, c should be an odd function of .  This results from the even and odd properties [7] of an arbitrary frequency-dependent impedance Z().  For example, if an inductor L is simulated at frequency c the equivalent impedance ZL = jc L is purely imaginary, hence ZL and consequently c should be an odd function of .  (The odd property is also preserved for low-pass filter to high- or band-pass transformations [7]).  Exploit-ing this property with dimensional analysis further reduces the number of ANN or MRF parameters approximating c.

### Transformation of FDEMs into FIEMs

The advantage of using MRFs to approximate the frequency mapping is that we can transform the FDEM into an equivalent FIEM.  This transformation involves one-port impedance synthesis, which states that the impedance we want to realize should be a rational function.  For example, the frequency mapping used in two examples presented here (microstrip right angle bend and microstrip via) takes the form

| ωc=ω f1−ω2f2f3−ω2f4 | (6) |
| --- | --- |

where f1, f2, f3, f4 are polynomials of the device physical parameters.  We believe that (6) may be useful for other devices such as microstrip mitered bends, microstrip step junctions, etc.

To display the results in a compact way we define the error in the scattering parameter Sij as the modulus of the difference between the scattering parameter Sijf computed by the fine model and the scattering parameter Sijc computed by the coarse model

| error in Sij=Sijf−Sijc | (7) |
| --- | --- |

where i = 1,2,..., M and j = 1, 2,..., M (M is the number of ports of the microwave device).

### Microstrip Right Angle Bend

Here, we develop an FIEM and an FDEM for a microstrip right angle bend.  The fine model is analyzed by Sonnet’s em [8] and the coarse model consists of the LC circuit [9].  The vector of input parameters xf=[W H εr]T and the vector y=[LH CH]T.  Applying dimensional analysis [4,5], we can show that y is given by

LH=μ0 f(W/H), CH= ε0 f(W/H, εr)    (8)

Therefore, y is a function of xr=[W/H εr]T.  We first develop an FIEM in the frequency range [1, 11] GHz.  The region of interest is 0.2 <W/H< 6 and 2 <r< 11.  The substrate height H is chosen in the range [5, 30] mil.  We use a three-layer perceptron ANN to approximate y.  The training points are chosen according to the Central Composite Design (CCD) [6] in addition to 4 more points (total 13 training points).  The parameters of the ANNs are obtained by the Huber optimizer in OSA90/hope [10].  Fig. 1 shows the error in the scattering parameter S11 at 16 test points in the region of interest for the FIEM.  These results are comparable with those of the Jansen model [11] at the same test points.

The results obtained by the FIEM and by the Jansen empirical model [11] over the range [1, 31] GHz are shown in Figs. 2 (a) and (b), respectively.  It is clear that neither the FIEM nor the empirical model in [11] are accurate at high frequencies.  Therefore, we develop an FDEM, where c is a function of  and the other parameters.  Applying dimensional analysis and using the odd property of c we get

| ωc=ω γ(W/H, εr, (ωH/c)2) | (9) |
| --- | --- |

where c is the speed of light and  is an unknown function to be approximated.  We use MRFs to approximate y as well as .  Fig. 3 shows the error in the scattering parameters S11 at 16 test points in the region of interest for the FDEM.

We transform the FDEM into an equivalent FIEM as follows.  The frequency c is given by (6), hence the impedances associated with L and C are given by

| ZL=jωcL=j ωLf1−ω2f2f3−ω2f4 ZC=1jωcC=1jωf3−ω2f4f1−ω2f2 | (10) |
| --- | --- |

which can be realized by network synthesis [3], in our case by the first Foster realization.

Fig. 1.	The error in S11 of the microstrip right angle bend FIEM with respect to emTM at the test points.

(a)

(b)

Fig. 2.	The error in S11 of the microstrip right angle bend with respect to emTM at the test points: (a) the FIEM in [1, 31] GHz; (b) the empirical model from [11].

Fig. 3.	The error in S11 of the microstrip right angle bend FDEM with respect to emTM at the test points.

### Coplanar Waveguide (CPW) Short-circuit Stub

In this example, we develop an FIEM for the CPW short-circuit stub.  The reference plane is taken at the stub end.  The fine model is analyzed by Momentum [12] and the coarse model consists of an inductance L to ground.  The input parameter vector xf=[W G H]T. The vector y contains only one element, namely

| LH=μ0 f(W/H, G/W) | (13) |
| --- | --- |

Therefore, y is a function of xr=[W/H GW]T.  We develop an FIEM in the range [1, 25] GHz.  The region of interest is 0.2 <W/H< 2 and 0.2 <G/W< 2.  The substrate used is GaAs (r=12.9) with height H in the range [100, 635] m.  We use a three-layer perceptron ANN  to approxim-ate L/H.  The training points are chosen according to the Central Composite Design (CCD) [6] in addition to 4 more points (total 13 training points).

The worst case % errors in S11 of the FIEM of the CPW short-circuit stub is 2%.  We observe good results of the FIEM in the range [1, 40] GHz (worst case % error of 4%).

### Conclusions

- We present a unified computer-aided modeling methodology for developing broadband models of microwave passive components.  Two types of model are considered: FIEMs and FDEMs.  FDEMs can be transformed to equivalent FIEMs if we use an MRF to approximate the frequency mapping.  This is important since the FIEMs are readily implementable in conventional circuit simulators.  We applied our modeling metho-dology to develop broadband empirical models for several microwave components.

### REFERENCES

- [1]	J.W. Bandler, M.A. Ismail, J.E. Rayas-Sánchez and Q.J. Zhang, “Neuromodeling of microwave circuits exploiting space mapping technology,” IEEE Trans. Microwave Theory Tech., vol. 47, 1999, pp. 2417-2427.

- [2]	H. Leung and S. Haykin, “Rational function neural network,” Neural Computation, vol. 5, 1993, pp. 928-938.

- [3]	G.C. Temes and J.W. Lapatra, Introduction to Circuit Synthesis and Design.  New York: McGraw-Hill, Inc., 1977.

- [4]	W.H. Middendorf, Design of Devices and Systems.  New York: Marcel Dekker, Inc., 1986, Chapter 7.

- [5]	P. Watson, M.Y. Mah and L.L. Liou, “Input variable space reduction using dimensional analysis for artificial neural network modeling,” IEEE MTT-S Int. Microwave Symp. Dig. (Anaheim, CA), 1999, pp. 269-272.

- [6]	D.C. Montgomery, Design and Analysis of Experiments.  New York: Wiley, 1991.

[7]	R.E. Collin, Foundations for Microwave Engineer-ing.  New York: McGraw-Hill, Inc., 1966.

[8]	emTM Version 4.0b, Sonnet Software, Inc., 1020 Seventh North Street, Suite 210, Liverpool, NY 13088, 1997.

- [9]	K.C. Gupta, R. Garg and I.J. Bahl, Microstrip Lines and Slotlines.  Dedham, MA: Artech House, 1979.

[10]	OSA90/hopeTM Version 4.0, formerly Optimization Systems Associates Inc., P.O. Box 8083, Dundas, Ontario, Canada L9H 5E7, now Agilent Technologies, 1400 Fountaingrove, Parkway, Santa Rosa, CA 95403-1799.

[11]	M. Kirschning, R. Jansen and N. Koster, “Measurement and computer-aided modeling of microstrip discontinuities by an improved resonator method,” IEEE MTT-S Int. Microwave Symp. Dig. (Boston, MA), 1983, pp. 495-497.

- [12]	MomentumTM Version 3.0, Agilent Technologies, 1400 Fountaingrove Parkway, Santa Rosa, CA 95403-1799, 1998.

This work was supported in part by the Natural Sciences and Engineering Research Council of Canada (NSERC) under Grants OGP0007239, STP0201832 and through the Micronet Network of Centres of Excellence.

J.W. Bandler is also with Bandler Corporation, P.O. Box 8083, Dundas, Ontario, Canada L9H 5E7.