SPACE MAPPING BASED DEVICE MODELING

AND CIRCUIT OPTIMIZATION

J.W. Bandler, J.E. Rayas-Sánchez

M.A. Ismail and M.H. Bakr

Simulation Optimization Systems Research Laboratory

and Department of Electrical and Computer Engineering

McMaster University, Hamilton, Canada L8S 4K1

bandler@mcmaster.ca

www.sos.mcmaster.ca

presented at

WORKSHOP ON AUTOMATED CIRCUIT OPTIMIZATION USING ELECTROMAGNETIC SIMULATORS

2000 IEEE MTT-S International Microwave Symposium, Boston, MA, June 16, 2000

SPACE MAPPING BASED DEVICE MODELING

AND CIRCUIT OPTIMIZATION

J.W. Bandler, J.E. Rayas-Sánchez

M.A. Ismail and M.H. Bakr

Simulation Optimization Systems Research Laboratory

and Department of Electrical and Computer Engineering

McMaster University, Hamilton, Canada L8S 4K1

bandler@mcmaster.ca

www.sos.mcmaster.ca

# Abstract

Electromagnetics (EM) based device modeling and circuit optimization through Artificial Neural Network (ANN) and Space Mapping (SM) technologies are reviewed.  These two concepts continue to promise important benefits in the next generation of design optimization methodologies.  ANNs can learn from and generalize patterns in data and model nonlinear relationships.  On the other hand, Aggressive Space Mapping (ASM) optimization closely follows the traditional experience and intuition of designers, while being rigorously grounded mathematically.  Current progress in the development of suitable algorithms and software engines are presented.  The ANN and SM concepts address the contradictory challenge of exploitation of device models for CAD that are both accurate and fast.

Outline

Generalized Space Mapping (GSM) tableau approach to engineering device modeling is reviewed

new work on Space Mapping optimization exploiting surrogate models is described

a Neural Space Mapping (NSM) optimization approach exploiting our SM-based neuromodeling techniques is presented

Generalized Space Mapping (GSM)

GSM is a comprehensive framework for engineering device modeling

GSM exploits the Space Mapping (SM), the Frequency Space Mapping (FSM) (Bandler et al., 1994) and the Multiple Space Mapping (MSM) (Bandler et al., 1998) concepts to build a new engineering device modeling framework

two cases are considered:

the basic Space Mapping Super Model (SMSM) concept maps the device parameters

the Frequency-Space Mapping Super Model (FSMSM) concept maps the device parameters as well as frequency

two variations of MSM are presented (Bandler et al., 1999):

MSM for Device Responses (MSMDR) develops a different mapping for each subset of responses

MSM for Frequency Intervals (MSMFI) develops a different mapping for each frequency interval

Multiple Space Mapping (MSM) Concept

MSM for Frequency Intervals (MSMFI)

Mathematical Formulation for GSM

the kth mapping targeting the sub-response Rk or the response R in the kth frequency sub-range is given by

(xck, ωck)=Pk(xf, ω)

or, in matrix form, assuming a linear mapping

xck

the mapping parameters ck, Bk, sk, tk, σk, δk can be evaluated, directly or indirectly, by solving the optimization problem

minck, Bk, sk, tk, σk, δk  [ek1Tek2T⋯ekmT]T

where m is the number of base points selected in the fine model space and ekj is an error vector given by

ekj = Rf(xf(j), ω)−Rc(xck(j), ωck) , j=1,2,...,m

Microstrip Shaped T-Junction

the fine and coarse models

Microstrip Shaped T-Junction

the region of interest

| Parameter | Minimum value | Maximum value |
| --- | --- | --- |
| H | 15 mil | 25 mil |
| X | 5 mil | 15 mil |
| Y | 5 mil | 15 mil |
| εr | 8 | 10 |

the frequency range is 2 GHz to 20 GHz with a step of 2 GHz

the number of base points is 9 and the number of test points is 50

the width W of the input lines is determined in terms of H and εr so that the characteristic impedance of the input lines is 50 ohm

the width W1 is taken as 1/3 of the width W

the width W2 is obtained so that the characteristic impedance of the microstrip line after the step connected to port 2 is twice that of the microstrip line after the step connected to port 1

Microstrip Shaped T-Junction

MSM for Frequency Intervals (MSMFI) was developed to enhance the accuracy of the T-Junction coarse model

the total frequency range was divided into two intervals: 2 GHz to 16 GHz and 16 GHz to 20 GHz

the mapping parameters are

|  | 2 GHz to 16 GHz | 16 GHz to 20 GHz |
| --- | --- | --- |
| B | 1.04 0.07 0.01 0.08 −0.06 0.00 0.22 0.00 0.89 0.00 −0.07 −0.20 0.06 −0.03−0.00 0.07 0.99 0.04 −0.12 0.01 −0.06−0.04 0.00 −0.01 0.97 0.10 −0.06 −0.27 0.01 0.04 0.00 0.03 0.99 −0.05 −0.03 −0.13 −0.05 −0.04 −0.16 0.12 0.99 0.62 | 0.99 0.02 −0.00 0.01 −0.09 −0.01 0.13 0.05 0.85 0.01 −0.07 −0.28 0.01 −0.01−0.06 0.15 0.98 0.04 −0.25 0.00 0.02−0.10 −0.06 −0.03 0.88 0.13 −0.09 −0.27 0.08 0.04 0.03 0.11 1.07 −0.04 −0.12−0.14 −0.02 −0.05 −0.15 0.23 1.03 0.51 |
| c | [ 0.02 0.01 −0.01 −0.03 −0.01 0.07 −0.03 ]T | [ 0.01 0.01 −0.01 −0.03 −0.01 0.05 −0.03]T |
| s | [−0.01 0.09 −0.10 −0.02 0.00 −0.02 −0.20 ]T | [0.00 0.01 −0.01 0.00 0.00 0.00 −0.02]T |
| t | 0 | [0.01 0.00 −0.02 0.00 0.00 0.00 0.00]T |
| σ | 0.851 | 0.957 |
| δ | 0.003 | 0.008 |

Microstrip Shaped T-Junction

the responses of the shaped T-Junction at two test points in the region of interest by Sonnet’s em (●), by the coarse model (---) and by the enhanced coarse model (―)

Microstrip Shaped T-Junction

the error in S11 and in S22 of the shaped T-Junction coarse model at the test points

the error in S11 and in S22 of the shaped T-Junction enhanced coarse model at the test points

Microstrip Shaped T-Junction

the enhanced coarse model for the shaped T-Junction can be utilized in optimization

the optimization variables are X and Y

the other parameters are kept fixed (W = 24 mil, H = 25 mil and εr=9.9)

the design specifications are

S11≤13, S22≤13

in the frequency range 2 GHz to 16 GHz

the minimax optimizer in OSA90/hope reached the solution

X = 2.1 mil and Y = 21.1 mil

Microstrip Shaped T-Junction

responses of the optimum shaped T-Junction by Sonnet’s em (●), by the coarse model (---) and by the enhanced coarse model (―)

.

Space Mapping Optimization Exploiting Surrogates

a powerful new Space Mapping (SM) optimization algorithm is presented

it draws upon recent developments in both surrogate model-based optimization and modeling of microwave devices

SM optimization is formulated as a general optimization problem of a surrogate model

this model is a convex combination of a mapped coarse model and a linearized fine model

it exploits, in a novel way, a linear frequency-sensitive mapping

during the optimization iterates, the coarse and fine models are simulated at different sets of frequencies.

this approach is shown to be especially powerful if a significant response shift exists

SM Optimization vs. Surrogate Model Optimization

the optimal fine model design xf is obtained by solving

xf=argminxf U(Rf(xf))

solving this problem using direct optimization methods can be prohibitive

SM optimization algorithms efficiently solve this design problem

they exploit the existence of a less accurate but fast coarse model of the circuit under consideration

a mapping xc = P(xf) is established between the two spaces such that Rf (xf)  Rc(xc)

the space-mapped design xf is a solution of the nonlinear system

f(xf)=P(xf)−xc=0

the mapping P(xf) is approximated through Parameter Extraction (PE)

SM Optimization vs. Surrogate Model Optimization (continued)

the ASM algorithm solves this problem using a quasi-Newton method

the TRASM algorithm integrates a trust region methodology with the ASM technique

surrogate model optimization approximates the fine model at the ith iteration by a surrogate model Rs(i)(xf)  m1

the step suggested is obtained by solving

h(i)=argminh(i) U(Rs(i)(xf(i)+h(i))), h(i)≤δ(i)

h(i) is validated using fine model simulation

the accuracy of the surrogate model is improved in every iteration using the simulated fine model points

The Surrogate Model

our surrogate model is a convex combination of a mapped coarse model and a linearized fine model

the ith iteration surrogate model is

Rs(i)(xf)=λ(i)Rm(i)(xf)+(1−λ(i))(Rf(xf(i))+Jf(i)Δxf),λ(i)∈0,1

the mapped coarse model utilizes the frequency-sensitive mapping

Rf(xf,ωj)≈Rm(i)(xf,ωj)=Rc(P(i)(xf,ωj),Pω(i)(xf,ωj))

where

P(i)(xf,ωj)Pω(i)(xf,ωj)=B(i)s(i)t(i)Tσ(i)Δxfωj+c(i)γ(i)

the parameters B(i)  nn, s(i)  n1, t (i)  n1, c(i)  n1,  (i)  11  and  (i)  11  are obtained such that the mapped coarse model approximates the fine model over a given set of fine model points V (i) and frequencies 

The Surrogate Model (continued)

the mapping parameters are obtained through the optimization process

[B(i),s(i),t(i),σ(i), c(i),γ(i)]=                             argminB,s,t,σ, c,γ  [e1Te2T⋯eNpT]T where

ek=Rm(i)(xf(k))−Rf(xf(k))    ∀xf(k)∈V(i)

Illustration of One Iteration of the Algorithm



rp

ra

0

1

rp =	predicted reduction in the objective function using the surrogate model

ra =	actual reduction in the objective function

The Algorithm Flowchart

Neural Space Mapping (NSM) Optimization

exploits the SM-based neuromodeling techniques

(Bandler et al., 1999)

coarse models are used as sources of knowledge that reduce the amount of learning data and improve the generalization and extrapolation performance

NSM requires a reduced set of upfront learning base points

the initial learning base points are selected through sensitivity analysis using the coarse model

neuromappings are developed iteratively: their generalization performance is controlled by gradually increasing their complexity starting with a 3-layer perceptron with 0 hidden neurons

Neural Space Mapping (NSM) Optimization Concept

step 1

step 2

(2n + 1 learning base points for a microwave circuit with n design parameters)

Neural Space Mapping (NSM) Optimization Concept

step 3

step 4

Neural Space Mapping (NSM) Optimization Algorithm

Neuromappings

Space Mapped neuromapping

Frequency-Dependent Space Mapped neuromapping

Frequency Mapped neuromapping

Neuromappings (continued)

Frequency Space Mapped neuromapping

Frequency Partial-Space Mapped neuromapping

we chose a unit mapping (xc = x f  and c = ) as the starting point for the optimization problem

SM-Based Neuromodel Optimization

we use an SM-based neuromodel as an improved coarse model, optimizing its parameters to generate the desired response

RSMBN  is the SM-based neuromodel response

RSMBN(xf)=[RSMBN1(xf) T…RSMBNm(xf) T] T

where

RSMBNr(xf)=[Rcr(xc1,ωc1)…Rcr(xcFp,ωcFp)] T, r=1,…,m

with

xcjωcj=P(i)(xf,ωj,w)   ,  j=1,…,Fp

the next iterate is obtained by solving

xf(2n+i+1)=arg minxf U(RSMBN(xf))

if an SMN neuromapping is used to implement P(i), the next iterate can be obtained in a simpler manner by solving

xf(2n+i+1)=arg minxf PSM(i)(xf,w)−xc

HTS Quarter-Wave Parallel Coupled-Line Microstrip Filter

(Westinghouse, 1993)

NSM Optimization of the HTS Microstrip Filter

specifications

|S21|  0.95 for 4.008 GHz  f  4.058 GHz
	|S21|  0.05 for f  3.967 GHz and f   4.099 GHz

“coarse” model: OSA90/hope built-in models of open circuits, microstrip lines and coupled microstrip lines

“fine” model: Sonnet’s em with high resolution grid

we take L0 = 50 mil, H = 20 mil, W = 7 mil, r = 23.425, loss tangent = 3105; the metalization is considered lossless

the design parameters are xf = [L1 L2 L3 S1 S2 S3] T

NSM Optimization of the HTS Filter (continued)

coarse and fine model responses at the optimal coarse solution,

xc* = [188.33  197.98  188.58  21.97  99.12  111.67] T  (mils)

OSA90/hope () and em ()

NSM Optimization of the HTS Filter (continued)

the initial 2n+1 points are chosen by performing sensitivity analysis on the coarse model: a 3% deviation from xc* for L1, L2, and L3 is used, while a 20% is used for S1, S2, and S3

coarse and fine model responses at base points:

OSA90/hope

em

NSM Optimization of the HTS Filter (continued)

learning errors at base points:

before any neuromapping

mapping  with a 3LP:7-3-1

NSM Optimization of the HTS Filter (continued)

learning errors at base points:

mapping  and L1 with a 3LP:7-4-2

mapping , L1 and S1 with a 3LP:-7-5-3

NSM Optimization of the HTS Filter (continued)

em () and FPSM 7-5-3 () model responses at the next point predicted after the first NSM iteration

xf (14) = [185.37  195.01  184.24  21.04  86.36  91.39] T  (mils)

NSM Optimization of the HTS Filter (continued)

em () and FPSM 7-5-3 () model responses at the NSM solution using a fine frequency sweep

NSM Optimization of the HTS Filter (continued)

em () and FPSM 7-5-3 () model responses at the NSM solution in the passband using a fine frequency sweep

Conclusions

we review a comprehensive framework called Generalized Space mapping (GSM) to engineering device modeling

in GSM we utilize a few relevant full-wave EM simulations to match the responses of the fine model and the coarse model over a designable region of parameters and frequency

GSM generalizes the Space Mapping (SM), the Frequency Space Mapping (FSM) and the Multiple Space Mapping (MSM) concepts to build a new engineering device modeling framework

two fundamental concepts are presented: one is a basic Space Mapping Super Model (SMSM) and the other is a basic Frequency-Space Mapping Super Model (FSMSM)

MSM can be combined with SMSM and FSMSM to provide a powerful and reliable modeling tool for microwave devices

a novel SM optimization algorithm based on surrogate models is presented

SM optimization is formulated as a general optimization problem of a surrogate model

the surrogate model is a convex combination of a mapped coarse model and a linearized fine model

Conclusions (continued)

it exploits, in a novel way, a linearized frequency-sensitive mapping

we present an innovative algorithm for EM optimization based on Space Mapping technology and Artificial Neural Networks

Neural Space Mapping (NSM) optimization exploits our SM-based neuromodeling techniques

NSM does not require parameter extraction to predict the next point

an initial mapping is established by performing upfront fine model analysis at a reduced number of base points

coarse model sensitivity is exploited to select those base points

Huber optimization is used to train simple SM-based neuromodels at each iteration

the SM-based neuromodels are developed without using testing points: their generalization performance is controlled by gradually increasing their complexity starting with a 3-layer perceptron with 0 hidden neurons

an HTS filter illustrates our NSM optimization technique

References

J.W. Bandler, R.M. Biernacki, S.H. Chen and Q.H. Wang, “Multiple space mapping EM optimization of signal integrity in high-speed digital circuits,” Proc. 5th Int. Workshop on Integrated Nonlinear Microwave and Millimeterwave Circuits (Duisburg, Germany), 1998, pp. 138-140.

J.W. Bandler, N. Georgieva, M.A. Ismail, J.E. Rayas-Sánchez and Q.J. Zhang, “A generalized space mapping tableau approach to device modeling,” 29th European Microwave Conf. (Munich, Germany), vol. 3, 1999, pp. 231-234.

J.W. Bandler and J.E. Rayas-Sánchez, “Circuit CAD and modeling through space mapping,” IEEE MTT-S Int. Microwave Symp., Workshop WSFD (Anaheim, CA), 1999.

J.W. Bandler, R.M. Biernacki, S.H. Chen, R.H. Hemmers and K. Madsen, “Electromagnetic optimization exploiting aggressive space mapping,” IEEE Trans. Microwave Theory Tech., vol. 43, 1995, pp. 2874-2882.

M.H. Bakr, J.W. Bandler, R.M. Biernacki, S.H. Chen and K. Madsen, “A trust region aggressive space mapping algorithm for EM optimization,” IEEE Trans. Microwave Theory Tech., vol. 46, 1998, pp. 2412-2425.

M.H. Bakr, J.W. Bandler, N. Georgieva and K. Madsen, “A hybrid aggressive space mapping algorithm for EM optimization,” IEEE Trans. Microwave Theory Tech., vol. 47, 1999, pp. 2440-2449.

A.J. Booker, J.E. Dennis, Jr., P.D. Frank, D. B. Serafini, V. Torczon and M.W. Trosset, “A rigorous framework for optimization of expensive functions by surrogates,” Structural Optimization, vol. 17, 1999, pp. 1-13.

V. Torczon and M.W. Trosset, “Using approximations to accelerate engineering design optimization,” Technical Report 98-33, ICASE, Langley Research Center, Hampton, Virginia 23681-2199, 1998.

N. Alexandrov, J.E. Dennis, Jr., R.M. Lewis and V. Torczon, “A trust region framework for managing the use of approximation models in optimization,” Structural Optimization, vol. 15, 1998, pp. 16-23.

A.H. Zaabab, Q.J. Zhang and M.S. Nakhla, “A neural network modeling approach to circuit optimization and statistical design,” IEEE Trans. Microwave Theory Tech., vol. 43, 1995, pp. 1349-1358.

References (continued)

P. Burrascano, M. Dionigi, C. Fancelli and M. Mongiardo, “A neural network model for CAD and optimization of microwave filters,” IEEE MTT-S Int. Microwave Symp. Dig. (Baltimore, MD), 1998, pp. 13-16.

P.M. Watson and K.C. Gupta, “Design and optimization of CPW circuits using EM-ANN models for CPW components,” IEEE Trans. Microwave Theory Tech., vol. 45, 1997, pp. 2515-2523.

P.M. Watson, G.L. Creech and K.C. Gupta, “Knowledge based EM-ANN models for the design of wide bandwidth CPW patch/slot antennas,” IEEE AP-S Int. Symp. Digest (Orlando, FL), 1999, pp. 2588-2591.

J.W. Bandler, M.A. Ismail, J.E. Rayas-Sánchez and Q.J. Zhang, “Neuromodeling of microwave circuits exploiting space mapping technology,” IEEE Trans. Microwave Theory Tech., vol. 47, 1999, pp. 2417-2427.

J.W. Bandler, R.M. Biernacki, S.H. Chen, P.A. Grobelny and R.H. Hemmers, “Space mapping technique for electromagnetic optimization,” IEEE Trans. Microwave Theory Tech., vol. 42, 1994, pp. 2536-2544.

R.M. Biernacki, J.W. Bandler, J. Song and Q.J. Zhang, “Efficient quadratic approximation for statistical design,” IEEE Trans. Circuit Syst., vol. 36, 1989, pp. 1449-1454.

J.W. Bandler and S.H. Chen, “Circuit optimization: the state of the art,” IEEE Trans. Microwave Theory Tech., vol. 36, 1988, pp. 424-443.

J.W. Bandler, R.M. Biernacki, S.H. Chen, W.J. Getsinger, P.A. Grobelny, C. Moskowitz and S.H. Talisa, “Electromagnetic design of high-temperature superconducting microwave filters,” Int. J. Microwave and Millimeter-Wave CAE, vol. 5, 1995, pp. 331-343.

em Version 5.1a, Sonnet Software, Inc., 1020 Seventh North Street, Suite 210, Liverpool, NY 13088, 1998.

Empipe Version 4.0, formerly Optimization Systems Associates Inc., P.O. Box 8083, Dundas, Ontario, Canada L9H 5E7, 1997, now Agilent EEsof EDA, 1400 Fountaingrove Parkway Santa Rosa, CA 95403-1799.

OSA90/hope Version 4.0, formerly Optimization Systems Associates Inc., P.O. Box 8083, Dundas, Ontario, Canada L9H 5E7, 1997, now Agilent EEsof EDA, 1400 Fountaingrove Parkway, Santa Rosa, CA 95403-1799.

References (continued)

J.W. Bandler, M.A. Ismail and J.E. Rayas-Sánchez, “Broadband physics-based modeling of microwave passive devices through frequency mapping,” IEEE MTT-S Int. Microwave Symp. Digest (Boston, MA), 2000.

M.H. Bakr, J.W. Bandler, K. Madsen, J.E. Rayas-Sánchez and J. Søndergaard, “Space mapping optimization of microwave circuits exploiting surrogate models,” IEEE MTT-S Int. Microwave Symp. Digest (Boston, MA), 2000.

M.H. Bakr, J.W. Bandler, M.A. Ismail, J.E. Rayas-Sánchez and Q.J. Zhang, “Neural space mapping optimization of EM microwave structures,” IEEE MTT-S Int. Microwave Symp. Digest (Boston, MA), 2000.