NEURAL SPACE MAPPING EM OPTIMIZATION

OF MICROWAVE STRUCTURES

M.H. Bakr, J.W. Bandler, M.A. Ismail,

J.E. Rayas-Sánchez and Q.J. Zhang

Simulation Optimization Systems Research Laboratory

and Department of Electrical and Computer Engineering

McMaster University, Hamilton, Canada L8S 4K1

bandler@mcmaster.ca

www.sos.mcmaster.ca

presented at

2000 IEEE MTT-S International Microwave Symposium, Boston, MA, June 14, 2000

Artificial Neural Networks (ANN) in Microwave Design

ANNs are suitable models for microwave circuit optimization and statistical design (Zaabab, Zhang and Nakhla, 1995, Gupta et al., 1996, Burrascano and Mongiardo, 1998, 1999)

once they are trained with reliable learning data, the neuromodel can be used for efficient and accurate optimization within the region of training

the principal drawback of this ANN optimization approach is the cost of generating sufficient learning samples

additionally, it is well known that the extrapolation ability of neuromodels is very poor, making unreliable any solution predicted outside the training region

introducing knowledge can alleviate these limitations (Gupta et al., 1999)

Conventional ANN Optimization Approach

step 1

step 2

## many fine model simulations are usually needed

solutions predicted outside the training region are unreliable

Neural Space Mapping (NSM) Optimization

exploits the SM-based neuromodeling techniques

(Bandler et al., 1999)

coarse models are used as source of knowledge that reduce the amount of learning data and improve the generalization and extrapolation performance

NSM requires a reduced set of upfront learning base points

the initial learning base points are selected through sensitivity analysis using the coarse model

neuromappings are developed iteratively: their generalization performance is controlled by gradually increasing their complexity starting with a 3-layer perceptron with 0 hidden neurons

Neural Space Mapping (NSM) Optimization Concept

step 1

step 2

(2n + 1 learning base points for a microwave circuit with n design parameters)

Neural Space Mapping (NSM) Optimization Concept

step 3

step 4

Neural Space Mapping (NSM) Optimization Algorithm

Neuromappings

Space Mapped neuromapping

Frequency-Dependent Space Mapped neuromapping

Frequency Mapped neuromapping

Neuromappings (continued)

Frequency Space Mapped neuromapping

Frequency Partial-Space Mapped neuromapping

we chose a unit mapping (xc = x f  and c = ) as the starting point for the optimization problem

Coarse Optimization Phase

we want to find the optimal coarse model solution xc* that generates the desired response over the frequency range of interest

vector of coarse model responses Rc might contain r different responses of the circuit

Rc(xc)=[Rc1(xc) T…Rcr(xc) T] T

where each individual response has been sampled at Fp frequency points

Rck(xc)=[Rck(xc,ω1)…Rck(xc,ωFp)] T, k=1,…,r

the problem of circuit design using the coarse model can be formulated as

xc=arg minxc U(Rc(xc))

where U is a suitable objective function

Training the SM-Based Neuromodel During NSM Optimization

we solve the problem

w=arg minw  [ ⋯esT⋯] T

with

es = Rf(xf(l),ωj) − Rc(xcj(l),ωcj) ,  es∈ℜr

xcj(l)ωcj=P(i)(xf(l),ωj,w)

j=1,…,Fp

l=1,…,2n+i

s=j+Fp(l−1)

P(i) is the input-output relationship of the ANN at the ith iteration

w contains the free parameters of the current ANN

2n+i is the number of training base points and Fp is the number of frequency points per frequency sweep

the complexity of the ANN (the number of hidden neurons and the SM-based neuromodeling technique) is gradually increased according to the learning error, starting with a linear mapping (3-layer perceptron with 0 hidden neurons)

SM-Based Neuromodel Optimization

we use an SM-based neuromodel as an improved coarse model, optimizing its parameters to generate the desired response

RSMBN  is the SM-based neuromodel response:

RSMBN(xf)=[RSMBN1(xf) T…RSMBNr(xf) T] T

where

RSMBNk(xf)=[Rck(xc1,ωc1)…Rck(xcFp,ωcFp)] T, k=1,…,r

with

xcjωcj=P(i)(xf,ωj,w)   ,  j=1,…,Fp

the next iterate is obtained by solving

xf(2n+i+1)=arg minxf U(RSMBN(xf))

if an SMN neuromapping is used to implement P(i), the next iterate can be obtained in a simpler manner by solving

xf(2n+i+1)=arg minxf PSM(i)(xf,w)−xc

HTS Quarter-Wave Parallel Coupled-Line Microstrip Filter

(Westinghouse, 1993)

NSM Optimization of the HTS Microstrip Filter

specifications

|S21|  0.95 in the passband and |S21|  0.05 in the stopband,

where the stopband includes frequencies below 3.967 GHz and above 4.099 GHz, and the passband lies in the range [4.008GHz, 4.058GHz]

“coarse” model: OSA90/hope empirical models

“fine” model: Sonnet’s em with high resolution grid

we take L0 = 50 mil, H = 20 mil, W = 7 mil, r = 23.425, loss tangent = 3105; the metalization is considered lossless

the design parameters are xf = [L1 L2 L3 S1 S2 S3] T

NSM Optimization of the HTS Filter (continued)

coarse and fine model responses at the optimal coarse solution,

xc* = [188.33  197.98  188.58  21.97  99.12  111.67] T  (mils)

OSA90/hope () and em ()

NSM Optimization of the HTS Filter (continued)

the initial 2n+1 points are chosen by performing sensitivity analysis on the coarse model: a 3% deviation from xc* for L1, L2, and L3 is used, while a 20% is used for S1, S2, and S3

coarse and fine model responses at base points:

OSA90/hope

em

NSM Optimization of the HTS Filter (continued)

Learning errors at base points:

before any neuromapping

mapping  with a 3LP:7-3-1

NSM Optimization of the HTS Filter (continued)

Learning errors at base points:

mapping  and L1 with a 3LP:7-4-2

mapping , L1 and S1 with a 3LP:-7-5-3

NSM Optimization of the HTS Filter (continued)

em () and FPSM 7-5-3 () model responses at the next point predicted after the first NSM iteration

xf (14) = [185.37  195.01  184.24  21.04  86.36  91.39] T  (mils)

NSM Optimization of the HTS Filter (continued)

em () and FPSM 7-5-3 () model responses at the NSM solution using a fine frequency sweep

NSM Optimization of the HTS Filter (continued)

em () and FPSM 7-5-3 () model responses at the NSM solution in the passband using a fine frequency sweep

Bandstop Microstrip Filter with Quarter-Wave Open Stubs

we take H = 25 mil, W0 = 25 mil, r = 9.4 (alumina)

the design parameters are xf = [W1 W2 L0 L1 L2] T

NSM Optimization of the Bandstop Filter

specifications

|S21|  0.01 in the stopband and |S21|  0.9 in the passband,

where the stopband lies between 9.3 GHz and 10.7 GHz, and the passband includes frequencies below 8 GHz and above 12 GHz

“coarse” model: transmission line sections and empirical formulas

“fine” model: Sonnet’s em with high resolution grid

NSM Optimization of the Bandstop Filter (continued)

coarse and fine model responses at the optimal coarse solution,

xc* = [6.00  9.01  106.45  110.15  108.81] T  (mils)

coarse model () and em ()

the initial 2n+1 points are chosen by performing sensitivity analysis on the coarse model: a 50% deviation from xc* for W1, W2, and L0 is used, while a 15% is used for L1, and L2.

NSM Optimization of the Bandstop Filter (continued)

FM (3LP:6-2-1,) neuromodel () and the fine model () responses at the optimal coarse solution

coarse () and fine model () responses at the next point predicted by the first NSM iteration

NSM Optimization of the Bandstop Filter (continued)

FPSM (3LP:6-3-2,,W2) neuromodel () and the fine model () responses at the point predicted by the first NSM iteration

coarse () and fine model () responses at the next point predicted by the second NSM iteration

NSM Optimization of the Bandstop Filter (continued)

fine model response () at the next point predicted by the second NSM iteration and optimal coarse response (), using a fine frequency sweep

xf (13) = [5.92  13.54  83.34  114.14  124.81] T  (mils)

Conclusions

we present an innovative algorithm for EM optimization based on Space Mapping technology and Artificial Neural Networks

Neural Space Mapping (NSM) optimization exploits our SM-based neuromodeling techniques

NFSM does not require parameter extraction to predict the next point

an initial mapping is established by performing upfront fine model analysis at a reduced number of base points

the coarse model sensitivity is exploited to select those base points

Huber optimization is used to train simple SM-based neuromodels at each iteration

the SM-based neuromodels are developed without using testing points: their generalization performance is controlled by gradually increasing their complexity starting with a 3-layer perceptron with 0 hidden neurons

an HTS filter and a bandstop microstrip filter illustrate our optimization technique