Broadband physics-based modeling of microwave passive devices through frequency mapping

J.W. Bandler, M.A. Ismail and J.E. Rayas-Sánchez

SOS-00-17-V

June 2000

 J.W. Bandler, M.A. Ismail and J.E. Rayas-Sánchez 2000

No part of this document may be copied, translated, transcribed or entered in any form into any machine without written permission.  Address enquiries in this regard to Dr. J.W. Bandler.  Excerpts may be quoted for scholarly purposes with full acknowledgement of source.  This document may not be lent or circulated without this title page and its original cover.

###### Broadband physics-based modeling of microwave passive devices through frequency mapping

J.W. Bandler, M.A. Ismail and J.E. Rayas-Sánchez

Simulation Optimization Systems Research Laboratory

and Department of Electrical and Computer Engineering

##### McMaster University, Hamilton, Canada L8S 4K1

bandler@mcmaster.ca

www.sos.mcmaster.ca

presented at

2000 IEEE MTT-S International Microwave Symposium, Boston, MA, USA, June 14, 2000

Basic Concepts and Notation

fine models: EM simulators, measurements

the objective is to develop empirical models valid over a wide frequency range

xf: an n-dimensional vector representing the physical parameters

y   : a vector representing the elements of the circuit model

ω : the fine model frequency (the actual frequency)

ωc: the circuit model frequency

Frequency Mapping

frequency mapping is a transformation from the fine model frequency and parameters to the circuit model frequency

frequency mapping has been utilized in the Space Mapping context (Bandler et al., 1994, 1999, 2000)

frequency transformations have roots in classical filter design, for example, low-pass to band-pass or high-pass transformations

dimensional analysis (Buckingham, 1914, Watson et al., 1999) is applied to determine the functionality of the circuit model elements and the frequency mapping of the geometrical and physical parameters

the circuit model frequency should be an odd function of the fine model frequency

Artificial Neural Networks (ANN) or Rational Functions (RF) can be used to approximate the circuit model elements and frequency mapping

Frequency Independent Empirical Models (FIEM)

xr: an m-dimensional vector representing the reduced set of input parameters, where m < n

Frequency Dependent Empirical Models (FDEM)

the circuit model elements and the frequency mapping are evaluated by solving

min w1,w2  [e11T⋯e1MT e21T⋯e2MT⋯eN1T⋯eNMT]T

where N is the total number of training points, M is the number of frequency points per frequency sweep and

eij = Rf(xfi,ωj)−Rc(Q(xr i,w1), Ω(xr i,ωj,w2))

##### Transformation of FDEMs into FIEMs

this involves one-port impedance synthesis, which states that the impedance we want to realize should be a real rational function

the circuit model frequency can take the form

ωc=ω f1−ω2f2f3−ω2f4

the impedance associated with an inductor L in the FDEM is given by

ZL=j ωcL=j ωLf1−ω2f2f3−ω2f4

this impedance can be realized using one-port impedance synthesis such as the first Foster or second Foster or ladder realization

FIEM of the Microstrip Right Angle Bend

the fine and the circuit models

xf=[W H εr]T					y=[L C]T

applying dimensional analysis the circuit model elements are given by

LH=μ φ1(WH),  CH=ε φ2(WH, εr)

[LH CH]T=Q(xr), xr=[WH εr]T

FIEM of the Microstrip Right Angle Bend

the region of interest

0.2 < W/H < 6

2 < εr < 11

1 GHz < freq < 11 GHz

the training points are chosen according to Central Composite Design (CCD) (Montgomery, 1996) plus 4 more points (13 training points)

the circuit model elements are approximated by a three layer perceptron ANN with 5 hidden neurons and by RF

the ANN and the RF were implemented in OSA90/hope and the Huber optimizer was used for training

FIEM of the Microstrip Right Angle Bend

the error in S11 with respect to emTM at 16 test points in the region of interest

| the FIEM is developed by ANNs | the FIEM is developed by RFs |
| --- | --- |

Jansen’s model (1989)

FIEM of the Microstrip Right Angle Bend

the error in S11 with respect to emTM over a broad frequency range

| the FIEM is developed by ANNs | the FIEM is developed by RFs |
| --- | --- |

Jansen’s model (1989)

FDEM of the Microstrip Right Angle Bend

frequency range from 1 GHz to 31 GHz

applying dimensional analysis to the frequency mapping and noticing that c is an odd function of   we get

ωc=ω f(WH,εr,(ωHc)2)

where c is the speed of light

the model elements and the circuit model frequency ωc are approximated by RFs

FDEM of the Microstrip Right Angle Bend

expressions of the elements of the FDEM of the microstrip right angle bend

| element | expression |
| --- | --- |
| L/H(nH/mil) | 0.03192 −0.09−0.018x1+0.3x121+2.853x12 |
| C/H(pF/mil) | 0.000225(−0.46+0.162x1−0.014x2+0.275x12+2.855x1x2+0.262x12x2) |
| c/ | f1(x1,x2,x3)f2(x1,x2,x3) f1(x1,x2,x3)=0.759−0.0192x1−0.0179x2+0.0187x3+0.0738x12+0.0026x1x2 −0.1405x1x3+0.0079x2x3+0.0018x13−0.0071x12x2+0.1188x12x3 +0.0017x1x22+0.0419x1x2x3−0.0022x22x3 f2(x1,x2,x3)=1+0.0282x1−0.0086x2−0.0175x3+0.0051x12−0.0063x1x2 +0.1674x1x3+0.0037x22−0.0067x2x3+0.0055x13−0.0028x12x2 +0.0011x12x3+0.0056x1x2x3−0.0012x22x3 where x1=WH, x2=εr, x3=1.816e-7 (ω(GHz)H(mil))2 |

FDEM of the Microstrip Right Angle Bend

the error in S11 and S21 at 16 test points in the region of interest

FDEM of the Microstrip Right Angle Bend

the scattering parameters obtained by the FDEM and the fine model (Sonnet’s em) of the microstrip right angle bend

Extraction of an Equivalent FIEM from the FDEM of the Microstrip Right Angle Bend

the FDEM of the microstrip right angle bend is

the working frequency is ωc

the equivalent impedance of L is

ZL=jωcL = jωLf1(x1,x2)−ω2x3f2(x1,x2)f3(x1,x2)−ω2x3f4(x1,x2)

the equivalent impedance of C is

ZC=1jωcC =1jωC f3(x1,x2)−ω2x3f4(x1,x2)f1(x1,x2)−ω2x3f2(x1,x2)

x1=W/H, x2=εr and x3=1.816∗10−7H2

Extraction of an Equivalent FIEM from the FDEM of the Microstrip Right Angle Bend

the equivalent FIEM is

Conclusions

we present a unified computer-aided modeling methodology for developing broadband models of microwave passive components

we integrate full-wave EM simulations, artificial neural networks, multivariable rational functions, dimensional analysis and frequency mapping

two types of models are considered: FIEMs and FDEMs

FDEMs can be transformed to equivalent FIEMs if we use a RF to approximate the frequency mapping

examples include a microstrip right angle bend, a microstrip via and a CPW step junction

Dimensional Analysis for the Frequency Mapping of the Microstrip Right Angle Bend

π=Hx1 Wx2 cx3εx4 ε0x5ωx6 ωcx7

|  | x1 | x2 | x3 | x4 | x5 | x6 | x7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | H | W | c |  | 0 | ω | ωc |
| Kg | 0 | 0 | 0 | -1 | -1 | 0 | 0 |
| M | 1 | 1 | 1 | -3 | -3 | 0 | 0 |
| S | 0 | 0 | -1 | 4 | 4 | -1 | -1 |
| A | 0 | 0 | 0 | 2 | 2 | 0 | 0 |

#of independent π terms =7-3=4

these terms can be evaluated by solving

−x4−x5=0, x1+x2+x3=0, x3−x6−x7=0

that is

Dimensional Analysis for the Frequency Mapping of the Microstrip Right Angle Bend

x4=−x5, x3=−x1−x2, x6=−x7+x1+x2

the solutions are

| x1 | x2 | x5 | x7 | x3 | x4 | x6 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | -1 | 0 | 1 |
| 0 | 1 | 0 | 0 | -1 | 0 | 1 |
| 0 | 0 | 1 | 0 | 0 | -1 | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 | -1 |

the three π terms are given by

π1=ωHc, π2=ωWc, π3=εε0=εr, π4=ωcω

from π1 and π2 we get π2'=WH

Dimensional Analysis for the Frequency Mapping of the Microstrip Right Angle Bend

the circuit model frequency is given by

ωc=ω f(WH,εr,(ωHc))