YIELD-DRIVEN EM OPTIMIZATION USING

SPACE MAPPING-BASED NEUROMODELS

- J.W. Bandler, 1,2 J.E. Rayas-Sánchez 2 and Q.J. Zhang 3

1 Bandler Corporation, P.O. Box 8083, Dundas, Canada L9H 5E7, j.bandler@ieee.org

2 McMaster University, 1280 Main St. West, Hamilton, Canada L8S 4K1, bandler@mcmaster.ca

3 Carleton University, 1125 Colonel By Drive, Ottawa, Canada K1S 5B6, qjz@doe.carleton.ca

- ABSTRACT

In this work, an efficient procedure to realize electromagnetics-based yield optimization and statistical analysis of microwave structures using space mapping-based neuromodels is proposed.  A generalized relationship between the fine and coarse model sensitivities through the Jacobian of the neuromapping is proposed.  Our technique is illustrated by the EM-based statistical analysis and yield optimization of an HTS microstrip filter.

- INTRODUCTION

With the increasing availability of commercial EM simulators, it is very desirable to include them in statistical analysis and yield-driven design of microwave circuits.  Given the high cost in computational effort imposed by the EM simulators, creative procedures must be developed to efficiently use them for statistical analysis and design.

We propose the use of space mapping-based neuromodels for efficient and accurate EM-based statistical analysis and yield optimization of microwave structures.  A general equation to express the relationship between the fine and coarse model sensitivities through a nonlinear, frequency-sensitive neuromapping is presented.  We illustrate our technique by the yield analysis and optimization of a high-temperature superconducting (HTS) quarter-wave parallel coupled-line microstrip filter.

YIELD ANALYSIS AND OPTIMIZATION VIA SPACE MAPPING BASED

NEUROMODELS

Let the vectors xc, xf  n represent the design parameters of the coarse and fine models, respectively.  The operating frequency ω, used by the fine model, can be different to that used by the coarse model ωc.  Let Rc(xc,c), Rf (xf,)  r represent the coarse and fine model responses at c and , respectively.  We denote the corresponding SM-based neuromodel responses at frequency  as RSMBN (xf,), given by

| RSMBN(xf,ω)=Rc(xc,ωc) | (1) |
| --- | --- |

with

| xcωc=P(xf,ω) | (2) |
| --- | --- |

where the mapping function P is implemented by a neural network following any of the 5 neuromapping variations (SM, FDSM, FSM, FM or FPSM) described in the work by Bandler et al. [].  We assume that a suitable mapping function P has already been found (i.e., a neural network with suitable complexity has already been trained).

If the SM-based neuromodel is properly developed,

| Rf(xf,ω)≈RSMBN(xf,ω) | (3) |
| --- | --- |

for all xf and ω in the training region.

Let the Jacobian of the fine model responses w.r.t. the fine model parameters be Jf  rn; let the Jacobian of the coarse model responses w.r.t. the coarse model parameters and mapped frequency be Jc  r(n+1) and let the Jacobian of the mapping w.r.t. the fine model parameters be JP  (n+1)n.  Then the sensitivities of the fine model responses can be approximated using

| Jf≈Jc JP | (4) |
| --- | --- |

The accuracy of the approximation of Jf using (4) will depend on how well the SM-based neuromodel reproduces the behavior of the fine model in the training region.

If the mapping is implemented with a 3-layer perceptron with h hidden neurons, (2) is given by

| P(xf,ω)=WoΦ(xf,ω)+bo Φ(xf,ω)=[ϕ(s1)ϕ(s2)…ϕ(sh)]T s=Whxfω+bh | (5) (6) (7) |
| --- | --- |

where W o  (n+1)h is the matrix of output weighting factors, bo n+1 is the vector of output bias elements,   h is the vector of hidden signals, s  h is the vector of activation potentials, W h  h(n+1) is the matrix of hidden weighting factors, bh h is the vector of hidden bias elements and h is the number of hidden neurons.  A typical choice for the nonlinear activation functions is hyperbolic tangents, i.e., () = tanh().  All the internal parameters of the neural network, bo, bh, W o and  W h are constant since the SM-based neuromodel has been already developed.

The Jacobian JP is obtained from (5-7) as

| JP=WoJΦWh | (8) |
| --- | --- |

where J  hh is a diagonal matrix given by J = diag( ' (sj)), with j = 1… h.  If the SM-based neuromodel uses a 2-layer perceptron, the Jacobian JP is simply

| JP=Wo | (9) |
| --- | --- |

which corresponds to the case of a frequency-sensitive linear mapping.  Notice that by substituting (9) in (4) and assuming a frequency-insensitive neuromapping we obtain the lemma found by Bakr et al. [], since in the case of a 2-layer perceptron with no frequency dependence, W o  nn.

YIELD OPTIMIZATION OF AN HTS FILTER (SYMMETRIC CASE)

Consider a high-temperature superconducting (HTS) parallel coupled-line microstrip filter [Error: Reference source not found, ] illustrated in Fig. 1.  OSA90/hope built-in linear elements connected by circuit theory form the “coarse” model.  Sonnet’s em driven by Empipe forms the fine model, using a high-resolution grid.  The SM-based neuromodel of the HTS filter of [Error: Reference source not found] is used.  The corresponding SM-based neuromodel is illustrated in Fig. 2, which implements a frequency partial-space mapped neuromapping with 7 hidden neurons, mapping only L1, S1 and the frequency (3LP:7-7-3).  Applying direct minimax optimization to the coarse model, we obtain the optimal coarse solution xc*.  We apply direct minimax optimization to the SM-based neuromodel, starting at xc*, to obtain the optimal SM-based neuromodel nominal solution xSMBN*.

For yield analysis, we consider 0.2% of variation for the dielectric constant and for the loss tangent, as well as 75 micron of variation for the physical dimensions, with uniform statistical distributions.  We perform Monte Carlo yield analysis of the SM-based neuromodel around xSMBN* with 500 outcomes.  This takes a few tens of seconds on a PC (AMD 640MHz, 256M RAM, Windows NT 4.0).  A single outcome calculation for the same circuit using an EM simulation takes about 5 hours.  The responses for 50 outcomes are shown in Fig. 3.  The yield calculation is shown in Fig. 4.  A yield of only 18.4% is obtained at xSMBN*.  We then apply yield optimization to the SM-based neuromodel with 500 outcomes using the Yield-Huber optimizer available in OSA90/hope, obtaining the optimal yield solution: xSMBNY*.  The corresponding responses for 50 outcomes are shown in Fig. 5.  The yield is increased from 18.4% to 66%, as shown in Fig. 6. Excellent agreement between the EM responses and the SM-based neuromodel responses was found at both the optimal nominal solution and the optimal yield solution.

#### CONCLUSIONS

An efficient procedure to realize EM-based statistical analysis and yield optimization of microwave structures is proposed.  A general equation relates the fine and coarse model sensitivities through the Jacobian of the neuromapping.  The yield-driven design of an HTS filter is illustrated.

- ACKNOWLEDGEMENT

- The authors thank Dr. J.C. Rautio, President of Sonnet Software, Inc., Liverpool, NY, for making em available.

##### REFERENCES

[?] 	M.H. Bakr, J.W. Bandler, M.A. Ismail, J.E. Rayas-Sánchez and Q.J. Zhang, “Neural space mapping optimization for EM-based design,” IEEE Trans. Microwave Theory Tech., vol. 48, 2000, pp. 2307-2315.

[?] 	M.H. Bakr, J.W. Bandler, N. Georgieva and K. Madsen, “A hybrid aggressive space-mapping algorithm for EM optimization,” IEEE Trans. Microwave Theory Tech., vol. 47, 1999, pp. 2440-2449.

[?] 	J.W. Bandler, M.A. Ismail, J.E. Rayas-Sánchez and Q.J. Zhang, “Neuromodeling of microwave circuits exploiting space mapping technology,” IEEE Trans. Microwave Theory Tech., vol. 47, 1999, pp. 2417-2427.

Fig. 1.  HTS quarter-wave parallel coupled-line microstrip filter.Fig. 2.  SM-based neuromodel of the HTS filter for yield analysis assuming symmetry (L1c and S1c correspond to L1 and S1 as used by the coarse model).

Fig. 3.  Monte Carlo yield analysis of the SM-based neuromodel responses around the optimal nominal solution xSMBN* with 50 outcomes.

Fig. 4.  Histogram of the yield analysis of the SM-based neuromodel around the optimal nominal solution  xSMBN* with 500 outcomes.

Fig. 5.  Monte Carlo yield analysis of the SM-based neuromodel responses around the optimal yield solution xSMBN Y* with 50 outcomes.Fig. 6.  Histogram of the yield analysis of the SM-based neuromodel around the optimal yield solution xSMBNY* with 500 outcomes (considering symmetry).

This work was supported in part by the Natural Sciences and Engineering Research Council of Canada (NSERC) under Grants OGP0007239, STR234854-00 and through the Micronet Network of Centres of Excellence.  J.E. Rayas-Sánchez is supported by an Ontario Graduate Scholarship, by McMaster University and by ITESO (Instituto Tecnológico y de Estudios Superiores de Occidente, Mexico).